3GPP TSG-SA5 Meeting #128	S5-197187

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0403 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.260 IMS charging trigger |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Applicable Triggers for IMS converged online and offline charging has not been defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Applicable Triggers for IMS converged online and offline charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS converged online and offline charging is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First changes

5.X	IMS charging scenarios for service based charging interface scenarios

5.X.3	Applicable Triggers in the IMS-GWF, MRFC and AS

It is operator configurable in the nodes for which SIP method a Charging Data Request is sent. The table below describes all possible Charging Data Requests that might be sent from an IMS-GWF or an MRFC or an AS.

It is configurable for the operators to enable or disable the generation of a Charging Data Request message by the IMS node in response to a particular "triggering SIP method".

Table 5.x.3.1: Default Trigger conditions in or MRFC

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP INVITE for initiating a multimedia ad hoc conferencing session | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP 2xx acknowledging an SIP INVITE for initiating a multimedia ad hoc conferencing session, and no charging session exists. | - | Immediate | Not Applicable | Not Applicable |  |
| SIP ACK acknowledging a SIP INVITE to connect an UE to the conferencing session | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or SIP UPDATE[e.g. change in media components] | - | Immediate | Not Applicable | Not Applicable |  |
| SIP BYE(NOTE 1) | - | Immediate | Not Applicable | Not Applicable |  |
| Expiration of Interim | - | Immediate | Not Applicable | Not Applicable |  |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). | - | Immediate | Not Applicable | Not Applicable |  |
| SIP BYE message (both normal and abnormal session termination cases) (NOTE 2) | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP CANCEL | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response with error codes 4xx, 5xx or 6xx indicating termination of an ongoing session | - | Immediate | Not Applicable | Not Applicable |  |
| NOTE 1: This trigger only applies to a user leaving an ongoing conferencing session NOTE 2: This trigger only applies if this causes the ongoing conferencing session to terminate |  |  |  |  |  |

Table 5.x.3.2: Default Trigger conditions in or IMS-GWF or AS

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP INVITE | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] ECUR: Charging Data Request [Initial] |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable |  |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or SIP UPDATE [e.g. change in media components] | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Update] |
| RE-INVITE or SIP UPDATE [e.g. change in media components, terminating identity change] | - | Immediate | Not Applicable | Not Applicable |  |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). | - | Immediate | Not Applicable | Not Applicable |  |
| Any SIP message (except those triggering a Debit / Reserve Units Request[Initial] or those not covered by the above triggers for Reserve Units Request[Update] conveying a SDP offer or its associated SDP answer before SIP session establishment | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | Not Applicable | Not Applicable |  |
| SIP response (4xx, 5xx or 6xx), indicating an unsuccessful SIP RE-INVITE or SIP UPDATE |  |  |  |  |  |
| SIP BYE message (both normal and abnormal session termination cases) | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE message (only when last user location information of originating/ terminating party is required by operator for legal purpose). | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up procedure | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging non-session related SIP messages | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure | - | Immediate | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger, or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] SCUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final/Redirection Response 3xx | - | Immediate | Not Applicable | Not Applicable |  |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure | - | Immediate | Not Applicable | Not Applicable |  |

For converged charging, the following details of chargeable events and corresponding actions in the MRFC are defined in Table 5.X.3.3:

Table 5.X.3.3: Chargeable events and their related actions in MRFC

| Chargeable event | Conditions | MRFC action |
| --- | --- | --- |
| SIP INVITE for initiating a multimedia ad hoc conferencing session | If quota management is required | Charging Data Request [Initial] with a possible request quota for later use |
| SIP 2xx acknowledging an SIP INVITE for initiating a multimedia ad hoc conferencing session, and no charging session exists. | If charging resource, i.e. charging session, for the PDU session does not exist | Charging Data Request [Initial] |
| SIP ACK acknowledging a SIP INVITE to connect an UE to the conferencing session |  | Charging Data Request [Update] |
| SIP RE-INVITE or SIP UPDATE[e.g. change in media components] |  | Charging Data Request [Update] with a possible request quota |
| SIP BYE(NOTE 1) |  | Charging Data Request [Update] with a possible request quota |
| Expiration of Interim |  | Charging Data Request [Update] |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). |  | Charging Data Request [Update] with a possible request quota |
| SIP BYE message (both normal and abnormal session termination cases) (NOTE 2) |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP CANCEL |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP Final Response with error codes 4xx, 5xx or 6xx indicating termination of an ongoing session |  | Charging Data Request [Termination], indicating that charging session is terminated |
| NOTE 1: This trigger only applies to a user leaving an ongoing conferencing session NOTE 2: This trigger only applies if this causes the ongoing conferencing session to terminate |  |  |

For converged charging, the following details of chargeable events and corresponding actions in the IMS-GWF and AS are defined in Table 5.x.3.4:

Table 5.X.3.4: Chargeable events and their related actions in IMS-GWF and AS

| Chargeable event | Conditions | IMS-GWF and AS action |
| --- | --- | --- |
| SIP INVITE |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP NOTIFY |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP MESSAGE |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP REGISTER |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP SUBSCRIBE |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP REFER |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP PUBLISH |  | Charging Data Request [Initial] with a possible request quota for later use |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or SIP UPDATE [e.g. change in media components] |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| RE-INVITE or SIP UPDATE [e.g. change in media components, terminating identity change] |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| Expiration of quota, Validity time expiry or other authorization triggers (quota threshold reached, …). |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| Any SIP message (except those triggering a Debit / Reserve Units Request[Initial] or those not covered by the above triggers for Reserve Units Request[Update] conveying a SDP offer or its associated SDP answer before SIP session establishment |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP response (4xx, 5xx or 6xx), indicating an unsuccessful SIP RE-INVITE or SIP UPDATE |  | Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP BYE message (both normal and abnormal session termination cases) |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP 2xx acknowledging a SIP BYE message (only when last user location information of originating/ terminating party is required by operator for legal purpose). |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up procedure |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP 2xx acknowledging non-session related SIP messages |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure |  | Charging Data Request [Termination], indicating that charging session is terminated |
| Aborting a SIP session set-up procedure, using an internal trigger, or a SIP CANCEL |  | Charging Data Request [Termination], indicating that charging session is terminated |
| Deregistration |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP Final/Redirection Response 3xx |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP NOTIFY |  | IEC: Charging Data Request [Event] |
| SIP MESSAGE |  | IEC: Charging Data Request [Event] |
| SIP REGISTER |  | IEC: Charging Data Request [Event] |
| SIP SUBSCRIBE |  | IEC: Charging Data Request [Event] |
| SIP REFER |  | IEC: Charging Data Request [Event] |
| SIP PUBLISH |  | IEC: Charging Data Request [Event] |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure |  | IEC: Charging Data Request [Event] |

End of change