3GPP TSG-SA5 Meeting #128 	S5-197216

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019								Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0775 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Furnish Charging Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to EPC charging, PS Furnish Charging Information procedure can only apply when online and offline charging is performed simultaneously for the same session (IP-CAN bearer or TDF session) or rating group. For 5G converged charging, some specific information for converged online charging is required in the CHF CDR. The Furnish Charging Information procedure should be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the furnish charging information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of Furnish Charging Information procedure is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.0,5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.5.0	CHF record (CHF-CDR)

If enabled, CHF records shall be produced for chargeable events, with or without quota management. The generic fields in the record are specified in table 5.1.5.0.1. The NF specific parts will be concatenated to this e.g. the PDU Session Information, PDU Container Information and Roaming QBC Information are concatenated for the SMF.

Table 5.1.5.0.1: CHF record (CHF-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Charging Session Identifier | OC | This field holds the Session Identifier described in TS 32.290 [57]. |
| Subscriber Identifier | M | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party, if available. |
| NF Consumer Information | M | This field holds the information of the NF consumer of the charging service. |
| NF Functionality | M | This field holds the type of functionality the NF provides. |
| NF Name | OC | This field holds the name of the NF used. |
| NF Address | OC | This field holds the IP Address of the NF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the NF. |
| Triggers | OC | This field holds the triggers that are common to all Multiple Unit Information. Can be the same as in Used Unit Container. |
| SMF Triggers | OC | This field holds the 5G data connectivity specific triggers described in TS 32.255 [15]. |
| List of Multiple Unit Information | OC | This field holds the parameters for the unit reporting. It may have multiple occurrences. |
| Rating Group | M | This filed holds the rating group. The parameter corresponds to the Charging Key as specified in TS 23.203 [203] |
| Used Unit Container | OC | This field holds the used units and information connected to the reported units. |
| Service Identifier | OC | This field holds the Service Identifier. |
| Quota management Indicator | Oc | This field holds an indicator on whether the reported used units are with or without quota management control. If the field is not present, it indicates the used unit is without quota management applied. |
| Local Sequence Number | OM | This field holds the container sequence number. |
| Time | OC | This field holds the amount of used time. |
| Uplink Volume | OC | This field holds the amount of used volume in uplink direction. |
| Downlink Volume | OC | This field holds the amount of used volume in downlink direction. |
| Total Volume | OC | This field holds the amount of used volume in both uplink and downlink directions. |
| Service Specific Units | Oc | This field holds the amount of used service specific units. |
| Event Time Stamp | OC | This field holds the timestamps of the event reported in the Service Specific Units, if the reported units are event based. |
| Rating Indicator | OC | This field indicates if the units have been rated or not. |
| Triggers | OC | This field holds the triggers that caused the Used Unit Container to be reported, independently on if they are PDU Session or RG level triggers. |
| SMF Triggers | OC | This field holds the 5G data connectivity specific triggers described in TS 32.255 [15]. |
| Trigger Time Stamp | Oc | This field holds the timestamp of the trigger. |
| PDU Container Information | OC | This field holds the 5G data connectivity specific information described in TS 32.255 [15]. |
| Furnish Charging Information | OC | This field holds specific information for online charging (i.e. with quota management). |
| UPF ID | OC | This field holds the UPF identifier used to identify the UPF when reporting the usage for the UPF. |
| Record Opening Time | OC | Time stamp when the PDU session is activated in the SMF or record opening time on subsequent partial records. |
| Duration | M | This field holds the duration of this record. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Cause for Record Closing | M | The reason for the release of the record. |
| Local Record Sequence Number | OM | Consecutive record number created by the CDF. The number is allocated sequentially including all CDR types. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in TS 32.255 [15] |
| Roaming QBC Information | OM | This field holds the roaming 5G data connectivity specific information described in TS 32.255 [15] |
| SMS Charging Information | OC | This field holds the SMS specific information described in TS 32.274 [34]. |

Next change

5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

CauseForRecClosing,

ChargingID,

DataVolumeOctets,

Diagnostics,

EnhancedDiagnostics,

DynamicAddressFlag,

InvolvedParty,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSTimeZone,

NodeAddress,

PLMN-Id,

PriorityType,

RATType,

RecordType,

ServiceSpecificInfo,

Session-Id,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

ChargingCharacteristics,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

RatingGroupId,

ServiceIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

OriginatorInfo,

RecipientInfo,

SMMessageType,

SMSResult,

SMSStatus

FROM SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

;

--

--  CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord

}

ChargingRecord 	::= SET

{

recordType						[0] RecordType,

recordingNetworkFunctionID		[1] NetworkFunctionName,

subscriberIdentifier			[2] SubscriptionID OPTIONAL,

nFunctionConsumerInformation	[3] NetworkFunctionInformation,

triggers						[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage			[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime				[6] TimeStamp,

duration						[7] CallDuration,

recordSequenceNumber			[8] INTEGER OPTIONAL,

causeForRecClosing				[9] CauseForRecClosing,

diagnostics						[10] Diagnostics OPTIONAL,

localRecordSequenceNumber		[11] LocalSequenceNumber OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation	[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation			[14] RoamingQBCInformation OPTIONAL,

sMSChargingInformation			[15] SMSChargingInformation OPTIONAL,

chargingSessionIdentifier		[16] ChargingSessionIdentifier OPTIONAL

}

--

-- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID			[0] ChargingID,

userIdentifier					[1] InvolvedParty OPTIONAL,

userEquipmentInfo				[2] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation			[3] OCTET STRING OPTIONAL,

userRoamerInOut					[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo		[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId					[6] PDUSessionId,

networkSliceInstanceID			[7] NetworkSliceInstanceID OPTIONAL,

pDUType							[8] PDUSessionType OPTIONAL,

sSCMode							[9] SSCMode OPTIONAL,

sUPIPLMNIdentifier				[10] PLMN-Id OPTIONAL,

servingNetworkFunctionID		[11] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

rATType							[12] RATType OPTIONAL,

dataNetworkNameIdentifier		[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress						[14] PDUAddress OPTIONAL,

authorizedQoSInformation		[15] AuthorizedQoSInformation OPTIONAL,

uETimeZone 						[16] MSTimeZone OPTIONAL,

pDUSessionstartTime				[17] TimeStamp OPTIONAL,

pDUSessionstopTime				[18] TimeStamp OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

chargingCharacteristics			[20] ChargingCharacteristics,

chChSelectionMode				[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus			[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 		[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL,

subscribedQoSInformation 		[24] SubscribedQoSInformation OPTIONAL,

authorizedSessionAMBR 			[25] SessionAMBR OPTIONAL,

subscribedSessionAMBR 			[26] SessionAMBR OPTIONAL,

servingCNPLMNID					[27] PLMN-Id OPTIONAL,

sUPIunauthenticatedFlag 		[28] NULL OPTIONAL,

homeProvidedChargingID			[29] ChargingID OPTIONAL,

dnnSelectionMode				[30] DNNSelectionMode OPTIONAL

}

--

-- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1] NetworkFunctionName OPTIONAL,

roamingChargingProfile			[2] RoamingChargingProfile OPTIONAL

}

--

-- SMS Charging Information

--

SMSChargingInformation	::= SET

{

sMSNodeAddress				[0] AddressString,

originatorInfo				[1] OriginatorInfo OPTIONAL,

recipientInfos				[2] SEQUENCE OF RecipientInfo OPTIONAL,

userEquipmentInfo			[3] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation		[4] OCTET STRING OPTIONAL,

uETimeZone 					[5] MSTimeZone OPTIONAL,

rATType						[6] RATType OPTIONAL,

sMSCAddress					[7] AddressString OPTIONAL,

eventtimestamp				[8] TimeStamp,

-- 9 to 19 is for future use

sMDataCodingScheme			[20] INTEGER OPTIONAL,

sMMessageType				[21] SMMessageType OPTIONAL,

sMReplyPathRequested		[22] SMReplyPathRequested OPTIONAL,

sMUserDataHeader			[23] OCTET STRING OPTIONAL,

sMSStatus					[24] SMSStatus OPTIONAL,

sMDischargeTime				[25] TimeStamp OPTIONAL,

sMTotalNumber 				[26] INTEGER OPTIONAL,

sMServiceType				[27] SMServiceType OPTIONAL,

sMSequenceNumber 			[28] INTEGER OPTIONAL,

sMSResult					[29] SMSResult OPTIONAL,

submissionTime				[30] TimeStamp OPTIONAL,

sMPriority					[31] PriorityType OPTIONAL,

messageReference			[32] MessageReference,

messageSize					[33] INTEGER OPTIONAL,

messageClass				[34] MessageClass OPTIONAL,

sMdeliveryReportRequested	[35] SMdeliveryReportRequested OPTIONAL

}

--

-- PDU Container Information

--

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName				[0] ChargingRuleBaseName OPTIONAL,

afChargingIdentifier				[1] ChargingId OPTIONAL,

timeOfFirstUsage					[2] TimeStamp OPTIONAL,

timeOfLastUsage						[3] TimeStamp OPTIONAL,

qoSInformation						[4] FiveGQoSInformation OPTIONAL,

userLocationInformation				[5] OCTET STRING OPTIONAL,

presenceReportingAreaInfo			[6] PresenceReportingAreaInfo OPTIONAL,

rATType								[7] RATType OPTIONAL,

sponsorIdentity						[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[9] OCTET STRING OPTIONAL,

servingNetworkFunctionID			[10] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

uETimeZone 							[11] MSTimeZone OPTIONAL,

threeGPPPSDataOffStatus				[12] ThreeGPPPSDataOffStatus OPTIONAL

}

--

-- QFI Container Information

--

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId							[0] QoSFlowId OPTIONAL,

triggers							[1] SEQUENCE OF Trigger,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

dataTotalVolume						[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[5] DataVolumeOctets OPTIONAL,

localSequenceNumber					[6] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[8] TimeStamp OPTIONAL,

timeOfLastUsage						[9] TimeStamp OPTIONAL,

qoSInformation						[10] FiveGQoSInformation OPTIONAL,

userLocationInformation				[11] OCTET STRING OPTIONAL,

uETimeZone	 						[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[13] PresenceReportingAreaInfo OPTIONAL,

rATType								[14] RATType OPTIONAL,

reportTime							[15] TimeStamp,

servingNetworkFunctionID			[16] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

threeGPPPSDataOffStatus				[17] ThreeGPPPSDataOffStatus OPTIONAL,

threeGPPChargingID					[18] ChargingID OPTIONAL,

diagnostics					[19] Diagnostics OPTIONAL,

extensionDiagnostics				[20] EnhancedDiagnostics OPTIONAL

}

--

--  CHF CHARGING TYPES

--

FiveGQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi					[1] INTEGER,

aRP						[2] AllocationRetentionPriority,

qoSNotificationControl	[3] BOOLEAN OPTIONAL,

reflectiveQos			[4] BOOLEAN OPTIONAL,

maxbitrateUL			[5] Bitrate OPTIONAL,

maxbitrateDL			[6] Bitrate OPTIONAL,

guaranteedbitrateUL		[7] Bitrate OPTIONAL,

guaranteedbitrateDL		[8] Bitrate OPTIONAL,

priorityLevel 			[9] INTEGER OPTIONAL,

averWindow				[10] INTEGER OPTIONAL,

maxDataBurstVol			[11] INTEGER OPTIONAL,

maxPacketLossRateDL 	[12] INTEGER OPTIONAL,

maxPacketLossRateUL 	[13] INTEGER OPTIONAL

}

AllocationRetentionPriority	::= SEQUENCE

{

priorityLevel 			[1] INTEGER,

preemptionCapability	[2] PreemptionCapability,

preemptionVulnerability	[3] PreemptionVulnerability

}

AMFID	::= OCTET STRING (SIZE(6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

ChargingSessionIdentifier	::= OCTET STRING

-- See 3GPP TS 32.290 [57] for details.

AuthorizedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

aRP					[2] AllocationRetentionPriority,

priorityLevel 		[3] INTEGER OPTIONAL,

averWindow			[4] INTEGER OPTIONAL,

maxDataBurstVol		[5] INTEGER OPTIONAL

}

Bitrate	::= OCTET STRING

--

--  See 3GPP TS 29.571 [249] Bitrate data type.

--

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainers					[1] SEQUENCE OF UsedUnitContainer OPTIONAL,

uPFID								[2] NetworkFunctionName OPTIONAL

}

NGRANSecondaryRATType	::= OCTET STRING

--

-- "NR" or "EUTRA"

--

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

PDUSessionId 		::= INTEGER (0..255)

--

-- See 3GPP TS 29.571 [249] for details

--

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink				[4] DataVolumeOctets

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality				[0] NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName OPTIONAL,

networkFunctionIPv4Address			[2] IPAddress OPTIONAL,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL,

networkFunctionIPv6Address			[4] IPAddress OPTIONAL,

networkFunctionFQDN					[5] NodeAddress OPTIONAL

}

NetworkFunctionName	::= IA5String (SIZE(1..20))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkFunctionality	::= ENUMERATED

{

cHF			(0), 	-- this value is not used

sMF			(1),

aMF			(2),

sMSF		(3),

sGW		(4)

-- SGW is only applicable for interworking with EPC scenario when UE is connected to P-GW+SMF via EPC

}

NetworkSliceInstanceID	::= SEQUENCE

-- See S-NSSAI subclause 28.4.2 of TS 23.003 [200] for encoding.

{

sST			[0] SliceServiceType,

sD 			[1] SliceDifferentiator OPTIONAL

}

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUAddress 	::= SEQUENCE

{

pDUIPv4Address				[0] IPAddress OPTIONAL,

pDUIPv6AddresswithPrefix	[1] IPAddress OPTIONAL,

iPV4dynamicAddressFlag		[2] DynamicAddressFlag OPTIONAL,

iPV6dynamicPrefixFlag		[3] DynamicAddressFlag OPTIONAL

}

PDUSessionType		::= ENUMERATED

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

PreemptionCapability		::= ENUMERATED

{

nOT-PREEMPT			(0),

mAY-PREEMPT			(1)

}

PreemptionVulnerability		::= ENUMERATED

{

nOT-PREEMPTABLE		(0),

pREEMPTABLE			(1)

}

QoSFlowId		::= INTEGER

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoamingTrigger 		::= SEQUENCE

{

trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeOctets OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

ServingNetworkFunctionID	::= SEQUENCE

{

servingNetworkFunctionInformation	[0] NetworkFunctionInformation,

aMFIdentifier						[1] AMFID OPTIONAL

}

SessionAMBR	::= SEQUENCE

{

ambrUL				[1] Bitrate,

ambrDL				[2] Bitrate

}

SliceServiceType ::= INTEGER (0..255)

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SliceDifferentiator		::= OCTET STRING (SIZE(3))

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SSCMode	::= INTEGER

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3)

}

-- See 3GPP TS 29.501 [248] for details.

SMdeliveryReportRequested ::= ENUMERATED

{

yes		(0),

no		(1)

}

SMFTrigger				::= INTEGER

{

startOfPDUSession							(1),

atartOfServiceDataFlowNoSession			(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

aMFChange									(102),

presenceReportingAreaChange				(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit				(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unusedQuotaTimerExpiry						(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601),

-- interworking with EPC

eCGIChange									(700),

tAIChange									(701),

handoverCancel								(702),

handoverStart								(703),

handoverComplete							(704)

}

-- See TS 32.255 [15] for details.

SMServiceType 	::= INTEGER

{

-- 0 to 10 VAS4SMS Short Message, see TS TS 22.142 [x] for details

contentProcessing					(0),

forwarding							(1),

forwardingMultipleSubscriptions		(2),

filtering 							(3),

receipt								(4),

networkStorage						(5),

toMultipleDestinations				(6),

virtualPrivateNetwork				(7),

autoreply							(8),

personalSignature					(9),

deferredDelivery					(10)

-- 11 to 99	Reserved for 3GPP defined SM services

-- 100 to 199 Vendor specific SM services

}

SMReplyPathRequested	::= ENUMERATED

{

noReplyPathSet 			(0),

replyPathSet			(1)

}

SubscribedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

aRP					[2] AllocationRetentionPriority OPTIONAL,

priorityLevel 		[3] INTEGER OPTIONAL

}

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger

}

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9] LocalSequenceNumber OPTIONAL,

ratingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL,

quotaManagementIndicator			[12] BOOLEAN OPTIONAL,

furnishChargingInformation			[13] OCTET STRING OPTIONAL

}

DnnSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.502 [250] for more information

--

{

uEorNetworkProvidedSubscriptionVerified				(0),

uEProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

.#END

End of change