3GPP TSG-SA5 Meeting #128	S5-197392

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0089 | rev | 1 | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of state machine |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current Nchf relies on the state machine described by Diameter but since this is no longer relevant there is a need to have this described within the Nchf. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause for a description of the state machine implied by Nchf |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interpretation of when a message can or should be sent is left for interpretation and may cause interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-196380 initial CR |  |  |  |  |  |  |  |  |

First change

### 5.4.x	Session State Machine

The following state machine is to be observed by both the CTF and CHF, the main use of this is to know when it can send what information.

The table below defines the different states which apply to a charging session for the CHF.

Table 5.4.x.1: Description of states in the CHF

| State: | Description: |
| --- | --- |
| Idle | No charging resource created and new requests can be received |
| Created | A charging resource has been created which can be updated |
| Closed | The charging resource has been closed and can no longer be updated |

The table below defines the different states which apply to a charging session for the CTF.

Table 5.4.x.2: Description of states in the CTF

| State: | Description: |
| --- | --- |
| Idle | No trigger received and no session started |
| Waiting | Session started and waiting for an answer to a request |
| Active | Session started and monitored |
| Terminated | Session has terminated |

For the CHF the triggers are the reception of the requests from the CTF, but for the CTF

Table 5.4.x.3: Description of triggers in the CTF

| CTF Event: | Description: |
| --- | --- |
| Start of Multiple Service session | There is new Multiple Service session detected in the CTF that needs charging. |
| Trigger for the Multiple Service session | The information for the Multiple Service session is changed |
| Start of Rating Group | Within the Multiple Service session there is a service started that require a new Rating Group. |
| Trigger for Rating Group | The information for a monitored Rating Group is changed |
| Threshold for Rating Group | The limits set for reporting for the Rating Group |
| Re-authorization for Rating Group | The CHF has demanded a re-authorization of a Rating Group |
| End of Rating Group | There are no services using a specific Rating Group |
| Re-authorization for Multiple Service Session | The CHF has requested that the Multiple Service session to be terminated |
| End of Multiple Service session | The Multiple Service session has terminated |

#### 5.4.x.1	CTF State transitions

The table below defines the different states transitions that apply at the initial part of a charging session for the CTF.

Table 5.4.x.1.1: State changes at initial in the CTF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Idle | Start of Multiple Service session | Send Charging Data Request [Initial] Start request timer | Waiting |
| Idle | Start of Multiple Service session, event based | Send Charging Data Request [Event] May grant service | Waiting |
| Idle | Start of Rating Group | Send Charging Data Request [Initial] Start request timer | Waiting |
| Idle | Re-authorization for Rating Group | Send Charging Notify Response with re-authorization rejected | Idle |
| Idle | Re-authorization for Multiple Service Session | Send Charging Notify Response with re-authorization rejected | Idle |
| Idle | End of Multiple Service session | Send Charging Data Request [Termination] Start request timer | Waiting |
| Waiting | Charging Data Response, granted units | Stop request timer Grant service Start monitoring of the service | Active |
| Waiting | Charging Data Response, no granted units | Stop request timer Reject service | Terminated |
| Waiting | Trigger for the Multiple Service session | Queue Charging Data Request [Update] | Waiting |
| Waiting | Start of Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Trigger for Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Threshold for Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Re-authorization for Rating Group | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | End of Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Re-authorization for Multiple Service Session | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | End of Multiple Service session | Queue Charging Data Request [Termination] | Waiting |
| Waiting | Request timeout, with retry | Send Charging Data Request [Initial] Start request timer | Waiting |
| Waiting | Request timeout, without retry | Send Charging Data Request [Termination] Start request timer | Waiting |

The table below defines the different states transitions that apply at the update part of a charging session for the CTF.

Table 5.4.x.1.2: State changes at update in the CTF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Active | Trigger for Multiple Service session | Send Charging Data Request [Update] Start request timer | Waiting |
| Active | Start of Rating Group | Send Charging Data Request [Update] Start request timer | Waiting |
| Active | Trigger for Rating Group | Send Charging Data Request [Update] Start request timer | Waiting |
| Active | Threshold for Rating Group | Send Charging Data Request [Update] Start request timer | Waiting |
| Active | End for Rating Group | Send Charging Data Request [Update] Start request timer | Waiting |
| Active | Re-authorization for Rating Group | Send Charging Notify Response with re-authorization accepted | Active |
| Active | Re-authorization for Multiple Service Session | Send Charging Notify Response with re-authorization accepted | Active |
| Waiting | Charging Data Response, granted units | Stop request timer Grant service Start monitoring of the service | Active |
| Waiting | Charging Data Response, no granted units | Stop request timer Reject service | Terminated |
| Waiting | Trigger for the Multiple Service session | Queue Charging Data Request [Update] | Waiting |
| Waiting | Start of Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Trigger for Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Threshold for Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Re-authorization for Rating Group | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | End of Rating Group | Queue Charging Data Request [Update] | Waiting |
| Waiting | Re-authorization for Multiple Service Session | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | End of Multiple Service session | Queue Charging Data Request [Termination] | Waiting |
| Waiting | Request timeout, with retry | Send Charging Data Request [Update] Start request timer | Waiting |
| Waiting | Request timeout, without retry | Send Charging Data Request [Termination] Start request timer | Waiting |

The table below defines the different states transitions that apply at the termination part of a charging session for the CTF.

Table 5.4.x.1.2: State changes at termination in the CTF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Active | End of Multiple Service session | Send Charging Data Request [Termination] Start request timer | Waiting |
| Waiting | Charging Data Response | Stop request timer | Terminated |
| Waiting | Re-authorization for Rating Group | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | Re-authorization for Multiple Service Session | Charging Notify Response with Charging Data Request ongoing | Waiting |
| Waiting | End of Multiple Service session | Queue Charging Data Request [Termination] | Waiting |
| Waiting | Request timeout, with retry | Send Charging Data Request [Termination] Start request timer | Waiting |
| Waiting | Request timeout, without retry | Stop request timer | Terminated |

#### 5.4.x.2	CHF State transitions

The table below defines the different states transitions that apply at the initial part of a charging session for the CHF.

Table 5.4.x.2.1: State changes at initial in the CHF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Idle | Receive Charging Data Request [Initial] | Open CDR Send Charging Data Response [Initial] | Created |
| Idle | Receive Charging Data Request [Event] | Open and close CDR Send Charging Data Response [Event] | Closed |
| Idle | Receive Charging Data Request [Update] | Open CDR Send Charging Data Response [Update] | Created |
| Idle | Receive Charging Data Request [Termination] | Open and close CDR Send Charging Data Response [Termination] | Closed |

The table below defines the different states transitions that apply at the update part of a charging session for the CHF.

Table 5.4.x.2.2: State changes at update in the CHF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Created | Receive Charging Data Request [Initial] | Resend Charging Data Response [Initial] | Created |
| Created | Receive Charging Data Request [Event] | Send Charging Data Response [Event] with an error Update and close CDR | Closed |
| Created | Receive Charging Data Request [Update] | Send Charging Data Response [Update] Update CDR | Created |

The table below defines the different states transitions that apply at the termination part of a charging session for the CHF.

Table 5.4.x.2.2: State changes at termination in the CHF

| Initial State | Event | Action | New State |
| --- | --- | --- | --- |
| Created | Receive Charging Data Request [Termination] | Close CDR Send Charging Data Response [Termination] | Closed |
| Closed | Receive Charging Data Request [Initial] | Open and close CDR Send Charging Data Response [Initial] with error | Closed |
| Closed | Receive Charging Data Request [Event] | Open and close CDR Send Charging Data Response [Event] with error | Closed |
| Closed | Receive Charging Data Request [Update] | Open and close CDR Send Charging Data Response [Update] with error | Closed |
| Closed | Receive Charging Data Request [Termination] | Resend Charging Data Response [Termination] | Closed |

End of changes