3GPP TSG-SA5 Meeting # 128	S5-197438

# Zhuhai,China,  18 - 22 November 2019											Revision of S5-19xxxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.421 | CR |  | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New structure for 5G MDT requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MDT requirements for 5G need to be added to this specification. In current revision of TS, the clause for Trace requirement covers MDT requirements too. MDT support for 5G introduces new MDT specific requirements. For increasing clarity and visibility, there should be one place for collecting all MDT related data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-structure the specification to increase visibility of Trace vs MDT requirements. All needed sub-clauses are created and for common parts refer to Trace sections. Correct the format of 6.2.1 (from Heading 2-> Heading 3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not haveing all MDT requirement will cause ambiguity and confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.A, 6.2.B, 6.2.C, 6.2.D, 6.2.E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 6.2	Specification level requirements

## 6.2.1	Logged MDT and Immediate MDT requirements

All requirements are valid for Logged MDT and Immediate MDT functionality if not mentioned otherwise:

REQ-MDT-FUN-01	It shall be possible to collect UE measurements based on one or more IMEI(SV) number.

REQ-MDT-FUN-02	It shall be possible to collect UE measurements based on one or more IMSI number.

REQ-MDT-FUN-03	It shall be possible to collect UE measurement logs preceding and following a particular event (e.g. radio link failure).

REQ-MDT-FUN-04	Each UE measurement result shall be linked to a time stamp. Accuracy of time information (absolute time, relative time) is FFS in RAN. (Editor’s Note:  FFS in RAN)

REQ-MDT-FUN-05	The solutions for collecting UE measurements for the purpose of minimization of drive tests shall be able to work independently from SON support in the network.

REQ-MDT-FUN-06	It shall be possible to collect UE measurements in one or more cells or TA/RA/LA.

REQ-MDT-FUN-07	It shall be possible to collect UE measurements based on one or more IMSI in one or more cells or TA/RA/LA.

REQ-MDT-FUN-08	It shall be possible to collect UE measurements based on one or more IMEI(SV) in one or more cells or TA/RA/LA.

REQ-MDT-FUN-09	It shall be possible to configure UE measurement types,and triggering conditions under which UE measurements would be collected for MDT.

REQ-MDT-FUN-10	Void.

REQ-MDT-FUN-11	It shall be possible to configure the condition of MDT data collection based on certain device capability information in one or more cells or in TA/RA/LA.

REQ-MDT-FUN-12	It shall be possible to configure MDT data collection based on one or more IMSI in one or more cells or TA/RA/TA with a set of device capability information.

REQ-MDT-FUN-13	It shall be possible to configure MDT data collection based on one or more IMEI(SV) in one or more cells or TA/RA/TA with a set of device capability information.

REQ-MDT-FUN-14	It shall be possible to configure MDT data collection based on one or more IMEI(SV) with a set of device capability information.

REQ-MDT-FUN-15	It shall be possible to configure MDT data collection based on one or more IMSI with a set of device capability information.

REQ-MDT-FUN-16	It shall be possible to activate a Trace Session for MDT data collection (or UE measurement collection for MDT purpose) independently from other mobility related performance measurements and call trace collection for both management based and signalling based.

REQ-MDT-FUN-17	It shall be possible to deactivate MDT data collection by using Trace Reference for both management based and signalling based.

REQ-MDT-FUN-18	It shall be possible to create a combine Trace Session for UE measurement collection and for subscriber and equipment/cell trace.

REQ-MDT-FUN-19	Void.

REQ-MDT-FUN-20	MDT activation for both management based and signalling based shall be supported for a UE belonging to any PLMN of the same Operator.

REQ-MDT-FUN-21	MDT data collection shall continue if a user is changing PLMN and the target PLMN is owned by the same operator.

REQ-MDT-FUN-22	It shall be possible to collect positioning data related to UE measurements, which can be either geographical coordinates or raw positioning measurements sufficient to be input for a post processing positioning algorithm.

REQ-MDT-CON-23	It shall be possible for management system to correlate MDT UE measurements with location information.

NOTE:	There may be regulatory obligation to delete MDT data after processing.

REQ-MDT-FUN-24	The PLMN where TCE collecting MDT data resides shall match the RPLMN of the UE providing the MDT data.

REQ-MDT-FUN-25	In the case of Area Based MDT the MOP shall be able to select UEs according to the POP intention.

REQ-MDT-FUN-26	The recorded Subscriber and Equipment Trace data related to a particular POP shall contain information so that if can be sent to that POP.

Next change

### 6.2.2	Logged MBSFN MDT requirements

Requirements in section 6.2.1 that apply to MBSFN MDT are as follows:

REQ-MDT-FUN-01

REQ-MDT-FUN-02

REQ-MDT-FUN-03

REQ-MDT-FUN-04

REQ-MDT-FUN-05

REQ-MDT-FUN-06

REQ-MDT-FUN-07

REQ-MDT-FUN-08

REQ-MDT-FUN-09

REQ-MDT-FUN-11

REQ-MDT-FUN-12

REQ-MDT-FUN-13

REQ-MDT-FUN-14

REQ-MDT-FUN-15

REQ-MDT-FUN-20

REQ-MDT-FUN-21

REQ-MDT-FUN-24

REQ-MDT-FUN-25

REQ-MDT-MBSFN-FUN-1 It shall be possible to activate a Session for Logged MBSFN MDT data collection independently from other mobility related performance measurements and call trace collection for both management based and signalling based.

REQ-MDT-MBSFN-FUN-2 It shall be possible to configure MBSFN MDT data collection based on certain device capability information in specific MBSFN area(s).

REQ-MDT-MBSFN-FUN-3 It shall be possible to deactivate Logged MBSFN MDT data collection by using Trace Reference for both management based and signalling based.

REQ-MDT-MBSFN-FUN-4 It shall not be possible to combine Logged MBSFN MDT session with any other trace sessions.

Next change

## 6.2.A	General MDT requirements

The general high-level requirements for MDT, common to both Management activation/deactivation and Signalling Based activation/deactivation, are the same as for general trace requirements, described in clause 5.1

## 6.2.B	Requirements for MDT data

The high-level requirements for MDT data, common to both Management activation/deactivation and Signalling Based Activation/Deactivation, are the same as for trace data requirements, described in clause 5.2.

## 6.2.C	Requirements for MDT activation

### 6.2.C.1	Requirements for MDT Session activation

The high-level requirements for MDT Session activation, common to both Management activation and Signalling based activation) are the same as the requirements for Trace Session activation described in clause 5.3.1

### 6.2.C.2	Requirements for starting MDT Recording Session

The high-level requirements for starting an MDT Recording Session, common to both Management activation and Signalling based activation) are the same as the requirements for Starting a Trace Recording Session described in clause 5.3.2

## 6.2.D	Requirements for MDT deactivation

### 6.2.D.1	Requirements for an MDT Session deactivation

The high-level requirements for MDT Session deactivation, common to both Management deactivation and Signalling based deactivation, are are the same as the requirements for Trace Session deactivation described in clause 5.4.1.

### 6.2.D.2	Requirements for stopping an MDT Recording Session

The high-level requirements for stopping an MDT Recording Session, common to both Management deactivation and Signalling based deactivation, are the same as the requirements for Stopping a Trace Recording Session described in clause 5.4.2.

## 6.2.E	Requirements for MDT Data reporting

The high-level requirements for MDT Data reporting, common to both Management activation/deactivation and Signalling Based activation/deactivation, are same as the requirements for Trace Data Reporting, described in clause 5.4.2.

End of changes