3GPP TSG-SA5 Meeting # 128	S5-197458

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0104 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add CHF-Controlled Quota Management functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Amdocs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFCQM |  |  |  |  |  | Date: |  |  | 2019-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the functionality of suspending and resuming quota management for a given rating group, controlled by CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify section 5.4.1 Add section 5.4.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CHF-controlled quota management capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.290

### 5.4.1	Re-authorization

The CHF (NF Service Producer) may trigger a re-authorization request and the NF Service Consumer shall report quota usage and resume quota management for the specified rating group (when previoulsy suspended) as described in section 5.4.x. The reason for the quota being reported shall be notified to the CHF (NF Service Producer). This is described under charging notification procedure in clause 5.3.2.4.

2nd change to TS 32.290

## 5.4	Other functionalities

### 5.4.X	CHF-Controlled Quota Management

CHF may instruct NF consumer to suspend quota management for a given rating group in the current PDU session by using a dedicated result code for the considered rating group in charging data reponse message. When NF consumer receives the instruction to suspend quota management it shall grant the service to the end user and report used units.

CHF may instruct NF consumer to resume quota magement for a given rating group for which quota management was privously suspended. CHF can instruct NF consumer to resume quota management in two ways:

- Explicitly by using a deicated result code in Charging Data Response

- Implicitly when receiveing Re-auatorization in charging notify message

In both cases, NF consumer shall resume quota management, report usage and request for quota.

End of change