3GPP TSG-SA5 Meeting # 128	S5-197484

# Zhuhai,China,  18 - 22 November 2019                                                           Revision of S5-197358

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0516 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add VoLTE information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | VBCLTE |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | P-GW cannot collect caller information and callee information for VoLTE service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce VoLTE information for volume based charging for VoLTE in PGW-CDR. Add Bindings of CDR parameter, Information Element and AVP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definition for volume based charging for VoLTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS32.298 CR 0784 TS32.299 CR 0823 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.1.3	FBC IP-CAN bearer charging data in P-GW (PGW-CDR)

If FBC is enabled and the collection of CDR data is enabled then the P-GW data specified in table 6.1.3.1 shall be available for each IP-CAN bearer when charging per IP-CAN session is not active, or for each IP-CAN session when charging per IP-CAN session is active.

Table 6.1.3.1: P-GW IP-CAN bearer data (PGW-CDR)

| Field | Category | Description |  |
| --- | --- | --- | --- |
| Record Type | M | P-GW IP-CAN bearer record. |  |
| Retransmission | OC | This parameter, when present, indicates that information from retransmitted Charging Data Request has been used in this CDR. |  |
| Served IMSI | C | IMSI of the served party, if available. |  |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEI | OC | The IMEI or IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |  |
| Served 3GPP2 MEID | OC | MEID of the served party's terminal equipment for 3GPP2 access. |  |
| Served MN NAI | OC | Mobile Node Identifier in NAI format (based on IMSI), if available. |  |
| P-GW Address used | M | The control plane IP address of the P-GW used. |  |
| P-GW Address IPv6 | OC | The control plane IPv6 address, in case of IPv4v6 dual stack, of the P-GW used. |  |
| Charging ID | M | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. This field holds the unique Charging Id in the PMIP case. When charging per IP-CAN session is active, this field holds the Charging Id of the EPS default bearer. It is the same as the PDN Connection Charging Id. |  |
| Charging per IP-CAN Session Indicator | Oc | This field indicates whether charging per IP-CAN session is active. If this field is not present, charging per IP-CAN session is not active. |  |
| PDN Connection Charging Id | OM | This field holds the explicit Charging Id for the PDN connection. When NBIFOM is supported, the PGW will assign an explicit PDN Connection Charging ID for the PDN connection. Otherwise the field holds the Charging Id of the EPS default bearer. This field is used to identify different records belonging to same PDN connection. |  |
| Serving node Address | M | List of SGSN/S-GW/TWAG control plane IP addresses, or the ePDG address used during this record. |  |
| Serving node IPv6 Address | OC | List of SGSN/S-GW/TWAG control plane IPv6 addresses, in case of IPv4v6 dual stack, used during this record. |  |
| Serving node Type | M | List of serving node types in control plane (SGSN, SGW, ePDG, AGW, TWAG). The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |  |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the P-GW. |  |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |  |
| PDP/PDN Type | OM | PDP/PDN type, i.e. IPv4, IPv6, IPv4v6 , or PDP type PPP. |  |
| PDP/PDN Type extension | OM | This field indicates Non-IP PDN type. |  |
| SGi PtP Tunnelling Method | OC | This field indicates whether SGi PtP tunnelling method based on UDP/IP or other methods are used for this PDN connection when non-IP PDN connection. |  |
| SCS/AS Address | OC | This field holds the Address of SCS/AS for SGi PtP tunnelling. |  |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, i.e. IPv4 address when PDP/PDN Type is IPv4 or IPv6 prefix when PDP/PDN Type is IPv6 or IPv4v6, or assigned IP address if any when PDP/PDN Type extension is Non-IP. This parameter shall be present except: - when both the PDP type is PPP and dynamic IP-CAN bearer address assignment is used or - when PDP/PDN Type extension is Non-IP, and no IP address has been assigned by the PGW for the PDN connection. |  |
| Served PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. |  |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDP/PDN type is IPv4v6. |  |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if IPv4 address is static when PDN Type is IPv4, or if IPv6 address is static when PDN Type is IPv6 or IPv4v6. |  |
| Dynamic Address Flag extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |  |
| List of Traffic Data Volumes | OC | A list of changes in charging conditions for all bearers within the IP-CAN session. Each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Applicable only for IP-CAN bearer charging when IP-CAN session charging is active. See table 6.1.3.1 for a description of List of Traffic Data Volumes handling. |  |
| List of Service Data | OM | A list of changes in charging conditions for all service data flows within this IP-CAN bearer categorized per rating group or per combination of the rating group and service id or per combination of rating group, sponsor identity and application service provider identity. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Online charging information (PS Furnish Charging Information) may be added per each service data flow container in case it is sent by the OCS. External charging identifiers may be added per each service data flow container in case sent by the PCRF. Failure-Handling: This field shall be present in case P-GW triggers the Failure-Handling procedure. It shall indicate the Failure Handling scenario and the instant the Failure Action is triggered (see annex B). Scenarios: Continue/New Session; Continue/Ongoing Session; Retry&Terminate/Ongoing Session; Terminate/Ongoing Session. When charging per IP-CAN session is active, see table 6.1.3.2 for a description of List of Service Data handling. |  |
| Record Opening Time | M | Time stamp when IP-CAN bearer is activated in this P-GW or record opening time on subsequent partial records. |  |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |  |
| Last MS Time Zone | OC | This field contains the UE Time Zone the UE is located at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| Duration | M | Duration of this record in the P-GW. |  |
| Cause for Record Closing | M | The reason for the release of record from this P-GW. |  |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a single cause is applicable. . |  |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a set of causes is applicable. |  |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |  |
| Node ID | OM | Name of the recording entity. |  |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |  |
| APN Selection Mode | OM | An index indicating how the APN was selected. |  |
| Served MSISDN | OC | The primary MSISDN of the subscriber, if available. |  |
| User Location Information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |  |
| Last User Location Information | OC | This field contains the User Location Information of the UE at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| User Location Information Time | OC | This field contains the time at which the user location information was acquired, if available. |  |
| User CSG information | OC | This field contains the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |  |
| 3GPP2 User Location information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.212 [71] for 3GPP2 access, if available. |  |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN) , if available: - "TWAN identifier" as defined in TS 29.274 [210] for S2a GTP-based case, or, -"Access Network Identifier Option IE" as defined in 29.275 [211] for S2a PMIP-based case. |  |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |  |
| Presence Reporting Area Information | OC | This grouped field contains the Presence Reporting Area Identifier for the UE PDN connection, additional the list(s) of elements comprising the Presence Reporting Area for a UE-dedicated Presence Reporting Area, and the initial status of the UE presence in the Presence Reporting Area, if available. This field is not applicable to multiple PRA(s). |  |
| Charging Characteristics | M | The Charging Characteristics applied to the IP-CAN bearer. |  |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |  |
| IMS Signalling Context | OC | Included if the IP-CAN default bearer IM-CN Subsystem Signalling Flag is set, see TS 23.060 [201] is used for IMS signalling. |  |
| Serving node PLMN Identifier | OM | This fields holds the PLMN Identifier (MCC and MNC) serving the UE during this record, as received in the "Serving Network” IE over S5/S8 or retrieved from the RAI received over Gn/Gp. When the Serving node is TWAG, this field includes the PLMN identifier used for UE authentication, i.e. the VPLMN in roaming case, and the HPLMN in non-roaming case. |  |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |  |
| PS Furnish Charging Information | OC | Online charging session specific information |  |
| CAMEL Information | OC | Set of CAMEL information related to IP-CAN bearer, if available. This field applies only for GPRS. When charging per IP-CAN session is active, this field is not applicable. |  |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. When NBIFOM is accepted by PCRF, this field holds the first default bearer’s RAT type when charging per IP-CAN session is active. |  |
| Start Time | OC | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session starts. |  |
| Stop Time | OC | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session is terminated. |  |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |  |
| QoS Information | OC | This field indicates the APN-AMBR uplink and downlink information for the IP-CAN session. Applicable only when charging per IP-CAN session is active. |  |
| NBIFOM Support | OC | This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session. |  |
| NBIFOM Mode | OC | This field indicates the NBIFOM mode selected for the PDN connection, when NBIFOM is supported for the IP-CAN session. This field is only present when NBIFOM is accepted. |  |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer, i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |  |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME during this record |  |
| APN Rate Control | Oc | This field holds the APN Rate Controls enforced in the PGW during this record. |  |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |  |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |  |
| 3GPP PS Data Off Status | Oc | This field holds the 3GPP PS Data Off Status. |  |
| List of RAN Secondary RAT Usage Reports | OC | A list of containers with volumes reported from RAN for secondary RAT usage, each container is time stamped. |  |

Table 6.1.3.2 contains special handling for the fields in the List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active and IP-CAN bearer charging is performed.

Table 6.1.3.2: List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| Data Volume Uplink | OC | This field includes the number of octets transmitted during the use of the packet data services in the uplink direction. |
| Data Volume Downlink | OC | This field includes the number of octets transmitted during the use of the packet data services in the downlink direction. |
| Change Condition | M | This field defines the reason for closing the container as specified in clause 5.2.1.10.1. |
| Change Time | M | This field is a time stamp, which defines the moment when the volume container is closed. |
| User Location Information | OC | This field contains the location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE is located and used during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "user location change". |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP or TCP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information the ePDG may have received from the 3GPP AAA server about the UE as "WLAN location information" defined in TS 29.274 [210]. |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN), if available: - as "TWAN identifier" defined in TS 29.274 [210] for S2a GTP-based case, or, - as "Access Network Identifier Option IE" defined in 29.275 [211] for S2a PMIP-based case. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area. This field is not applicable to multiple PRA(s). |
| List of Presence Reporting Area Information | OC | This field contains a list of Presence Reporting Area Identifier(s) for the UE PDN connection, and the Presence Reporting Area(s) status (UE presence or not, inactive), differentiated between PCRF and OCS subscription. |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer. First container for each QCI/ARP pair includes this field. In following containers this field is present if previous change condition is "QoS change". This field includes: QCI and ARP. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Charging Id | OC | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. Charging Id is generated by P-GW at IP-CAN bearer activation and is included in all containers in order to identify the containers which pertain to the IP-CAN bearer. |
| RAT Type | OC | This field contains the RAT type for the current IP-CAN bearer. |
| Access Availability Change Reason | OC | This field indicates the reason why the availability of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable. This field is only present when one access of a multi-access PDN connection becomes unusable. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. |
| 3GPP PS Data Off Status | OC | This field contains the status of the UE’s 3GPP PS Data off Status. |
| Serving PLMN Rate Control | OC | This field holds the Serving PLMN Rate Control used by the MME |
| APN Rate Control | OC | This field holds the APN Rate Controls enforced in the PGW |

Table 6.1.3.3 contains special handling for fields in the List of Service Data when charging per IP-CAN session is active. The complete set of fields in the List of Service Data is defined in TS 32.298 [51].

Table 6.1.3.3: List of Service Data in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| AF Record Information | OC | As specified in TS 32.298 [51]. |
| Charging Rule Base Name | OC | As specified in TS 32.298 [51]. |
| ADC Rule Base Name | OC | Not applicable to PGW-CDR. |
| Data Volume Downlink | OC | As specified in TS 32.298 [51]. |
| Data Volume Uplink | OC | As specified in TS 32.298 [51]. |
| Event Based Charging Information | OC | As specified in TS 32.298 [51]. |
| Local Sequence Number | OC | As specified in TS 32.298 [51]. |
| PS Furnish Charging Information | OC | As specified in TS 32.298 [51]. |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. This field includes: QCI and ARP. For non-GBR QCI values, this field also includes the MBR for the downlink and uplink direction. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Rating Group | M | As specified in TS 32.298 [51]. |
| Report Time | M | As specified in TS 32.298 [51]. |
| Result Code | OC | As specified in TS 32.298 [51]. |
| Service Condition Change | M | As specified in TS 32.298 [51]. Only those values specified for the events in clause 5.3.2.X apply. |
| Service Identifier | OC | As specified in TS 32.298 [51]. |
| Service Specific Info | OC | As specified in TS 32.298 [51]. |
| Serving Node Address | OC | As specified in TS 32.298 [51]. |
| Time of First Usage | OC | As specified in TS 32.298 [51]. |
| Time of Last Usage | OC | As specified in TS 32.298 [51]. |
| Tme Usage | OC | As specified in TS 32.298 [51]. |
| User Location Information | OC | As specified in TS 32.298 [51]. |
| 3GPP2 User Location Information | OC | As specified in TS 32.298 [51]. |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP or TCP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information the ePDG may have received from the 3GPP AAA server about the UE as "WLAN location information" defined in TS 29.274 [210]. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area as defined in TS 29.274[210]. This field is not applicable to multiple PRA(s). |
| List of Presence Reporting Area Information | OC | This field contains a list of Presence Reporting Area Identifier(s) for the UE PDN connection, and the Presence Reporting Area(s) status (UE presence or not, inactive), differentiated between PCRF and OCS subscription. |
| RAT Type | OC | This field contains the RAT type for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. |
| Sponsor Identity | OC | As specified in TS 32.298 [51]. |
| Application Service Provider Identity | OC | As specified in TS 32.298 [51]. |
| Time Quota Mechanism | OC | As specified in TS 32.298 [51]. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| 3GPP PS Data Off Status | OC | This field contains the status of the UE’s 3GPP PS Data Off Status. |
| Traffic Steering Policy Identifier Uplink | OC | This field contains the traffic steering policy identifier in the uplink direction. |
| Traffic Steering Policy Identifier Downlink | OC | This field contains the traffic steering policy identifier in the downlink direction. |
| Serving PLMN Rate Control | OC | This field holds the Serving PLMN Rate Control used by the MME |
| APN Rate Control | OC | This field holds the APN Rate Controls enforced in the PGW |
| VoLTE Information | OC | This field holds the caller and callee Information of VoLTE. |

Next change

6.5	Bindings for EPC offline charging

This clause aims to describe the mapping between the Service Information fields, AVPs and CDR parameter for EPC offline charging.

Table 6.5.1 describes the mapping of the Information Element, AVP and CDR parameter of ePDG-CDR, TWAG-CDR, SGW-CDR, PGW-CDR and TDF-CDR in EPC offline charging.

Table 6.5.1: Bindings of CDR parameter, Information Element and AVP

| CDR Parameter | Information Element | AVP |
| --- | --- | --- |
| Retransmission | - | - |
| Record Opening Time | - | - |
| Duration | - | - |
| Cause for Record Closing | - | - |
| Record Sequence Number | - | - |
| Local Record Sequence Number | - | - |
| IMS Signalling Context | - | - |
| CAMEL Information | - | - |
| Record Extensions | - | - |
|  | Service Information | Service-Information |
| Served IMSI | Subscriber Identifier | Subscription-Id |
| Served MN NAI | Subscriber Identifier | Subscription-Id |
| Served MSISDN | Subscriber Identifier | Subscription-Id |
|  | IMS Information | IMS-Information |
| Record Type | Node Functionality | Node-Functionality |
|  | PS Information | PS-Information |
| - | Supported Features | Supported-Features |
| Charging Id | Charging Id | 3GPP-Charging-Id |
| Charging per IP-CAN Session Indicator | Charging per IP-CAN Session Indicator | Charging-Per-IP-CAN-Session-Indicator |
| Node ID | Node Id | Node-ID |
| PDN Connection Charging Id | PDN Connection Charging Id | PDN-Connection-Charging-ID |
| PDP/PDN Type | PDP/PDN Type | 3GPP-PDP-Type |
| PDP/PDN Type extension | PDP/PDN Type | 3GPP-PDP-Type |
| SGi PtP Tunnelling Method | SGi PtP Tunnelling Method | SGi-PtP-Tunnelling-Method |
| SCS/AS Address | SCS/AS Address | SCS-AS-Address |
| Served PDP/PDN Address | PDP/PDN Address | PDP-Address |
| Served PDP/PDN Address prefix length | PDP/PDN Address prefix length | PDP-Address-Prefix-Length |
| Served PDP/PDN Address extension | PDP/PDN Address | PDP-Address |
| Dynamic Address Flag | Dynamic Address Flag | Dynamic-Address-Flag |
| Dynamic Address Flag extension | Dynamic Address Flag Extension | Dynamic-Address-Flag-Extension |
| EPC QoS Information/Qos Information | Negotiated QoS Profile | Qos-Information |
| Serving Node Address | Serving Node Address | SGSN-Address |
| Serving node IPv6 Address | Serving Node Address | SGSN-Address |
| Serving Node Type | Serving Node Type | Serving-Node-Type |
| S-GW Change | SGW Change | SGW-Change |
| PGW Address Used | PGW Address | GGSN-Address |
| PGW IPv6 Address | PGW Address | GGSN-Address |
| TDF Address Used | TDF Address | TDF-IP-Address |
| TDF IPv6 Address | TDF Address | TDF-IP-Address |
| S-GW Address used | SGW Address | SGW-Address |
| S-GW Address IPv6 | SGW Address | SGW-Address |
| ePDG Address Used | ePDG Address | ePDG-Address |
| ePDG IPv6 Address | ePDG Address | ePDG-Address |
| TWAG Address Used | TWAG Address | TWAG-Address |
| TWAG IPv6 Address | TWAG Address | TWAG-Address |
| - | CG Address | CG-Address |
| - | IMSI MCC MNC | 3GPP-IMSI-MCC-MNC |
| IMSI Unauthenticated Flag | IMSI Unauthenticated Flag | IMSI-Unauthenticated-Flag |
| PGW PLMN Identifier | PGW MCC MNC | 3GPP-GGSN-MCC-MNC |
| TDF PLMN Identifier | TDF MCC MNC | 3GPP-GGSN-MCC-MNC |
| - | NSAPI | 3GPP-NSAPI |
| Access Point Name Network Identifier | Called Station Id | Called-Station-Id |
| - | Session Stop Indicator | 3GPP-Session-Stop-Indicator |
| APN Selection Mode | Selection Mode | 3GPP-Selection-Mode |
| Charging Characteristics | Charging Characteristics | 3GPP-Charging-Characteristics |
| Charging Characteristics Selection Mode | Charging Characteristics Selection Mode | Charging-Characteristics-Selection-Mode |
| Serving node PLMN Identifier | Serving Node MCC MNC | 3GPP-SGSN-MCC-MNC |
| CN Operator Selection Entity | CN Operator Selection Entity | CN-Operator-Selection-Entity |
| MS Time Zone | MS Time Zone | 3GPP-MS-Time-Zone |
| Last MS Time Zone | MS Time Zone | 3GPP-MS-Time-Zone |
| User Location Information | User Location Info | 3GPP-User-Location-Info |
| User Location Information Time | User Location Info Time | User-Location-Info-Time |
| Last User Location Information | User Location Info | 3GPP-User-Location-Info |
| User CSG Information | User CSG Information | User-CSG-Information |
| 3GPP2 User Location information | 3GPP2 User Location Info | 3GPP2-BSID |
| TWAN User Location Information | TWAN User Location Information | TWAN-User-Location-Info |
| UWAN User Location Information | UWAN User Location Information | UWAN-User-Location-Info |
| Presence Reporting Area Information | Presence Reporting Area Information | Presence-Reporting-Area-Information |
| RAT Type | RAT Type | 3GPP-RAT-Type |
| PS Furnish Charging Information | Furnish Charging Information | PS-Furnish-Charging-Information |
| - | Offline Charging | Offline-Charging |
| - | PDP Context Type | PDP-Context-Type |
| List of Traffic Data Volumes | Traffic data volumes | Traffic-Data-Volumes |
| Data Volume Uplink | - | Accounting-Input-Octets |
| Data Volume Downlink | - | Accounting-Output-Octets |
| Change Condition | Change Condition | Change-Condition |
| Change Time | - | Change-Time |
| Access Availability Change Reason | - | Access-Availability-Change-Reason |
| List of Service Data | Service data container | Service-Data-Container |
| AF record information | AF Correlation Information | AF-Correlation-Information |
| Charging Rule Base Name | Charging Rule Base Name | Charging-Rule-Base-Name |
| ADC Rule Base Name | ADC Rule Base Name | ADC-Rule-Base-Name |
| Rating Group | Rating Group | Rating-Group |
| Service Identifier | Service Identifier | Service-Identifier |
| Service Specific Info | Service Specific Info | Service-Specific-Info |
| Report Time | - | Change-Time |
| Service Condition Change | - | Change-Condition |
| Result Code | - | Result-Code |
| Time of First Usage | - | Time-First-Usage |
| Time of Last Usage | - | Time–Last-Usage |
| Time Usage | - | Time-Usage |
| Traffic Steering Policy Identifier DL | Traffic Steering Policy Identifier DL | Traffic-Steering-Policy-Identifier-DL |
| Traffic Steering Policy Identifier UL | Traffic Steering Policy Identifier UL | Traffic-Steering-Policy-Identifier-UL |
| - | User Equipment Info | User-Equipment-Info |
| Served IMEI | Terminal Information | Terminal-Information |
| Served 3GPP2 MEID | Terminal Information | Terminal-Information |
| Start Time | Start Time | Start-time |
| Stop Time | Stop Time | Stop-time |
| Cause for Record Closing | Change Condition | Change-Condition |
| Diagnostics | Diagnostics | Diagnostics |
| Enhanced Diagnostics | Enhanced Diagnostics | Enhanced-Diagnostics |
| Low Priority Indicator | Low Priority Indicator | Low-Priority-Indicator |
| NBIFOM Support | NBIFOM Support | NBIFOM-Support |
| NBIFOM Mode | NBIFOM Mode | NBIFOM-Mode |
| CP CIoT EPS Optimisation Indicator | CP CIoT EPS Optimisation indicator | CP-CIoT-EPS-Optimisation-Indicator |
| UNI PDU CP Only Flag | UNI PDU CP Only Flag | UNI-PDU-CP-Only-Flag |
| Serving PLMN Rate Control | Serving PLMN Rate Control | Serving-PLMN-Rate-Control |
| MO exception data counter | MO exception data counter | RRC-Cause-Counter |
| APN Rate Control | APN Rate Control | APN-Rate-Control |
| VoLTE Information | VoLTE Information | VoLTE-Information |
| 3GPP PS Data Off Status | 3GPP PS Data Off Status | 3GPP-PS-Data-Off-Status |
| List of RAN Secondary RAT Usage Reports | RAN Secondary RAT Usage Reports | RAN-Secondary-RAT-Usage Report |

Table 6.5.2 describes the mapping of the Information Element, AVP and the CDR parameter of S-SMO/S-SMT-CDR in EPC offline charging for SMS.

Table 6.5.2: Bindings of S-SMO/S-SMT-CDR parameter, Information Element and AVP

| CDR Parameter | Information Element | AVP |
| --- | --- | --- |
| Retransmission | - | - |
| Local Record Sequence Number | - | - |
| CAMEL Information | - | - |
| Record Extensions | - | - |
|  | Service Information | Service-Information |
| Served IMSI | Subscriber Identifier | Subscription-Id |
| Served MSISDN | Subscriber Identifier | Subscription-Id |
|  | IMS Information | IMS-Information |
| Record Type | Node Functionality | Node-Functionality |
|  | PS Information | PS-Information |
| Recording Entity | MME Number for MT SMS | MME-Number-for-MT-SMS |
| MME Name | MME Name | MME-Name |
| MME Realm | MME Realm | MME-Realm |
|  | SMS Information | SMS-Information |
| Destination Number | Recipient-Address | Recipient-Address |
| Service Centre | SMSC-Address | SMSC-Address |
|  | MMS Information | MMS-Information |
| Originating Address | Originator Address | Originator-Address |
| Event Time Stamp | Submission-Time | Submission-Time |
| Message Reference | Message Id | MessageId |

NOTE:	The whole set of ePDG-/TWAG-/MME-/SGW-/PGW-/TDF-CDR parameters is described in the TS 32.298 [51].

End of changes