3GPP TSG-SA5 Meeting # 128	S5-197597

# Zhuhai, China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0073 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Heartbeat |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 and Stage 3 for the notifyHeartbeat notification are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Stage 2 and Stage 3 for the notifyHeartbeat notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Heartbeat would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.X (new), 12.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-197052 |  |  |  |  |  |  |  |  |

First change

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration ; Provisioning; ".

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Telecommunication management; Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://docs.onap.org/en/latest/_downloads/2c2b5962df52a0c1f2862f3bba3d67c7/CommonEventFormat_30.1_ONAP.json, accessed 21.03.2019.

[22]	Figure Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the figure: https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/ves7_1spec.html?highlight=heartbeatIntervalChange#resource-structure, accessed 21.03.2019).

[23]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#naming-standards-for-eventname, accessed 11.04.2019).

[24]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#datatype-commoneventheader, accessed 11.04.2019).

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[x]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://onap.readthedocs.io/en/latest/submodules/vnfrqts/requirements.git/docs/Chapter8/ves7_1spec.html#datatype-heartbeatfields, accessed 06.11.2019).

Next change

# 10	Management services – Stage 2

## 10.X	Heartbeat

### 10.X.1	Operations and notifications

#### 10.X.1.1	Notification notifyHeartbeat

##### 10.X.1.1.1	Definition

This notification notifies the subscribed consumer(s) that the MnS provider heartbeat period has expired or that a MnS consumer requested the emission of an immediate heartbeat notification.

##### 10.X.1.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | It shall carry the ManagedEntity class name. | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | It shall carry the DN of the ManagedEntitiy. | It specifies a new instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| eventTime | M | It indicates the time at which the notification is emitted. | The semantics of Generalised Time specified by ITU-T [17] shall be used here. |
| notificationId | M | This is an identifier for the notification, | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant. It uniquely identifies the notification from other notifications generated by the subject MOI. |
| systemDN | C | It shall carry the DN of management service providers. | - |
| heartbeatNtfPeriod | M | ManagedEntity. heartbeatNtfPeriod | The value of the ManagedEntity. heartbeatNtfPeriod |

##### 10.X.1.1.3	Triggering event

###### 10.X.1.1.3.1	From-state

stateBeforeHeartbeatNotification1 OR stateBeforeHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification1 | The internal countdown timer of the ManagedEntity instance emitting the notifyHeartbeat notification has reached the value ‘0’ (zero). |
| stateBeforeHeartbeatNotification2 | The value of the attribute triggerHeartbeatNtf of the ManagedEntity instance emitting the notifyHeartbeat notification is TRUE. |

###### 10.X.1.1.3.2	To-state

stateAfterOHeartbeatNotification1 OR stateAfterOHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification1 | If From-state is stateBeforeHeartbeatNotification1 then: the internal countdown timer of the ManagedEntity instance is reset to the value of its heartbeatNtfPeriod attribute. |
| stateAfterHeartbeatNotification2 | If From-state is stateBeforeHeartbeatNotification2 then: the value of the internal countdown timer of the ManagedEntity instance is not affected. |

Next change

# 12	Management services – Stage 3

## 12.X	Heartbeat

### 12.X.1	RESTful HTTP-based solution set

#### 12.X.1.1	Mapping of operations

N/A.

#### 12.X.1.2	Mapping of notifications

##### 12.X.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.X.1.2.1-1.

Table 12.X.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyHeartbeat | POST | /notificationSink | M |

##### 12.X.1.2.2	Notification "notifyHeartbeat"

The IS notification parameters are mapped to SS equivalents according to table 12.X.1.2.2-1.

Table 12.X.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | request body | href | uri-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| notificationId | request body | notificationId | notificationId-Type | M |
| systemDN | request body | systemDN | systemDN-Type | O |
| notificationType | request body | notificationType | notificationType-Type | M |
| heartbeatPeriod | request body | heartbeatPeriod | heartbeatPeriod-Type | M |
| triggerFlag | request body | triggerFlag | triggerFlag-Type | M |

### 12.X.2	RESTful HTTP-based solution set for integration with ONAP VES API

#### 12.X.2.1	Mapping of operations

N/A.

#### 12.X.2.2	Mapping of notifications

##### 12.X.2.2.1	Introduction

###### 12.X.2.2.1.1	General

The 3GPP IS heartbeat notifications are mapped to SS equivalents according to table 12.X.2.2.1.1-1.

Table 12.2.2.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyHeartbeat | POST | /eventListener | M |

###### 12.X.2.2.1.2	Notification parameter mapping principles

3GPP heartbeat notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields [21] – see Table 12.X.2.2.1.2-1, or to

- ONAP VES API heartbeat fields [x] – see mapping tables in the following clauses.

Table 12.X.2.2.1.2-1: Mapping of 3GPP IS heartbeat notification parameters to ONAP VES common event header fields

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES common event header field name | SQ in ONAP VES common event header | Remark |
| --- | --- | --- | --- | --- |
| no corresponding IS parameter | N/A | sourceId | O |  |
| notificationId | M | eventId | M |  |
| no corresponding IS parameter | N/A | eventName | M | See [23]. |
| eventTime | M | lastEpochMicrosec | M | Type conversion needed [21]. |
| no corresponding IS parameter | N/A | reportingEntityName | M | See [24]. |
| eventTime | M | startEpochMicrosec | M | Same value as lastEpochMicrosec. Type conversion needed [21]. |
| no corresponding IS parameter | N/A | domain | M | “heartbeat” |
| no corresponding IS parameter | N/A | priority | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sequence | M | Field not used by this solution set. Value ‘0’ [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sourceName | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | version | M | Version of the event header [21]. |
| no corresponding IS parameter | N/A | vesEventListenerVersion | M | Version of the VES Event Listener API [21]. |

The ONAP VES API commonEventHeader datatype consists of fields common to all events, as specified in [21].

NOTE 1: Table 12.X.2.2.1.2-1 has the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 10.2.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 10.2.1 of the present document;

- Corresponding VES common event header field name: name of the field of the VES common event header [21];

- SQ in ONAP VES common event header: specified in [21], where ‘M’ corresponds to ‘Required = Yes’ in [21], and ‘O’ corresponds to ‘Required = No’ in [21];

NOTE 2: In Table 12.X.2.2.1.2-1, some VES common event header fields have no corresponding IS parameter. In case those VES common event header field Support Qualifier is ‘M’ (as specified in [21]), the producer of the notification shall provide a value to these parameters. See column ‘Remark’ in Table 12.X.2.2.1.2-1.

NOTE 3: ONAP VES common event header fields which are both optional [21] and have no corresponding IS parameter are not listed in Table 12.X.2.2.1.2-1.

NOTE 4: The mapping tables in following clauses of clause 12.X.2 have the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 10.2.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 10.2.1 of the present document;

- SS parameter name (Corresponding ONAP VES heartbeatFields field name): name of the field in ‘heartbeatFields’;

- SQ: Support Qualifier in ‘heartbeatFields’. When the IS notification parameter Support Qualifier is ‘M’, this Support Qualifier is set to ‘M’. When the IS notification parameter Support Qualifier is ‘C’, this Support Qualifier is set to ‘O’. When the IS notification parameter Support Qualifier is ‘O’, this Support Qualifier is set to ‘O’.

##### 12.X.2.2.2	Notification notifyHeartbeat

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.X.2.2.1.2-1 and to table 12.X.2.2.2.1-1.

Table 12.X.2.2.2.1-1: Mapping of IS notification parameters to SS equivalents in ‘heartbeatFields’

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES heartbeatFields field name | SQ in ONAP VES heartbeatFields datatype | Remark |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | M | additionalFields | M | This is an array of key:value pairs. Struct < dN, systemDN, notificationType, triggerFlag > where: - objectClass, objectInstance are mapped to dN - notificationType = “notifyHeartbeat”. |
| systemDN | O |  |  |  |
| notificationType | M |  |  |  |
| triggerFlag | M |  |  |  |
| heartbeatPeriod | M | heartbeatInterval | M | Conversion from minutes to seconds is required. |
| no corresponding IS parameter | N/A | heartbeatFieldsVersion | M | Version of the heartbeatFields block as “#.#” where # is a digit. |

Next change

Annex A (normative): 
OpenAPI specification

## A.0	Introduction

This clause describes the capabilities of the service in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI document is represented in the JSON format option.

## A.X	Heartbeat

### A.X.1	Heartbeat

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.532 Heartbeat",

"version": "16.1.0",

"description": "OAS 3.0.1 specification of the Heartbeat"

},

"servers": [

{

"url": "http://{DN_prefix_authority_part}/{DN_prefix_remainder}/Heartbeat/v1610",

"variables": {

"DN_prefix_authority_part": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"DN_prefix_remainder": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/subscriptions": {

"post": {

"summary": "Create a subscription",

"description": "To create a subscription the representation of the subscription is POSTed on the /subscriptions collection resource.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Created\"). The representation of the newly created subscription resource shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

},

"callbacks": {

"notifyHeartbeat": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyHeartbeat-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

}

},

"delete": {

"summary": "Delete all subscriptions made with a specific consumerReferenceId",

"description": "The subscriptions are deleted by deleting the corresponding subscription resources. The resources to be deleted are identified with the path component of the URI pointing to the /subscription collection resource and filtering on the consumerReferenceId provided in the query part.",

"parameters": [

{

"name": "consumerReferenceId",

"in": "query",

"description": "Identifies the subscriptions to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/consumerReferenceId-QueryType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resources have been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/subscriptions/{subscriptionId}": {

"delete": {

"summary": "Delete a single subscription",

"description": "The subscription is deleted by deleting the corresponding subscription resource. The resource to be deleted is identified with the path component of the URI.",

"parameters": [

{

"name": "subscriptionId",

"in": "path",

"description": "Identifies the subscription to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/subscriptionId-PathType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"dateTime-Type": {

"type": "string",

"format": "date-Time"

},

"uri-Type": {

"type": "string"

},

"long-Type": {

"type": "string",

"format": "long"

},

"additionalText-Type": {

"type": "string"

},

"header-Type": {

"description": "Header used in notifications as notification header",

"type": "object",

"properties": {

"uri": {

"$ref": "#/components/schemas/uri-Type"

},

"notificationId": {

"$ref": "#/components/schemas/notificationId-Type"

},

"notificationType": {

"$ref": "#/components/schemas/notificationType-Type"

},

"eventTime": {

"$ref": "#/components/schemas/dateTime-Type"

}

}

},

"subscriptionId-PathType": {

"type": "string"

},

"filter-Type": {

"type": "string"

},

"notificationId-Type": {

"$ref": "#/components/schemas/long-Type"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyHeartbeat"

]

},

"subscription-ResourceType": {

"type": "object",

"properties": {

"consumerReference": {

"$ref": "#/components/schemas/uri-Type"

},

"timeTick": {

"$ref": "#/components/schemas/long-Type"

},

"filter": {

"$ref": "#/components/schemas/filter-Type"

}

}

},

"subscription-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"subscription-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"consumerReferenceId-QueryType": {

"$ref": "#/components/schemas/uri-Type"

},

"notifyHeartbeat-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

},

"heartbeatPeriod": {

"$ref": "#/definitions/heartbeatPeriod-Type"

},

"triggerFlag": {

"$ref": "#/definitions/triggerFlag-Type"

},

"mNSConsumerRef": {

"$ref": "#/definitions/mNSConsumerRef-Type"

},

"additionalProperties": false,

"required": [ "heartbeatPeriod", "triggerFlag" ]

},

"dN-Type": {

"type": "string"

},

"systemDN-Type": {

"type": "string"

},

"triggerFlag-Type": {

"type": "string",

"enum": [

"MnSConsumer",

"HBMnSProvider"

]

},

"hBMnSConsumerRef-Type": {

"type": "string"

}

}

}

}

}

}

}

}

### A.X.2	Heartbeat for integration with ONAP VES

See [21].

End