3GPP TSG-SA5 Meeting #128	S5-197683

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0161 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of charging rule base name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 6.2.2.1 and Table 6.2.2.2 wrongly includes “Charging Rule Base Name” in “PDU Session Information”, which does not align with clause 6.2.1.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “Charging Rule Base Name” from “PDU Session Information” group in Table 6.2.2.1 and Table 6.2.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistence may lead to different interpretation and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-197389 Initial CR |  |  |  |  |  |  |  |  |

First change

### 6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element |  | Node Type | SMF (FBC) |  | SMF (QBC) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | -UT- |  | -UT- |  |
| Subscriber Identifier |  |  | IUT- |  | IUT- |  |
| NF Consumer Identification |  |  | IUT- |  | IUT- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  |
| Notify URI |  |  | IU- |  | IU- |  |
| Triggers |  |  | -UT- |  | -UT- |  |
| Multiple Unit Usage |  |  | IUT- |  | - |  |
| Rating Group |  |  | IUT- |  | - |  |
| Requested Unit |  |  | IU-- |  | - |  |
| Used Unit Container |  |  | -UT- |  | - |  |
| Triggers |  |  | -UT- |  | - |  |
| PDU Container Information |  |  | -UT- |  | - |  |
| UPF ID |  |  | IUT- |  | IUT- |  |
| PDU Session Charging Information |  |  | IUT- |  | IUT- |  |
| Charging Id |  |  | IUT- |  | IUT- |  |
| User Information |  |  | IUT- |  | IUT- |  |
| User Location Info |  |  | IUT- |  | IUT- |  |
| UE Time Zone |  |  | IUT- |  | IUT- |  |
| Presence Reporting Area Information |  |  | -UT- |  | -UT- |  |
| PDU Session Information |  |  | IUT- |  | IUT- |  |
| PDU Session ID |  |  | IUT- |  | IUT- |  |
| Network Slice Instance Identifier |  |  | IUT- |  | IUT- |  |
| PDU Type |  |  | IUT- |  | IUT- |  |
| PDU Address |  |  | IUT- |  | IUT- |  |
| SSC Mode |  |  | IUT- |  | IUT- |  |
| SUPI PLMN ID |  |  | IUT- |  | IUT- |  |
| Serving Network Function ID |  |  | IUT- |  | IUT- |  |
| Serving CN PLMN ID |  |  | IUT- |  | IUT- |  |
| RAT Type |  |  | IUT- |  | IUT- |  |
| Data Network Name Identifier |  |  | IUT- |  | IUT- |  |
| DNN Selection Mode |  |  | IUT- |  | IUT- |  |
| Authorized QoS Information |  |  | IUT- |  | IUT- |  |
| Subscribed QoS Information |  |  | IUT- |  | IUT- |  |
| Authorized Session-AMBR |  |  | IUT- |  | IUT- |  |
| Subscribed Session-AMBR |  |  | IUT- |  | IUT- |  |
| PDU session start Time |  |  | I--- |  | I--- |  |
| PDU session stop Time |  |  | --T- |  | --T- |  |
| Diagnostics |  |  | --T- |  | --T- |  |
| Charging Characteristics |  |  | IUT- |  | IUT- |  |
| Charging Characteristics Selection Mode |  |  | IUT- |  | IUT- |  |
| 3GPP PS Data Off Status |  |  | IUT- |  | IUT- |  |
| Session Stop Indicator |  |  | --T- |  | --T- |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  |
| RAN Secondary RAT Usage Report |  |  | -UT- |  | -UT- |  |
| Roaming QBC information |  |  | - |  | IUT- |  |
| Multiple QFI container |  |  | - |  | IUT- |  |
| UPF ID |  |  | - |  | IUT- |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Response message

| Information Element |  | Node Type | SMF (FBC) |  | SMF (QBC) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | I--- |  | I--- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  |
| Invocation Result |  |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  |
| Session Failover |  |  | IU-- |  | IUT- |  |
| Triggers |  |  | IU-- |  | IU-- |  |
| Multiple Unit information |  |  | IU-- |  | - |  |
| Result Code |  |  | IU-- |  | - |  |
| Rating Group |  |  | IU-- |  | - |  |
| UPF ID |  |  | IU-- |  | - |  |
| Granted Unit |  |  | IU-- |  | - |  |
| Validity Time |  |  | IU-- |  | - |  |
| Final Unit Indication |  |  | IU-- |  | - |  |
| Time Quota Threshold |  |  | IU-- |  | - |  |
| Volume Quota Threshold |  |  | IU-- |  | - |  |
| Unit Quota Threshold |  |  | IU-- |  | - |  |
| Quota Holding Time |  |  | IU-- |  | - |  |
| Triggers |  |  | IU-- |  | - |  |
| PDU Session Charging Information |  |  | IU-- |  | IU-- |  |
| Charging Id |  |  | - |  | - |  |
| User Information |  |  | - |  | - |  |
| User Location Info |  |  | - |  | - |  |
| UE Time Zone |  |  | - |  | - |  |
| Presence Reporting Area Information |  |  | IU-- |  | IU-- |  |
| PDU Session Information |  |  | IU-- |  | IU-- |  |
| PDU Session ID |  |  | - |  | - |  |
| Network Slice Instance Identifier |  |  | - |  | - |  |
| PDU Type |  |  | - |  | - |  |
| PDU Address |  |  | - |  | - |  |
| SSC Mode |  |  | - |  | - |  |
| SUPI PLMN ID |  |  | - |  | - |  |
| Serving Network Function ID |  |  | - |  | - |  |
| Serving CN PLMN ID |  |  | - |  | - |  |
| RAT Type |  |  | - |  | - |  |
| Data Network Name Identifier |  |  | - |  | - |  |
| DNN Selection Mode |  |  | - |  | - |  |
| Authorized QoS Information |  |  | - |  | - |  |
| Subscribed QoS Information |  |  | - |  | - |  |
| Authorized Session-AMBR |  |  | - |  | - |  |
| Subscribed Session-AMBR |  |  | - |  | - |  |
| PDU session start Time |  |  | - |  | - |  |
| PDU session stop Time |  |  | - |  | - |  |
| Diagnostics |  |  | - |  | - |  |
| Charging Characteristics |  |  | - |  | - |  |
| Charging Characteristics Selection Mode |  |  | - |  | - |  |
| 3GPP PS Data Off Status |  |  | - |  | - |  |
| Session Stop Indicator |  |  | - |  | - |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  |
| RAN Secondary RAT Usage Report |  |  | - |  | - |  |
| Roaming QBC information |  |  | - |  | IU-- |  |
| Multiple QFI container |  |  | - |  | - |  |
| UPF ID |  |  | - |  | - |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  |

End of changes