3GPP TSG-SA5 Meeting # 128	S5-197690

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | DraftCR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure for 5G-RG scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution is to add work flow for 5G RG access scenario. This contribution is a DraftCR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add add work flow for 5G RG access scenario in TS 32.255. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No work flow for 5G RG access scenario in TS 32.255. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.x	PDU session charging – wireline access

5.2.2.x.1	General

After UE registration to 5GC via wireline access network, a PDU session can be established via this wireline access, as specified in TS 23.316 [xx] for different scenarios:

-	5G-RG connected via NR-RAN and W-5GAN;

-	FN-RG connected via W-5GAN;

-	Service Request procedures via trusted non-3GPP Access specified in clause 4.12a.5 TS 23.502 [202].

5.2.2.x.2	5G-RG Requested PDU Session Establishment via W-5GAN

The following figure 5.2.2.x.2.1 describes charging when 5G-RG requests a PDU session establishment via W-AGF.

- The 5G-RG connects to a W-5GAN and the W-AGF sends an EAP-Request/5G-Start packet over the W-CP connection as described in clause 7.3.1.1 in TS 23.316 [xxx1].

- The steps are specified as step 2-7 in clause 4.3.2.1 of TS 23.502 [202].

3ch-a. The SMF creates a Charging Id for the PDU session, and sends Charging Data Request [Initial] to CHF for authorization for the subscriber to start the PDU session which is triggered by start of PDU session charging event.

3ch-b. The CHF opens CDR for this PDU session.

3ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMF.

- The steps are specified as step 10-11 in clause 4.2.3.1 of TS 23.502 [202].

- The steps are specified as step 2-6 in clause 7.3.1.1 of TS 23.316 [xxx1].

- All steps are specified in clause 4.3.2.1 of TS 23.502 [xxx0]

5.2.2.x.3	5G-RG Requested PDU Session Establishment via NG-RAN

This clause describes PDU session charging when 5G-RG requests a PDU session establishment via NG-RAN. The specification for 5G RG connect 5G Core via NG-RAN can be seen in TS 23.501 [200] and TS 23.502 [202].

The work flow of PDU session charging in case that 5G-RG requests PDU session establishment is specified in clause 5.2.2.2 in this document, except:

- UE is replaced by 5G-RG.

End of change