3GPP TSG-SA5 Meeting # 128	S5-197728

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0053 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify numerous definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSCLICE |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarity of some definitions should be improved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The wording of the definitions is improved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dedfinitions may be not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.1, 4.2, 4.2.1, 4.2.2, 4.2.3.1, 4.2.3.2, 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-197409 |  |  |  |  |  |  |  |  |

First modification

# 4	Service Based Management Architecture

## 4.1	Management Services

The fundamental building block of the Service Based Management Architecture (SBMA) is the Management Service (MnS). A MnS offers a specific set of services related to network management and orchestration. The entity producing the MnS is the MnS producer. The entity consuming the MnS is the MnS consumer. Multiple MnS consumers can use the services offered by a MnS producer.

Figure 4.1.1: MnS producer and MnS consumer

A MnS is described by a set of meta data called MnS profile. A MnS producer shall allow communicating its profile to MnS consumers wishing to use the services of the MnS producer.

## 4.2	MnS components

### 4.2.1	Introduction

A MnS is specified using different independent components. A concrete MnS is composed of at least two of these components. Three different component types are defined, called MnS componenet type A, MnS componenet type B and MnS componenet type C. These components are defined in the following clauses.

### 4.2.2	MnS component type A

The MnS component type A is a group of management operations and/or notifications that is agnostic with regard to the entities managed. The operations and notifications as such are hence not involving any information related to the managed network. These operations and notifications are called generic or network agnostic.

For example, operations for creating, reading, updating and deleting managed object instances, where the managed object instance to be manipulated is specified only in the signature of the operation, are generic.

### 4.2.3	Management information

#### 4.2.3.1	MnS component type B

MnS component type B refers to management information represented by information models representing the managed entities. A MnS component type B is also called Network Resource Model (NRM).

MnS component type B examples are:

1)	Network resource models as defined in TS 28.622 [x]

2)	 Network resource models as defined in TS 28.541 [4]

#### 4.2.3.2	MnS component type C

MnS component type C is performance information of the managed entity and fault information of the managed entity.

The following are examples of Management service component type C:

1. Alarm information as defined in TS 28.532 [9] and TS 28.545 [28].

2. Performance data as defined in TS 28.552 [5], TS 28.554 [6] and TS 32.425 [7].

## 4.3	Combination of MnS components

A MnS is composed by a MnS component type A and

- a MnS component type B, or

- a MnS component type B and a MnS component type C.

The instances of management services carry information about specified management service components in the metadata attributes. Figure 4.3.1.illustrates examples of management service instances with various management service components of type A, type B and type C:

Figure 4.3.1: Example of Management Service and component type A, B and C

End