- 3GPP TSG SA WG5 Meeting #129-e	S5-201143

- February 24 – 28, 2020	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.313 LBO use cases

- Document for:	Approval

Agenda Item:	6.4.12

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.2.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

[2]	3GPP TR 37.816: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on RAN-centric data collection and utilization for LTE and NR".

# 3	Rationale

This contribution proposes the load balancing optimization use case for TS 28.313 [1]. Load balancing among NR cells can be done via the optimization of cell reselection/handover parameters and handover actions, as described in text below that is extracted from TR 37.816 [2].

## 5.4	Load Sharing and Load Balancing Optimisation

### 5.4.1		Use case description

The objective of load sharing and load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cell, or to offload users from one cell or carrier or RAT to achieve network energy saving, This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

# 4	Detailed proposal

1st modified section

# 6	Specification level requirements

## 6.1	Requirements

### 6.1.1	Distributed SON management

#### 6.1.1.x	Load balancing optimization

REQ-DLBO-FUN-1 Producer of provisioning MnS should have a capability allowing an authorized consumer to set or update the LBO policy, including traffic load threshold and target, for NR cell(s).

REQ-DLBO-FUN-2 Producer of provisioning MnS should have a capability allowing an authorized consumer to enable or disable the D-LBO function.

REQ-DLBO-FUN-3 Producer of performance assurance MnS should have a capability allowing an authorized consumer to receive the performance measurements for LBO performance evaluation.

REQ-DLBO-FUN-4 Producer of provisioning MnS should have a capability to notify the authorized consumer with the load balancing action being performed for NR cell(s).

6.1.2	Centralized SON

#### 6.2	Actor roles

## 6.3	Telecommunication resources

## 6.4	Use cases

### 6.4.1	Distributed SON management

#### 6.4.1.x	The management of load balancing optimization

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To manage the distributed load balancing optimization function. |  |
| Actors and Roles | D-SON management function to suppot the load balancing optimization function . |  |
| Telecom resources | - gNB; - The producer of provisioning |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. |  |
| Begins when | The D-SON management function has been deployed and in operation. |  |
| Step 1 (M) | The D-SON management function request producer of provisioning to configure the LBO policy (e.g. traffic load threshold and target) for NR cell(s). |  |
| Step 2 (M) | The D-SON management function request producer of provisioning to enable the D-LBO function. |  |
| Step 3 (M) | The D-LBO function analyses the load information, and determines to perform load balancing among NR cells by changing the cell reselection/handover parameters and handover actions (e.g. offloading UEs to neighboring cells). |  |
| Step 4 (O) | producer of provisioning notifies the D-SON management function about the load balancing action being performed. |  |
| Step 5 (O) | The D-SON management function collects the performance measurements to evaluate the D-LBO performance. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The traffic load has been balanced among neighbouring cells. |  |
| Traceability | REQ-DLBO-FUN-1, REQ-DLBO-FUN-2, REQ-DLBO-FUN-3, REQ-DLBO-FUN-4, |  |

End of modified section