- 3GPP TSG SA WG5 Meeting #129-e	S5-201145

- February 24 – 28, 2020	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.313 LBO information

- Document for:	Approval

Agenda Item:	6.4.12

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.2.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

# 3	Rationale

This contribution proposes the information definition for load balancing optimization for TS 28.313 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services”

[4]	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

[5]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements"

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2"

[8]	3GPP TS 36.300 "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[x]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[y]	3GPP TS 38.331 “NR; Radio Resource Control (RRC) protocol specification”

[z]	3GPP TS 28.531 "Management and orchestration; Provisioning"

Next modified section

# 7	Management services for SON

## 7.1	Management services for D-SON management

### 7.1.x	Management of distributed LBO

#### 7.1.x.1	Component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by MnS Provisioning for NF, as defined in 28.531 [z]. |

#### 7.1.x.2	Component type B definition

##### 7.1.x.2.1	Policy information

The policy of D-LBO function consists of targets and thresholds, as shown in Table 7.1.x.2.1-1.

Table 7.1.x.2.1-1.  LBO targets

| Target parameters | Definition | Legal Values |
| --- | --- | --- |
| Attempted RRC connection establishments successful rate | Successful RRC connection establishments / Attempted RRC connection establishments | [0..100] in unit percentage |
| RRC connection re-establishment successful rate with UE context | RRC connection re-establishment with UE context / Number of RRC connection re-establishment attempts | [0..100] in unit percentage |
| RRC connection re-establishment successful rate without UE context | RRC connection re-establishment without UE context / Number of RRC connection re-establishment attempts | [0..100] in unit percentage |
| RRC connection resuming successful rate | Successful RRC connection resuming / Number of RRC connection resuming attempts | [0..100] in unit percentage |
| Requested handover preparations failure rate | Number of failed handover preparations / Number of requested handover preparations | [0..100] in unit percentage |
| Requested handover resource allocations failure rate | Number of failed handover resource allocations / Number of requested handover resource allocations | [0..100] in unit percentage |
| Requested handover executions faiure rate | Number of failed handover executions / Number of requested handover executions | [0..100] in unit percentage |
| Requested PDU Sessions setup successful rate | Number of PDU Sessions failed to setup / Number of PDU Sessions requested to setup | [0..100] in unit percentage |

##### 7.1.x.2.2	Control information

The parameter is used to control the D-LBO function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO control | This attribute allows authorized consumer to enable/disable the D-LBO functionality. | Boolean On, off |

#### 7.1.x.3	Component type C definition

##### 7.1.x.3.1	Performance measurements

Performance measurements related D-LBO are captured in Table 7.1.x.2.3.-1,

Table 7.1.x.2.3-1.  D-LBO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of UEs handovered due to LBO | Need to measure the number of UEs handovered due to LBO actions |  |

Next modified section

## 7.2	Management services for C-SON

End of modified section