- 3GPP TSG SA WG5 Meeting #129-e	S5-201146

- February 24 – 28, 2020	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.313 changes to PCI configuration

- Document for:	Approval

Agenda Item:	6.4.12

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.2.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

# 3	Rationale

This contribution proposes the changes to PCI configuration SON functions in TS 28.313.

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services”

[4]	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

[5]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements"

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2"

[8]	3GPP TS 36.300 "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[10]	3GPP TS 28.545 "Management and orchestration; Fault Supervision (FS)”

[11]	3GPP TS 28.531 "Management and orchestration; Provisioning"

[12]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[x]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next modified section

# 7	Management services for SON

## 7.1	Management services for D-SON management

### 7.1.3	PCI configuration

#### 7.1.3.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |

#### 7.1.3.2	MnS Component Type B definition

##### 7.1.3.2.1	Control information

The parameter is used to control the D-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the D-SON PCI configuration functionality. See attribute pciConfigurationControl in TS 28.541 [x]. | enable, disable |

7.1.3.2.2	Parameters to be updated

The table below lists the parameter related to the D-SON PCI configuration function.

| Parameters | Definition | Legal Values |
| --- | --- | --- |
| PCI list | The list of PCI values to be used by D-SON PCI configuration function to assign the PCI for NR cells. (See attribute pciList in TS 28.541 [x]). | List of integers |

7.1.3.3	MnS Component Type C definition

##### 7.1.3.3.1	Alarm information

The table below lists the alarms related to D-SON PCI configuration,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. | A notification is sent to report this alarm. |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. | A notification is senti to report this alarm. |

7.2	Management services for C-SON

### 7.2.1	PCI configuration

#### 7.2.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2.1 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.1.2	MnS Component Type B definition

##### 7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. | disable, enable |

7.2.1.2.2	Parameters to be updated

The table below lists the parameter related to the C-SON PCI configuration function.

| NR PCI | This parameter contains the PCI of the NR cell. | Integer |
| --- | --- | --- |

#### 7.2.1.3	MnS Component Type C definition

##### 7.2.1.3.1	Alarm information

The table below lists the alarms related to PCI configuration are generated from the NR cells,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. | A notification is sentto report this alarm. |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. | A notification is sentto report this alarm. |

7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1.  PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI of candidate cells | The measurement contains cumulative counter with subcounters that is identified by the PCI value(s) of the candidate cells, and is derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) where it contains PCI in PhysCellId, and RSRP/RSRQ in MeasQuantityResults of candidate cells. It is generated when the RSRP received from the candidate cells exceeds certain thresholds. |  |

Next modified section

# 8	SON procedures

## 8.3	Centralized SON

### 8.3.1	PCI configuration

#### 8.3.1.1	Initial PCI configuration

Figure 8.3.1.1-1 depicts a procedure that describes how C-SON can assign the PCI values to NR cells the first time.

Figure 8.3.1.1-1: Initial PCI configuration procedure

1. The C-SON determines the PCI value(s) for NR cell(s).

2. The C-SON consumes the MnS of NF provisioning with modifyMOIAttributes operation to configure the PCI value(s) for NR cell(s).

2.a The MnS of provisioning sets the PCI value(s) for NR cell(s) (NOTE)

3. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to C-SON function to indicate the PCI value(s) being assigned to NR cell(s).

NOTE: The interface between MnS of provisioning and PCI configuration (D-SON) function is not subject to standardization.

#### 8.3.1.2	PCI re-configuration

Figure 8.3.1.2-1 depicts a procedure that describes how C-SON function can re-configure the PCI list for NR cell(s) when PCI collision or PCI confusion issues were detected. It is assumed that the C-SON function has consumed the MnS of performance assurance to create PM jobs to collect PCI related measurements.

Figure 8.3.1.2-1: PCI re-configuration procedure

1. The C-SON function collects PCI related performance measurements that are derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) from producer of performance assurance MnS.

2. The C-SON function analyzes the NRM data and PCI related measurements to detect the PCI collision or PCI confusion problems for NR cell(s).

3. The C-SON function determines the new PCI value(s) for NR cell(s).

4. The C-SON function consumes the MnS of NF provisioning with modifyMOIAttributes operation to re-configure the PCI Value(s) for NR cell(s).

4.a The MnS of NF provisioning set the PCI value(s) for NR cell(s).

5. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to C-SON function to indicate the PCI value(s) being assigned to NR cell(s).

End of modified section