- 3GPP TSG SA WG5 Meeting #129-e	S5-201148

- February 24 – 28, 2020	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.313 changes to RACH optimization

- Document for:	Approval

Agenda Item:	6.4.12

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.2.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

# 3	Rationale

This contribution proposes changes to RACH optimization in TS 28.313 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services”

[4]	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

[5]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements"

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2"

[8]	3GPP TS 36.300 "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[10]	3GPP TS 28.545 "Management and orchestration; Fault Supervision (FS)”

[11]	3GPP TS 28.531 "Management and orchestration; Provisioning"

[12]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[x]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next modified section

### 7.1.1	RACH Optimization (Random Access Optimisation)

#### 7.1.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.1.2	MnS Component Type B definition

##### 7.1.1.2.1	Targets information

The targets of RACH optimization are shown in Table 7.1.1.2.1-1.

Table 7.1.1.2.1-1.  RACH optimization targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| UE access delay probability | The probability distribution of UE access delay that is used to minimize the access delays for the UEs under the coverage of popular SSBs. | CDF of access delay |

Editor's Note:	The definition of targets to support the following targets needs clarification from RAN3.

-  Minimize the delays for the UEs to request the other SIs

-  Minimize the imbalance of UEs access delays on uplink (UL) and supplementary uplink (SUL) channel

-  Minimize the beam failure recovery delays for the UEs in RRC_Connected.

-  Minimize the failed/unnecessary RACH attempts on RACH resource before success.

##### 7.1.1.2.2	Control information

The parameter is used to control the RACH optimization function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| RACH optimization control | This attribute allows authorized consumer to enable/disable the RACH optimization functionality. See attribute rachOptimizationControl in TS 28.541 [x]. | Boolean On, off |

##### 7.1.1.2.3	Parameters to be updated

#### 7.2.1.3	MnS Component Type C definition

##### 7.2.1.3.1	Performance measurements

Performance measurements related to the RACH optimization are captured in Table 7.2.1.3.1-1:

Table 7.2.1.3.1-1.  RACH optimization related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Distribution of RACH preambles sent | Distribution of the number of preambles UEs sent to achieve synchronization, where the number of preambles sent corresponds to PREAMBLE_TRANSMISSION_COUNTER (see clause 5.1.1 in TS 38.321 [4]) in UE. | UE access delay probability |
| Distribution of UEs access delay | Distribution of the time needed for UEs to successfully attach to the network. | UE access delay probability |

Next modified section

### 8.2.1	RACH Optimization (Random Access Optimisation)

Figure 8.2.1-1 depicts a procedure that describes how D-SONmanagement function can manage the RACH optimization (D-SON) function. It is assumed that the D-SONmanagement function has consumed the performance assurance management service to collect RACH optimisation related measurements.

Figure 8.2.1-1: RACH Optimization procedure

1. The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation (see clause 5.1.3 in TS 28.532 [3]) to configure the targets for RACH optimization function.

1.a The provisioning MnS sets the targets for RACH optimization (D-SON) function (NOTE).

2. The D-SON management function consumes the management service for NF provisioning with modifyMOIAttributes operation to enable the RACH optimization function for a given NR cell.

3.a The provisioning MnS enables the RACH optimization (D-SON) function (NOTE).

3. The RACH optimization (D-SON) function receives the RACH information report from UE(s), and analyses them to determine the actions to optimize the RACH performance if the performance does not meet the targets by updating the RACH parameters.

4. The D-SON management function collects the RACH related performance measurements.

5. The D-SON management function analyses the measurements to evaluate the RACH performance,

6.  The D-SON management function consumes the provisioning MnS with modifyMOIAttributes operation to update the targets of the RACH optimization function, when the RACH optimization performance does not meet the targets:

6.a The provisioning MnS updates the targets for RACH optimization function (NOTE).

NOTE: The interface between provisioning MnS and RACH optimization is not subject to standardization.

End of modified section