3GPP TSG-SA5 Meeting #129-e	S5-202044

- February 24 – 28, 2020																					  s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0210 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add handover measurements related to MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The handover measurements related to neighbouring cells are needed for MRO to measure the handover performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the handover measurements related to MRO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MRO function is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.x (new), Annex A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 5		Performance measurements for 5G Network Functions

## 5.1.1.x	Measurements related to MRO

##### 5.1.1.x.1	Handover failures related to MRO for intra-system mobility

a)	This measurement provides the number of handover failure events related to MRO detected during the intra-system mobility within 5GS. The measurement includes separate counters for various handover failure types, classified as “Intra-system too early handover”, “Intra-system too late handover” and “Intra-system handover to wrong cell”.

NOTE: The exact clause of reference to TS 38.300 will be added when it is available.

b)	CC.

c)	The measurements of too early handovers, too late handovers and handover to wrong cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS.

d)	Each measurement is an integer value.

e)	HO.IntraSys.TooEarly

- HO.IntraSys.TooLate

- HO.IntraSys.ToWrongCell

f)	NRCellCU.

NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.x.2	Handover failures related to MRO for inter-system mobility

a)	This measurement provides the number of handover failure events delated to MRO detected during the inter-system mobility from 5GS to EPS. The measurement includes separate counters for various handover failure types, classified as “Inter-system too early handover” and “Inter-system too late handover”.

NOTE: The exact clause of reference to TS 38.300 will be added when it is available.

b)	CC.

c)	The measurements of too early handovers and too late handovers events are obtained respectively by accumulating the number of failure events detected by gNB during the inter-system mobility from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.TooEarly

- HO.InterSys.TooLate

f)	NRCellCU.

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.x.3	Unnecessary handovers for Inter-system mobility

a)	This measurement provides the number of unnecessary handover events detected during the inter-system mobility from 5GS to EPS. An example of unnecessary handover occurred when a UE handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient.

NOTE: The exact clause of reference to TS 38.300 will be added when it is available

b)	CC.

c)	The measurement of unnecessary handovers is obtained by accumulating the number of failure events detected gNB during the inter-system mobility from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.Unnecessary

f)	NRCellCU.

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.x.4	Handover ping-pong for inter-system mobility

a)	This measurement provides the number of handover ping-pong events detected during the inter-system mobility from 5GS to EPS. An example of handover ping-pong occurred when a UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE.

NOTE: The exact clause of reference to TS 38.300 will be added when it is available

b)	CC.

c)	The measurement of handover ping-pong events is obtained by accumulating the number of failure events detected by gNB during the inter-system mobility from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.Ping-pong

f)	NRCellCU.

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

Next Modified Section

# Annex A (informative):

# Use cases for performance measurements

# A.x	Monitoring of MRO performance

5G NR cells may experience issues, such as too early or too late handover, handover to wrong cell, ping-pong handover, that not only impact user experience, but also waste network resources, if handover parameters are not set properly. MRO is intended to automatically detect the handover issues, and determine actions to configure the handover parameters in cells in order to improve the handover performance

The MRO related measurements are used to support the mobility roburstness optimization SON function.

End of Modified Sections