3GPP TSG-SA5 Meeting #130e 	S5-202094

- e-meeting 20-28 April 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduce Architecture

- Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

This is a pCR to TS 28.202 to Introduce Architecture

# 2	References

[1]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to Introduce Architecture

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.202 [2]

First change

# 4	Architecture considerations

## 4.1	High level architecture

The high level service based management architecture for Network Slicing management is specified in TS 28.533 [250] clause 4, with Management Services (MnS) produced by MnS Producer and consumed by MnS Consumer.

## 4.2	Network Slice Management converged charging architecture

### 4.2.1	High level architecture network slice charging

The scope of the present document is network slice management charging, the details for other types of network slice charging are defined in TS 32.255 [x] and TS 28.201 [x].

### 4.2.2	Converged charging architecture

Network Slice Management converged charging can be achieved under the two alternative options of the architecture depicted in figure 4.2.2.1.

Figure 4.2.2.1: Network Slice Management converged charging architecture

The enhanced Charging Trigger Function (eCTF) encompasses following functionalities:

-	Charging Trigger Function as specified in TS 32.240 [x].

-	Nchf consumer.

-	MnS consumer.

The Service Producer(s) represent the MnS producer(s) of the management services specified in TS 28.532 [253] which are used for Network Slicing Management.

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components. Ga is described in clause X and Bnsm in clause Y of the present document, and Nchf is described in TS 32.290 [50].

End of change