3GPP TSG-SA5 Meeting #130e 	S5-202097

- e-meeting 20-28 April 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduce NS management charging triggers and flows

- Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

This is a pCR to TS 28.202 to Introduce NS management charging triggers and flows

# 2	References

[1]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to Introduce NS management charging triggers and flows

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.202 [2]

First change

#### 5.2.1.2	Applicable triggers in Network Slice Management charging

A set of trigger conditions are defined for the MnSproducer to invoke a Charging Data Request [Event] towards the CHF.

Table 5.2.1.2-1 summarizes the set of default trigger conditions and their category which shall be supported by the MnSproducer.

Table 5.2.1.2.1: Default Trigger conditions in MnSproducer

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| NSI Allocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Modification Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Deallocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the MnSproducer for these chargeable events, is specified in clause 5.2.3.

Editor’s Note: the case where there is no internal CTF in the MnS producer needs to be considered.

Next change

### 5.2.2	Message flows

#### 5.2.2.1	General

The flows in the present document specify the interaction between the MnSproducer and the CHF for Network Slice Management charging functionality, in different scenarios, based on TS 28.531 [252] clause 7 procedures and flows.

Editor’s Note: flows for the case where there is no internal CTF in the MnS producer need to be considered.

#### 5.2.2.2	NSI Allocation - PEC

Figure 5.2.2.2.1 describes the scenario where a NSI Allocation request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.2.1: NSI allocation - PEC

1.	MnS producer receives an NSI Allocation Request from an MnS consumer.

2.	MnS producer performs NSI Allocation process by creating a new NSI or using an existing NSI.

3.	The MnS producer sends the NSI Allocation Response with appropriate result.

3ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Allocation transaction.

3ch-b. The CHF creates a CDR for this NSI Allocation transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

#### 5.2.2.3	NSI Modification - PEC

Figure 5.2.2.3.1 describes the scenario where a NSI Modification request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.3.1: NSI Modification - PEC

1.	MnS producer receives an NSI Modification Request from an MnS consumer.

2.	MnS producer performs NSI Modification process.

3.	The MnS producer sends the NSI Modification Response with appropriate result.

3ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Modification transaction.

3ch-b. The CHF creates a CDR for this NSI Modification transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

#### 5.2.2.4	NSI Deallocation - PEC

Figure 5.2.2.4.1 describes the scenario where a NSI Deallocation request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.4.1: NSI Deallocation - PEC

1.	MnS producer receives an NSI Deallocation Request from an MnS consumer.

2.	The MnS producer sends the NSI Deallocation Response.

2ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Deallocation transaction.

2ch-b. The CHF creates a CDR for this NSI Deallocation transaction.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

3.	MnS producer performs the NSI Deallocation process.

End of change