3GPP TSG-SA5 Meeting #130e 	S5-202202

- e-meeting 20-28 April 2020

- Source:	Huawei

- Title:	update of use case in subclause 6.1.1

- Document for:	Approval

Agenda Item:	6.4.7

# 1	Decision/action requested

Propose to updatew use case and requirement in subclause 6.1.1.

# 2	References

[1]	3GPP TS 23.535 Management Services for Communication Service Assurance; Requirements

# 3	Rationale

As agreed in S5-201488, for communication service assurance, the CSP adjusts the CSI or the CSC adjusts the SLS continuously to achieve the assured requirements. The possible SLS adjustment then lead to also a CSI adjustment performed by CSP. Such scenario has not been described in any use case, therefore this contribution try to provide a complete view by updating the use case and the requirement.

# 4	Detailed proposal

First Change

### 6.1.1	Communication service quality assurance and optimization of communication services

The goal of the use case is to enable communication service quality assurance and optimization for the set of services provided by the network to certain group (category) of UEs. For example, the set can include the communication services provided via certain NSI(s) or to IoT devices in certain area.

It is assumed that the relevant NFs are deployed and active in NG-RAN and 5GC. The group of NG-RAN and 5GC nodes, which are essential for the set of E2E services, provide provisioning and PM management services. It is also assumed that the providers of the related NSI / NSSI provisioning and PM management services are deployed and active.

The management system is consuming above management services either directly or through proxy nodes that re-expose the management services; the management system is aware of the performance requirements imposed on the set of communication services.

The management system is collecting the service experience information and monitoring the key performance indicators, KPIs, related to the targeted services. Analytics hosted by the MDAF may be utilized for processing of the network data to derive and analyse the KPIs. When the service quality assurance and optimization function detects performance degradation, the 3GPP management system may continuously modifies the configuration parameters in the corresponding NG-RAN and 5GC nodes and NSI(s)/NSSI(s) to satisfy the SLA requirement. CSC may coordinate with CSP to adjust the SLA requirements if needed.

If the network performance does not recover or improve, the management system may adjust the modifications, for example roll back to previous configuration. In any case it continues collecting the network data and monitoring of the performance indicators.

## 6.2	Requirements

REQ-CSM_SAO-CON-01 The 3GPP management system shall have the capability to take actions based on the target communication service KPIs for a set of communication services supplied to a group or category of UEs.

REQ-CSM_SAO-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSM_SAO-CON-03The 3GPP management system shall have the capability to analyse the KPIs related to the set of communication services delivered to a group or category of UEs.

REQ-CSM_SAO-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services supplied to a group or category of UEs

REQ- CSM_SAO-CON-X The 3GPP management system shall have the capability to configure resources based on agreed SLA/SLS which may be modified if needed.

End of Change