3GPP TSG-SA5 Meeting #130-e	S5-202178

# Online, , 20th Apr 2020 - 28th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0215 | rev | 0 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure for 5G-RG handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution is to add procedure for 5G-RG handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure on 5G-RG handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G-RG handover is not covered by 5WWC charging scenarios in TS 32.255. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.16.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.16.x	Handover procedure

5.2.2.16.x.1		5G RG handover of a PDU Session procedure from W-5GAN access to 3GPP access

Following figure 5.2.2.16.x.1.1 describes charging when 5G-RG handover of a PDU Session procedure from W-5GAN access to 3GPP access.

Figure 5.2.2.16.x.1.1: Handover of PDU session from W-5GAN access to 3GPP access

As described in clause 7.6.3.1 in TS 23.316 [203], the handover of a PDU session from 3GPP access to W-5GAN access is as following.

2CH-a-c. SMF may interact with CHF with Charging Data Request [Update]. The access network change trigger, if enabled, applied during the PDU session establishment via the 3GPP access network which was performed prior to release of W-5GAN access resource.

5.2.2.16.x.2		5G RG handover of a PDU Session procedure from 3GPP to W-5GAN access

Following figure 5.2.2.16.x.1.1 describes charging when 5G-RG handover of a PDU Session procedure from 3GPP to W-5GAN access.

Figure 52.2.16.x.2.1: Handover of a PDU Session procedure from 3GPP to W-5GAN access

As described in clause 7.6.3.2 in TS 23.316 [203], the handover of a PDU session from 3GPP access to W-5GAN access is as following.

2ch-a-c. SMF may interact with CHF with Charging Data Request [Update]. The access network change trigger, if enabled, applied during the PDU session establishment via the W-5GAN which was performed prior to this release of 3GPP access resource.

End of change