3GPP TSG-SA5 Meeting #130e 	S5-202209

# e-meeting 20-28 April 2020

# Source:	Huawei

# Title:	pCR TS 28.313 Management service description

# Document for:	Approval

Agenda Item:	6.4.9

1	Decision/action requested

The group is asked to discuss and approve the proposals.

2	References

3	Rationale

This contribution proposes to update clause 7.1 and 7.2 by merging the description of typeA and TypeB into a single table so as to align with other management services, e.g. PM control management service.

4	Detailed proposal

1st modified section

7	Management services for SON

Editor’s note: this clause contains the management services for SON and definition of information, such as targets, control, parameters, and PM measurements that are needed to support SON functions.

7.1	Management services for D-SON management

7.1.1	RACH Optimization (Random Access Optimisation)

7.1.1.1	MnS component type A and type B definition

Table 7.1.1.1-1: Components of RACH Optimization MnS

| Management service | Management service component type A | Management service component type B |
| --- | --- | --- |
| RACH Optimization MnS | Generic provisioning MnS CRUD operations/notification defined in TS 28.532[3]. | IOCs for RACH Optimization NRM fragment defined in TS 28.541[13]. |

7.1.1.2	MnS Component Type C definition

7.1.1.2.1	Performance measurements

Performance measurements related to the RACH optimization are captured in Table 7.1.1.3.1-1:

Table 7.1.1.2.1-1.  RACH optimization related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Distribution of RACH preambles sent | Distribution of the number of preambles UEs sent to achieve synchronization, where the number of preambles sent corresponds to PREAMBLE_TRANSMISSION_COUNTER (see clause 5.1.1 in TS 38.321 [4]) in UE. | UE access delay probability |
| Distribution of UEs access delay | Distribution of the time needed for UEs to successfully attach to the network. | UE access delay probability |

7.1.2	MRO (Mobility Robustness Optimisation)

7.1.2.1	MnS component type A and type B definition

Table 7.1.2.1-1: Components of MRO MnS

| Management service | Management service component type A | Management service component type B |
| --- | --- | --- |
| Mobility Robustness Optimization MnS | Generic provisioning MnS CRUD operations/notification defined in TS 28.532 [3]. | IOCs for Mobility Robustness Optimization NRM fragment defined in TS 28.541 [13]. |

7.1.2.2	MnS Component Type C definition

7.1.2.2.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.2.1-1.  MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover eventsl | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes. | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes. | Total inter-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell. | Imbalance of UE access delay on UL and SUL probability |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time. | Total intra-RAT handover failure rate |
| Number of inter-RAT too late handover failures | Deteccted when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC. | Total inter-RAT handover failure rate |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |

7.1.3	PCI configuration

7.1.3.1	MnS component type A and type B definition

Table 7.1.3.1-1: Components of PCI configuration MnS

| Management service | Management service component type A | Management service component type B |
| --- | --- | --- |
| PCI configuration MnS | Generic provisioning MnS CRUD operations/notification defined in TS 28.532 [3]. | IOCs for PCI configuration NRM fragment defined in TS 28.541 [13]. |

7.1.3.3	MnS Component Type C definition

7.1.3.3.1	Alarm information

The table below lists the alarms related to D-SON PCI configuration,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add new alarms or modify existing alarms in TS 28.545 or TS 32.111-2) to support PCI collision and confusion.

7.1.4	ANR management

Stage 2 for ANR management is located in TS 28.541 [13], clauses 4.3.2.2, 4.3.2.3, 4.3.32.2 and 4.3.32.3.

Stage 3 for ANR management is located in TS 28.541 [13], clauses C.4.3, D.4.3, and E.5.

7.1.4.1	MnS component type A and type B definition

Table 7.1.4.1-1: Components of ANR management MnS

| Management service | Management service component type A | Management service component type B |
| --- | --- | --- |
| ANR management MnS | Generic provisioning MnS CRUD operations/notification defined in TS 28.532 [3]. | IOCs for ANR management NRM fragment defined in TS 28.541 [13]. |

7.2	Management services for C-SON

7.2.1	PCI configuration

7.2.1.1	MnS component type A and type B definition

Table 7.2.1.1-1: Components of PCI configuration MnS

| Management service | Management service component type A | Management service component type B |
| --- | --- | --- |
| PCI configuration MnS | Generic provisioning MnS CRUD operations/notification defined in TS 28.532 [3]. | IOCs for PCI configuration NRM fragment defined in TS 28.541 [13]. |

7.2.1.3	MnS Component Type C definition

7.2.1.3.1	Alarm information

The table below lists the alarms related to PCI configuration are generated from the NR cells,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI list.

7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1.  PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI of candidate cells | The measurement contains cumulative counter with subcounters that is identified by the PCI value(s) of the candidate cells, and is derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) where it contains PCI in PhysCellId, and RSRP/RSRQ in MeasQuantityResults of candidate cells. It is generated when the RSRP received from the candidate cells exceeds certain thresholds. |  |

Editor’s Note: Performance measurement needs to be defined in the TS 28.552 [5].

End of modified section