3GPP TSG SA WG5 (Telecom Management) Meeting #130e 	S5-202210

# 20-28 April 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0222 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify PRB usage measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020/4/10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRB resource allocation is managed by BWP in NR. When NR UE is connecting to NR Cell using initial BWP (only 48 PRBs), if the PRB usage is too high because of a lot of UEs accessing in the NR cell, the delay of UE accessing to the network will be increased. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Initial BWP PRB usage measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initial BWP PRB overload can’t be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.X(new), 5.1.1.2.Y(new), 5.1.1.2.Z(new), 5.1.1.2.A(new), A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.2.X	UL PRB used for Initial BWP

a)	This measurement provides the number of physical resource blocks (PRBs) in total used in uplink initial BWP (see TS 38.321[x]).

b)	CC.

c)	Each measurement is obtained as the all PRBs used for UL data traffic transmission used in uplink initial BWP.

d)	Each measurement is a single integer value.

e)	RRU.PrbUsedUlInitialBWP.

f)	BWP

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the UL PRB  load of the radio physical layer initial BWP.

5.1.1.2.Y	UL total available Initial BWP PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in available uplink of initial BWP (see TS 38.321[x]).

b)	CC.

c)	The measurement is obtained total available  count of PRBs available for uplink initial BWP.

d)	Each measurement is a single integer value.

e)	RRU.PrbAvailUlInitialBWP.

f)	BWP.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in  uplink of initial BWP .

5.1.1.2.Z	DL PRB used for Initial BWP

a)	This measurement provides the number of physical resource blocks (PRBs) in total used in downlink initial BWP (see TS 38.321[x]).

b)	CC.

c)	Each measurement is obtained as the all PRBs used for DL data traffic transmission used in downlink initial BWP.

d)	Each measurement is a single integer value.

e)	RRU.PrbUsedDlInitialBWP.

f)	BWP

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the DL PRB  load of the radio physical layer initial BWP.

5.1.1.2.A	DL total available Initial BWP PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in available downlink of initial BWP (see TS 38.321[x]).

b)	CC.

c)	The measurement is obtained total available   PRBs available for downlink initial BWP.

d)	Each measurement is a single integer value.

e)	RRU.PrbAvailDlInitialBWP.

f)	BWP.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in   downlink of initial BWP.

Next modified section

A.6	Monitoring of physical radio resource utilization

The physical radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. The physical radio resource utilization measurements should reflect the average usage and the usage distribution of the radio resource of the physical layer. The measurements can make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing.

When NR UE is connecting to NR Cell using initial BWP (only 48 PRBs), if the PRB usage is too high because of a lot of UEs access in the NR cell, the delay of UE accessing to the network will be increased. Monitoring the physical radio resource utilization of initial BWP is helpful for operators to be aware of the load.

Network slicing is an important feature in 5GS, monitoring physical radio resource utilization per S-NSSAI is helpful for opeators to be aware of the load.

End of modifications