3GPP TSG-SA5 Meeting #131e 	S5-203207

# e-meeting 25th May-3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0114 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Trace Record Schema |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oy LM Ericsson AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_RTT |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Trace Record definition is not correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct definition and Update schema to align with attribute description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Schema in current version is not aligned with description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.2	Definition of the Trace Record Header

The trace record header contains the fields defined in Table 5.2.2.1

Table 5.2.2.1: Fields in the trace record header

| Trace Record Header field name | Description |
| --- | --- |
| timeStamp (M) | Time stamp (milliseconds since Epoch) of when the trace record is produced internally in the Producer (64 bit integer) |
| nfInstanceId (M) | Unique id of the Producer NF instance (string) |
| nfType (M) | Type of the Producer NF (string) |
| traceReference (M) | Trace Reference (3 byte octet string) |
| traceRecordingSessionReference (M) | Trace Recording Session Reference (2 byte octet string) |
| traceRecordTypeId (M) | Identifier of the trace record type (64 bit integer) |
| ranUeId (O) | RAN defined Id of the UE (8 byte octet string) |
| payloadSchemaURI (O) | URI identifying the schema to decode the payload (string) |
| vendorExtension (O) | Vendor-specific extension (associative map, string-string) |

The traceReference value is as defined in TS 32.422 [23] clause 5.6.

The traceRecordingSessionReference is as defined in TS 32.422 [23] clause 5.7.

The ranUeId field is mandatory in UE-related trace records.  At present ranUeId is only defined for F1 (TS 38.473 [26]) and E1 (TS 38.463 [25]).

The vendorExtension is defined as a generic name-value associative map.

Next change

Annex G (normative):
Trace Record Protocol Buffer (GPB) definitions

Normative GPB Trace Record schema, defined per clause 5.2.2 in this document:

syntax = “proto3”;

/* Trace Record per 3GPP 32.423 specification.

* v16

*/

message TraceRecordHeader {

required int64  time_stamp = 1;

required string nf_instance_id = 2;

required string nf_type = 3;

required bytes trace_reference = 4;

required int64  trace_rec_type_id = 5;

required  bytes trace_recording_session_ref = 6;

optional int64  ran_ue_id = 7;

optional string payload_schema_uri = 8;

optional map<string, string> vendor_extension = 9;

}

message TraceRecordPayload {

optional int64 size = 1;

required bytes payload = 2;

}

message TraceRecord {

required TraceRecordHeader header = 1;

required TraceRecordPayload payload = 2;

}

End of changes