3GPP TSG-SA5 Meeting #132e 	S5-204042

- e-meeting 17th - 28th August 2020

- Source:	China Telecommunication Corp

- Title:	pCR TS 28.201 Add Isolation level and Area of service as the NS Performance and analytics charging attribute

- Document for:	Approval

Agenda Item:	7.4.1

# 1	Discussion

This is a pCR to TS 28.201 adding the NS performance and analytics charging information.

# 2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

[2]	GSMA NG. 116: "Generic Network Slice Template".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 32.845: " Study on Charging Aspects of Network Slicing".

# 3	Rationale

Isolation Level and Area of service are two of GST attributes which are defined in GSMA NG.116.  The ResourceSharingLevel and CoverageArea as the Attribute of ServiceProfile and SliceProfile are defined in TS 28.541, The Isolation Level and ResourceSharingLevel share the same intention; Area of service and CoverageArea share the same intention, which are important for Network Slice business.

This pCR proposes to introduce the NS performance and analytics charging information in TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

The following change is proposed:

- Add two new charging attribute to Network slice performance and analytics charging information

- Add two new charging attribute to  Structure of NSPA Charging Information

# 4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

### 5.1.3	Network slice performance and analytics charging information

The following is a non-exhaustive list related to one or more of the performance and analytics inputs relevant to charging for network slice:

-	Latency

-	Throughput

-	Maximum utilized bandwidth

-	Maximum packet loss rate as defined in GSMA NG.116 [500]

-	Service Experience statistics information as defined in clause 6.4, TS 23.288 [150]

-	The number of PDU sessions

-	Registered Subscribers

-	Load level information as defined in clause 6.3, TS 23.288 [150].

-     Isolation level as defined in GSMA NG.116 [500]

-    Area of service as defined in GSMA NG.116 [500]

Next changes

#### 6.2.1.2	Definition of Performance and Analytics Charging Information

Specific charging information used for network slice performance and analytics charging is provided within the Performance and Analytics Charging Information.

The detailed structure of the Performance and Analytics Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2-1: Structure of NSPA Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Trigger | OC | This field holds the reason for reporting the performance and analytics information to CHF. |
| Network Slice Instance Assistance Identifier | M | This field holds network slice information the performance and analytics information belongs to. |
| Performance and analytics information | OC | This field is the container of performance and analytics information It may have multiple occurrences. |
| Latency | OC | This field holds Latency. |
| Throughput | OC | This field holds Throughput |
| Maximum utilized bandwidth | OC | This field holds Maximum utilized bandwidth |
| Maximum packet loss rate | OC | This field holds Maximum packet loss rate |
| Service Experience statistics data | OC | This field holds service Experience statistics data |
| The number of PDU sessions | OC | This field holds the number of PDU sessions |
| Registered Subscribers | OC | This field holds registered subscribers |
| Load level | OC | This field holds the Load level. |
| Isolation level | OC | This field holds the resource sharing level for RAN、TN、CN |
| Area of service | OC | This field holds the coverage area of E2E network slice service |

Editor’s note: the content of table 6.2.1.2.1 is ffs.

Editor's Note: the table of Network Slice Charging Information is ffs.

End of changes