3GPP TSG-SA5 Meeting #132e 	S5-204058rev2

- e-meeting 17th 28th August 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Update the Charging Architecture

- Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

This is a pCR to TS 28.201 to Update Charging Architecture

# 2	References

[1]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[3]	S5-204081 DP on Common Charging architecture

# 3	Rationale

This pCR proposes to Update Charging Architecture based on conclusion from DP S5-204081 [3]

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.201 [1]

First change

### 4.2.2	Converged charging architecture

The architectural options for network slice performance and analytics converged charging are depicted in figure 4.2.2-1.

Figure 4.2.2-1: Network slice performance and analytics converged charging architecture

The Charging Enablement Function (CEF):

-	subscribes to notifications by consuming management services or services exposed by other functions, for receiving required charging information from the services.

-	determines the occurrence of chargeable events. When it determines that a chargeable event has occurred it then triggers charging by consuming the Nchf services.

Charging information addressed by the CEF in the present document, are related to performance and analytics for network slices.

The MnS producer is defined in TS 28.533 [250]: the MnS producer in the present document is the producer of performance MnS.

For network slice performance and analytics converged charging the CEF is a consumer of either one or both:

-	performance management service (MnS) for network slice exposed by the MnS Producer, specified in TS 28.532 [253]:

-	Network Data Analytics service (Nnwdaf) for network slice exposed by the Network Data Analytics Function (NWDAF), described in TS 23.288 [150];

And the CEF is a consumer of:

-	charging (Nchf) service.

The general architecture components can be found in TS 32.240 [1].

Ga is described in clause 5.2.4 and Bns in clause 5.2.5 of the present document, and Nchf is described in TS 32.290 [50].

End of change