- 3GPP TSG SA WG5 Meeting #132-e	S5-204107

- August 17 – 28, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.814 use cases and requirements for EAS deployment

- Document for:	Approval

Agenda Item:	6.6.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

This contribution proposes to add the use cases and requirements for EAS deployment to TR 28.814 [1]. The deployment of EAS is defined in Figure 1 in TS 23.558 [2].

Figure 1: Architecture for enabling edge applications

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558: "Architecture for enabling Edge Applications;".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 5	Use cases and potential requirements

## 5.1	Business level use cases

## 5.1.x	The deployment of edge application server(s)

#### 5.1.x.1	The deployment of EAS on EDN by PLMN/EC service provider

##### 5.1.x.1.1	Goal

The goal is to enable the ASP to install or terminate one or more EAS on the EDN (see clause 6.5.2.5 TS 23.558 [x]). It is assumed that the both PLMN networks and EDN are owned by the same organization, called PLMN/EC service provider (see annex B in TS 23.558 [x]).

##### 5.1.x.1.2	Description

###### 5.1.x.1.2.1	EAS installation

1.	The ASP requests the PLMN/EC service provider to install one or more EAS with deployment requirements.

2.	The PLMN/EC service provider selects the EDN to host the EAS, based on the deployment requirements.

3.	The PLMN/EC service provider installs the EAS at the EDN that meets the deployment requirements.

4.	The PLMN/EC service provider notifies the ASP that the EAS has been installed successfully.

###### 5.1.x.1.2.2	EAS termination

1.	The ASP requests the PLMN/EC service provider to terminate one or more EAS(s) with the EAS identifier(s).

2.	The PLMN/EC service provider terminates the EAS(s) based on the identifier at the EDN.

3.	The PLMN/EC service provider notifies the ASP that the EAS(s) has been terminated successfully.

#### 5.1.x.2	The deployment of EAS on EDN by ECSP

##### 5.1.x.2.1	Goal

The goal is to enable the ASP to install or terminate one or more EAS on EDN in the ECSP.

##### 5.1.x.2.2	Description

###### 5.1.x.2.2.1	EAS installation

1.	The ASP requests the ECSP to install one or more EAS(s) with deployment requirements.

2.	The ECSP selects the EDN to host the EAS(s), based on the deployment requirements.

3.	The ECSP installs EAS(s) at the selected EDN.

4. 	The ECSP notifies the ASP that the EAS has been install successfully.

###### 5.1.x.2.2.2	EAS termination

1.	The ASP requests the ECSP to terminate one or more EAS(s) with EAS identifier(s).

2.	The ECSP terminates the EAS(s) based on the identifier at the EDN.

3. 	The ECSP notifies the ASP that the EAS(s) has been terminated successfully.

## 5.2	Specification level use cases

Editor’s note: this clause is to describe the use cases associated with the management services to support orchestration and management of edge computing.

### 5.2.x	The deployment of edge application server(s)

#### 5.2.x.1	The deployment of EAS on EDN by PLMN/EC service provider

##### 5.2.x.1.1	Goal

The goal is to enable the ASP to install or terminate the EAS(s) on the EDN.

##### 5.2.x.1.2	Description

###### 5.2.x.1.2.1	EAS installation

1.	A consumer consumes the management service of PLMN/EC management system to install the EAS(s) with application packages (e.g. VNF, VNFD, ,,, etc) and deployment requirements (e.g. virtualized resources requirements (i.e. compute, memory, …), QoS requirements (i.e. latency, bandwidth), and geographical service areas, … etc)).

2.	The PLMN/EC management system determines in which EDN the EAS(s) will be deployed, based on the deployment requirements. For example, the selection of EDN in which the EAS(s) will be installed may be determined by the QoS and geographical service areas requirements.

3.	The PLMN/EC management system consumes the management service of ETSI NFV MANO to perform the following operations:

- Instantiate the EAS VNF(s).

- Connect the EAS(s) to the EES.

- Connect the EAS(s) to the 5GC network functions for sending the information over N6 and Edge 7-interfaces

4.	The PLMN/EC management system notifies the consumer that the EAS(s) has been installed successfully.

###### 5.2.x.1.2.2	EAS termination

1.	A consumer consumes the management service of PLMN/EC management system to terminate the EAS with the EAS identifier(s).

2.	The PLMN/EC management system terminates the EAS VNF(s) based on the EAS identifier(s), and disconnects the EAS from the 5GC network functions.

3.	The PLMN/EC management system notifies the consumer that the EAS(s) has been terminated successfully.

##### 5.2.x.1.3	Requirements

REQ-EAS-DEPLOY-FUN-a PLMN/EC management system should provide management services to allow consumers to request the installation of EAS with application packages (e.g. VNF, VNFD, ,,, etc) and deployment requirements (e.g. virtualized resources requirements (i.e. compute, memory, …), QoS requirements (i.e. latency, bandwidth), and geographical service areas, … etc)).

REQ-EAS-DEPLOY-FUN-b PLMN/EC management system should provide management services to allow consumers to request the termination of EAS with the EAS identifier.

REQ-EAS-DEPLOY-FUN-c PLMN/EC management system should be able to instantiate the EAS VNF at the EDN, connect the EAS to the EES, and connect the EAS to the 5GC network functions for sending the information over N6 and edge-7 interfaces.

REQ-EAS-DEPLOY-FUN-d PLMN/EC management system should be able to terminate the EAS VNF based on the EAS identifier, and disconnect the EAS from the 5GC network functions.

REQ-EAS-DEPLOY-FUN-e PLMN/EC management system should be able to send the notification indicating the status of EAS installation and termination.

#### 5.2.x.2	The deployment of EAS on EDN by ECSP

##### 5.2.x.2.1	Goal

The goal is to enable the ASP to install or terminate the EAS(s) on EDN in the ECSP.

##### 5.2.x.2.2	Description

###### 5.2.x.2.2.1	EAS installation

1.	A consumer consumes the management service of ECSP management system to install the EAS with application packages (e.g. VNF, VNFD, ,,, etc) and deployment requirements (e.g. virtualized resources requirements (i.e. compute, memory, …), QoS requirements (i.e. latency, bandwidth), and geographical service areas, … etc)).

2.	The ECSP management system determines in which EDN the EAS(s) will be deployed, based on the deployment requirements. For example, the selection of EDN in which the EAS(s) will be installed may be determined by the QoS and geographical service areas requirements.

3.	The ECSP management system consumes the management service of ETSI NFV MANO to perform the following operations:

- Instantiate the EAS VNF(s).

- Connect the EAS(s) to the EES.

- Connect the EAS(s) to the 5GC network functions for sending the information over N6 and Edge 7-interfaces

4.	The ECSP management system notifies the consumer that the EAS has been installed successfully.

###### 5.2.x.2.2.2	EAS installation

1.	A consumer consumes the management service of ECSP management system to terminate the EAS(s) with the EAS identifier.

2.	The ECSP management system terminates the EAS VNF(s) based on the EAS identifier, and disconnects the EAS from the 5GC network functions.

3.	The ECSP management system notifies the consumer that the EAS(s) has been terminated successfully.

##### 5.2.x.2.3	Requirements

REQ-EAS-DEPLOY-FUN-f ECSP management system should provide management services to allow consumers to request the installation of EAS with application packages (e.g. VNF, VNFD, ,,, etc) and deployment requirements (e.g. virtualized resources requirements (i.e. compute, memory, …), QoS requirements (i.e. latency, bandwidth), and geographical service areas, … etc)).

REQ-EAS-DEPLOY-FUN-g ECSP management system should provide management services to allow consumers to request the termination of EAS with the EAS identifier.

REQ-EAS-DEPLOY-FUN-h ECSP management system should be able to instantiate the EAS VNF at the EDN, connect the EAS to the EES, and connect the EAS to the 5GC network functions for sending the information over N6 and Edge-7 interfaces.

REQ-EAS-DEPLOY-FUN-i ECSP management system should be able to terminate the EAS VNF based on the EAS identifier, and disconnect the EAS from the 5GC network functions.

REQ-EAS-DEPLOY-FUN-j ECSP management system should be able to send the notification indicating the status of EAS installation and termination.

End of modified section