- 3GPP TSG SA WG5 Meeting #132-e	S5-204109

- August 17 – 28, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.814 use cases and requirements for ECS deployment

- Document for:	Approval

Agenda Item:	6.6.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

This contribution proposes to add the use cases and requirements for ECS deployment to TR 28.814 [1]. The deployment of ECS is defined in Figure 1 in TS 23.558 [2].

Figure 1: Architecture for enabling edge applications

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558: "Architecture for enabling Edge Applications;".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 5	Use cases

## 5.1	Business level use cases

## 5.1.x	The deployment of edge configuration server(s)

#### 5.1.x.1	The deployment of ECS by PLMN operator

##### 5.1.x.1.1	Goal

The goal is to enable the installation and termination of one or more ECS in PLMN operator (see clause 6.5.2.3 in TS 23.558 [2]).

##### 5.1.x.1.2	Description

###### 5.1.x.1.2.1	ECS installation

1.	The PLMN operator receives a request to install one or more ECS.

2.	The PLMN operator installs the ECS.

3.	The PLMN operator sends a notification to indicate that the ECS has been installed successfully.

###### 5.1.x.1.2.2	ECS termination

1.	The PLMN operator receives a request to termination one or more ECS with the ECS identifier.

2.	The PLMN operator terminates the ECS based on the ECS identifier.

3.	The PLMN operator sends a notification to indicate that the ECS has been terminated successfully.

#### 5.1.x.2	The deployment of ECS by ECSP

##### 5.1.x.2.1	Goal

The goal is to enable the installation and termination of one or more ECS in ECSP (see clause 6.5.2.3 in TS 23.558 [2]).

##### 5.1.x.2.2	Description

###### 5.1.x.2.2.1	ECS installation

1.	The ECSP receives a request to install one or more ECS(s).

2.	The ECSP selects the EDN to the ECS.

3.	The ECSP installs the ECS.

3.	The ECSP sends a notification to indicate that the ECS has been installed successfully.

###### 5.1.x.2.2.2	ECS termination

1.	The ECSP receives a request to terminate one or more ECS(s) with the ECS identifiers(s).

2.	The ECSP terminate the ECS(s) based on the ECS identifiers(s).

3.	The ECSP sends a notification to indicate that the ECS has been terminated successfully

## 5.2	Specification level use cases

Editor’s note: this clause is to describe the use cases associated with the management services to support orchestration and management of edge computing.

### 5.2.x	The deployment of edge configuration server(s)

#### 5.2.x.1	The deployment of ECS by PLMN operator

##### 5.2.x.1.1	Goal

The goal is to enable the installation and termination of one or more ECS in PLMN operator (see clasue 6.5.2.3 in TS 23.558 [2]).

##### 5.2.x.1.2	Description

###### 5.1.x.1.2.1	ECS installation

1.	A consumer consumes the management service of PLMN management system to install the ECS with application packages (e.g. VNF, VNF package, ,,, etc).

2.	The PLMN management system performs the following operations to install the ECS:

-	Instantiate the ECS VNF.

-	Connect the ECS to the 5GC network functions for sending the information over the edge-8 interfaces.

3.	The PLMN management system sends a notification to indicate that the ECS has been installed successfully.

###### 5.1.x.1.2.2	ECS termination

1.	A consumer consumes the management service of PLMN management system to terminate the ECS with the ECS identifier.

2.	The PLMN management system terminates the ECS VNF based on the ECS identifier, and disconnects the ECS from the 5GC network functions.

3.	The PLMN management system sends a notification to indicate that the ECS has been terminated successfully.

##### 5.2.x.1.3	Requirements

REQ-ECS-DEPLOY-FUN-a The PLMN management system should provide management services to allow consumers to request the installation of ECS with application packages (e.g. VNF, VNF package, ,,, etc).

REQ-ECS-DEPLOY-FUN-b The PLMN management system should provide management services to allow consumers to request the termination of ECS with the ECS identifier.

REQ-ECS-DEPLOY-FUN-c The PLMN management system should be able to instantiate the ECS VNF at the EDN, and connect the ECS to the 5GC network functions for sending the information over edge-8 interfaces.

REQ-ECS-DEPLOY-FUN-d The PLMN management system should be able to terminate the ECS VNF baed on the ECS identifier, and disconnect the ECS from the 5GC network functions.

REQ-ECS-DEPLOY-FUN-e The PLMN management system should be able to send the notification indicating the status of EAS installation and termination.

#### 5.2.x.2	The deployment of ECS by ECSP

##### 5.2.x.2.1	Goal

The goal is to enable the installation and termination of one or more ECS in ECSP (see clasue 6.5.2.3 in TS 23.558 [2]).

##### 5.2.x.2.2	Description

###### 5.1.x.2.2.1	ECS installation

1.	A consumer consumes the management service of ECSP management system to install the ECS with application packages (e.g. VNF, VNF package, ,,, etc).

2.	The ECSP management system selects the EDN to install ECS.

3. The ECSP management system performs the following operations to install the EES:

-	Instantiate the ECS VNF.

-	Connect the ECS to the 5GC network functions for sending the information over the Edge-8 interfaces.

4. 	The ECSP management system sends a notification to indicate that the ECS has been deployed successfully.

###### 5.1.x.2.2.2	ECS termination

1.	A consumer consumes the management service of ECSP management system to terminate the ECS with the ECS identifier.

2.	The ECSP management system terminates the ECS VNF based on the ECS identifier, and disconnect the ECS from the 5GC network functions.

3. 	The ECSP management system sends a notification to indicate that the ECS has been deployed successfully.

##### 5.2.x.2.3	Requirements

REQ-ECS-DEPLOY-FUN-f The ECSP management system should provide management services to allow consumers to request the installation of ECS application packages (e.g. VNF, VNF package, ,,, etc).

REQ-ECS-DEPLOY-FUN-g The ECSP management system should provide management services to allow consumers to request the termination of ECS with the ECS identifier.

REQ-ECS-DEPLOY-FUN-h The ECSP management system should be able to instantiate the ECS VNF at the EDN, and connect the ECS to the 5GC network functions for sending the information over edge-8 interfaces.

REQ-ECS-DEPLOY-FUN-i The PLMN management system should be able to terminate the ECS VNF baed on the ECS identifier, and disconnect the ECS from the 5GC network functions.

REQ-ECS-DEPLOY-FUN-j The ECSP management system should be able to send the notification indicating the status of EAS installation and termination.

End of modified section