- 3GPP TSG SA WG5 Meeting #132-e	S5-204112

- August 17 – 28, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.313 add references for type 2 parameters

- Document for:	Approval

Agenda Item:	6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.4.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

# 3	Rationale

This contribution proposes to add references to NRM for type 2 parameters.

# 4	Detailed proposal

1st modified section

## 7.1	Management services for D-SON management

### 7.1.1	RACH Optimization (Random Access Optimisation)

#### 7.1.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.2.1	MnS Component Type B definition

##### 7.1.2.1.1	Targets information

The targets of RACH optimization are shown in Table 7.1.2.1.1-1.

Table 7.1.2.1.1-1.  RACH optimization targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| UE access delay probability per SSB | The probability distribution of UE access delay that is used to minimize the access delays for the UEs under the SSBs. See attribute ueAccDelayProbilityDistPerSSB in TS 28.541 [13]. | CDF of access delay |
| Number of preambles send per SSB probability | The probability of the number of preambles sent per SSB. See attribute ueAccProbilityDistPerSSB in TS 28.541 [13]. | CDF of access delay |

##### 7.1.2.1.2	Control information

The parameter is used to control the RACH optimization function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| RACH optimization control | This attribute allows authorized consumer to enable/disable the RACH optimization functionality. See attribute drachOptimizationControl in TS 28.541 [13]. | Boolean On, off |

Next modified section

### 7.1.2	MRO (Mobility Robustness Optimisation)

#### 7.1.2.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.2.2	MnS Component Type B definition

##### 7.1.2.2.1	Targets information

The targets of MRO are shown in the Table 7.1.2.2.1-1.

Table 7.1.2.2.1-1.  MRO targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total intra-RAT handover failure rate | (the number of failure events related to intra-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total inter-RAT handover failure rate | (the number of failure events related to inter-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |

##### 7.1.2.2.2	Control information

The parameter is used to control the MRO function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| MRO function control | This attribute allows the operator to enable/disable the MRO functionality. See attribute dmroControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.2.2.3	Parameters to be updated

Table 7.1.2.2.3-1.  Ranges of handover parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger | This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation (see TS 38.300 [7] and TS 38.423 [17]). See attribute maximumDeviationHoTrigger in TS 28.541 [13]. | [-20..20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0.. 604800] in unit Seconds |
| Tstore_UE_cntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in TS 38.300 [7]. See attribute tstoreUEcntxt in TS 28.541 [13]. | [0..1023] in unit 100 milliseconds |

Next modified section

### 7.1.3	PCI configuration

#### 7.1.3.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |

#### 7.1.3.2	MnS Component Type B definition

##### 7.1.3.2.1	Control information

The parameter is used to control the D-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the D-SON PCI configuration functionality. See attribute dPciConfigurationControl in TS 28.541 [13]. | enable, disable |

##### 7.1.3.2.2	Parameters to be updated

The table below lists the parameter related to the D-SON PCI configuration function.

| Parameters | Definition | Legal Values |
| --- | --- | --- |
| PCI list | The list of PCI values to be used by D-SON PCI configuration function to assign the PCI for NR cells. See attribute nRPciList in TS 28.541 [13]. | List of integers |

Next modified section

### 7.2	Management services for C-SON

### 7.2.1	PCI configuration

#### 7.2.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.1.2	MnS Component Type B definition

##### 7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. See attribute cPciConfigurationControl in TS 28.541 [13]. | disable, enable |

##### 7.2.1.2.2	Parameters to be updated

The table below lists the parameter related to the C-SON PCI configuration function.

| Updated parameters | Definition | Legal Values |
| --- | --- | --- |
| NR PCI | This parameter contains the PCI of the NR cell. See attribute cSonPciList in TS 28.541 [13]. | Integer |

End of modified section