3GPP TSG-SA5 Meeting #132e 	S5-204120

- Online, , 17 Aug- 28 Aug 2020

- Revision of S5-19xxxx

- Source:	Samsung

- Title:	pCR solution for paging optimizaion

- Document for:	Approval

- Agenda Item:	6.6.3

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Study on enhancement of management data analytics”.

# 3	Rationale

This contribution provide solution for the existing usecase of “Paging optimization”

# 4	Detailed proposal

1st Modified Section

## 6.5	Paging performance related issues

### 6.7.1		Paging optimization

#### 6.7.1.1	Use case

As per the current procedures, if the UE goes out-of-coverage (OOC) the paging which was initiated by the network Access and Mobility Management Function (AMF) fails. The re-attempts continue to fail until UE comes in the coverage and reacts to the paging attempts. This repetitive paging attempts result in the wastage of network resources. As an example, the use case includes a user or a group of users getting into an area, with no cellular coverage on a regular basis for a considerably long duration, for e.g., the user gets into a shielded room for some testing purpose every day for a defined period. The Network initiated paging for such users will fail until they are back in the area with cellular coverage. This would result in in-efficient network resource usage.

It is desirable to use MDAS (Management data analytic service) to optimize the current paging procedures in 5G networks. MDAS producer provides an analytical report containing the user(s) paging analytics indicating the time window at which the user is OOC on a regular basis at the particular location and hence will not be able to respond on a network-initiated paging. Based on the report MDAS consumer (e.g., AMF, gNB) decides on whether, when and where to initiate or not to initiate the paging procedures, thereby ensuring the efficient paging procedures and optimal network resource utilization, as paging can be initiated only when there are more chances for it to be successful.

#### 6.7.1.2	Potential Requirements

REQ-PA_OPT_CON-1: The MDAS producer should have a capability allowing the authorized consumer to get the paging analytics report describing paging results for a particular user or a group of users.

REQ-PA_OPT_CON-2: The MDAS producer should have a capability to provide the paging analytics report describing the paging results based on successful and un-successful paging attempts at a particular time and duration.

REQ-PA_OPT_CON-3: The paging analytics report describing the paging results should contain the following information:

-	User Identification: Identification of the user or a group of users.

-	Daily-OOC-Duration: Identifying the time window during which UE is out-of-coverage every day.

-	Daily-OOC-Location: Identifying the last known location before UE going out-of-coverage every day.

-	Recommended Action: The recommendation may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location.

#### 6.7.1.3	Possible Solutions

The solution requires MDAS producer to collect various data and provide the paging analytics report. Daily-OOC-Duration and Daily-OOC-Location will be included in the Paging Analytics Report mentioning the time window during which UE is out-of-coverage every day at a particular location. The paging is not initiated for the UE during the period provided as Daily-OOC-Duration if the last know UE location is the location identified by Daily-OOC-Location.

6.7.1.3.1 	Data required

The consumer (e.g AMF) can subscribe to obtain the paging analytics report for a user or a group of the user from MDAS producer. The subscription request may include identification for the target user or a group of the user, reporting interval etc. The MDAS producer collects the following per UE per day data from various sources.

| Data category | Required data |
| --- | --- |
| UE Paging Measurements | Number of successful paging attempt. |
|  | Successful Timestamp: The timestamp for each successful paging attempt |
|  | Successful Location: Last known location of UE |
|  | Number of un-successful paging attempt: Total number of un-successful paging attempt |
|  | Un-Successful Timestamp: The timestamp for each un-successful paging attempt |
|  | Un-Successful Location: Last known location of UE |

6.7.1.3.2 Analytics report

The paging analytics report contains the following information

| Paging Analyics Report | Attribute Name | Description |
| --- | --- | --- |
|  | User Identification | Identification of the user or a group of users. |
|  | Daily-OOC-Duration | Identifying the time window during which UE is out-of-coverage every day. This will be provided per UE |
|  | Daily-OOC-Location | Identifying the last known location before UE going out-of-coverage every day. This will be provided per UE |
|  | Recommended Action | The recommendation may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location. This will be provided per UE |

Based on the report and the recommendation, the consumer decides whether to change the paging strategy for a particular UE or a group of UE. If the paging policy needs to be changed, the AMF may decide whether, when and where to page the UE.  The AMF may not page the UE during the period provided as Daily-OOC-Duration if the last know UE location is the location identified by Daily-OOC-Location.

End of Modified Sections