3GPP TSG-SA5 Meeting #132e 	S5-204218

- e-meeting 17th 28th August 2020												Revision of S5-20xxxx

- Source:	Huawei Technologies

- Title:	pCR TS 28.201 Update the new charging function name

- Document for:	Approval

Agenda Item:	7.4.1

# 1	Discussion

This is a pCR to TS 28.201 updating the new charging function name.

# 2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

# 3	Rationale

This pCR proposes to update the new charging function except clause 4, in TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

# 4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

### 5.1.2	Requirements

The following are high-level charging requirements specific to the network slice performance and analytics charging:

-	The CEF shall support subscription to specific network slice performance and analytics information.

-	The CEF shall support reporting charging information based on network slice performance and analytics information.

-	The CEF  shall support reporting performance and analytics charging information per network slice.

-	The CEF shall support PEC scenario to report performance and analytics charging information.

Next changes

### 5.1.4	NWDAF discovery

The NWDAF service discovery by the CEF is specified in TS 29.510 [300].

### 5.1.5	CHF selection

The CHF address(es) selection, i.e. CHF address determination, by the CEF is done based on the following and with this priority order (highest to lowest):

-	NRF based discovery;

-	pre-configured CHF address in the CEF.

Next changes

#### 5.2.1.1	General

Network slice performance and analytics charging is performed by the CEF interacting with CHF using Nchf specified in TS 32.290 [50] and TS 32.291 [51].In order to provide the data required for the charging management activities outlined in TS 32.240 [1], the CEF shall be able to perform converged charging for each of the following:

-	the performance and analytics information obtained from NWDAF.

-	the performance and analytics information obtained from MnS;

The CEF shall be able to obtain performance and analytics information from the NWDAF and/or MnS as follows:

-	The Subscribe/Notify are exchanged between the CEF and the NWDAF using the Nnwdaf service. The interactions are issued by the CEF towards the NWDAF when certain conditions are met.

-	The Subscribe/Notify are exchanged between the CEF and the MnS producer. The interactions are issued by the CEF when certain conditions are met.

The CEF shall be able to perform converged charging by interacting with CHF, for charging data related to network slice performance and analytics. The Charging Data Request and Charging Data Response are exchanged between the CEF and the CHF, based on PEC scenarios specified in TS 32.290 [50]. The Charging Data Request is issued by the CEF towards the CHF when certain conditions (chargeable events) are met.

The charging information is collected per Single Network Slice Selection Assistance Information (S-NSSAI).

The services provided by NWDAF are defined in the TS 23.288[150] and the services provided by MnS producer are defined in the TS 28.533 [250]. The charging functionalitiesfor network slice performance and analytics, i.e. CEF functionality, charging events and charging procedures, are defined in the following clauses.

Whether performance and analytics charging for an S-NSSAI in the CEF is active or not is based on configuration. Which performance and analytics information is obtained from NWDAF or MnS by the CEF is based on configuration.

The configuration may be per S-NSSAI, detailed information about the configuration or provisioning of the configuration is outside the present document.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in clauses 5.2.2.

A detailed formal description of the converged charging parameters to CHF defined in the present document can be found in TS 32.291 [51].

A detailed formal description of the CDR parameters defined in the present document can be found in TS 32.298 [57].

#### 5.2.1.2	Applicable triggers in network slice performance and analytics charging

Triggers for CEF to invoke a Charging Data Request [Event] towards the CHF are defined as following trigger conditions.

When a charging event is issued towards the CHF, it includes details such as Single Network Slice Selection Assistance Information (S-NSSAI). Table 5.2.1.2-1 summarizes the set of default trigger conditions and their category which shall be supported by the CEF.

Table 5.2.1.2-1: Default Trigger conditions in CEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Notify event for a subscription | - | Deferred | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Expiry of time limit per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Threshold reached for a performance indicator (e.g. load level per S-NSSAI) | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Limit reached for number of notify events per subscription made by CEF | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Limit reached for number of notify events per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NOTE 1: In addition, there may be operator specific triggers. NOTE 2: The threshold and limit is default configured and updated based on the configuration. |  |  |  |  |  |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the CEF for these chargeable events, is specified in clause 5.2.3.

### 5.2.2	Message flows

#### 5.2.2.1	General

The message flows in the present document specify the interaction for performance and analytics charging functionality, based on the performance and analytics information from NWDAF and MnS.

The charging interaction is based on Charging Data Request /Response messages specified in TS 32.290 [50], exchanged between the CEF and the CHF.

One source of performance and analytics information is based on the messages specified in TS 23.288 [150], exchanged between the CEF and NWDAF.

Another source of performance and analytics information is based on the messages specified in TS 28.532 [251], exchanged between the CEF and MnS.

#### 5.2.2.2	Analytics and performance charging from CEF

##### 5.2.2.2.1	General

The clause below describes analytics and performance charging from CEF scenarios.

##### 5.2.2.2.2	General analytics and performance charging – PEC

The following figure 5.2.2.2.2-1 describes analytics and performance charging:

Figure 5.2.2.2.2-1: Analytics and performance reporting

1)	Collection of analytics and performance information: Collection of information related to the network slice analytics and performance is done by the CEF.

2)	Trigger condition met: the CEF determines based on the collected information and the trigger conditions that a report of charging information to the CHF is needed.

3)	Charging Data Request [Event]: The CEF generates charging data related to the collected information and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

4)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

5)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

#### 5.2.2.3	Network Slice data analytics subscription from NWDAF

##### 5.2.2.3.1	General

The clause below describes network data analytics subscription scenarios. The CEF acts as a NWDAF Service Consumer and uses the Nnwdaf_AnalyticsSubscription service to subscribe to the applicable analytics information required for the chargeable events, based on clause 6.1.1 in TS 23.288 [150].

##### 5.2.2.3.2	Network slice performance and analytics information Subscribe/Notify

The following figure 5.2.2.3.2-1 procedure is used by CEF to subscribe/unsubscribe at NWDAF to be notified on performance and analytics information, using Nnwdaf_AnalyticsSubscription service for Network Slice defined in TS 23.288 [150].

Figure 5.2.2.3.2-1: Analytics and performance from NWDAF

1)	Subscribe Request: the CEF decides to start the collection of performance and analytics information and subscribes to performance and analytics information from NWDAF with the performance indicator, S-NSSAI and subscription condition (e.g. Start time, stop time).

2)	Subscribe Respond: the NWDAF responses the subscription successful.

3)	Notification: based on the subscription of performance and analytics information, the NWDAF notifies the CEF with the performance and analytics information.

NOTE: 	There might be several “notification” during the subscription.

4)	Unsubscribe Request: the CEF cancels the subscription to performance and analytics information.

5)	Unsubscribe Respond: the NWDAF responses the unsubscription successful.

The CEF could update the subscription during the duration of the subscription.

#### 5.2.2.4	Network slice performance and analytics subscription using MnS

##### 5.2.2.4.1	General

CEF may support subscription to network slice performance information using MnS. The clauses below describes message flow of network slice performance and analytics charging using MnS defined in clause 11.3.1 in TS 28.532 [251] (e.g. Subscribe operation, unsubscribe operation and notifyFileReady).

##### 5.2.2.4.2	Network slice performance and analytics information Subscribe/Notify

The following figure 5.2.2.4.2-1 procedure is used by CEF to subscribe/unsubscribe the performance and analytics information using MnS.

Figure 5.2.2.4.2-1: Performance and analytics information from MnS Producer

1)	Subscribe Request: The CEF decides to start the collection of performance and analytics information and subscribes to performance and analytics information using MnS with the performance indicator, S-NSSAI and subscription condition (e.g. Start time, stop time).

2)	Subscribe Respond: the MnS producer responses the subscription successful.

3)	Notification: Based on the subscription of performance and analytics information, the MnS producer notifies the CEF with the performance and analytics information.

NOTE: 	There might be several “notification” during the subscription.

4)	Notification Acknowledge: the CEF sends the acknowledge notification.

5)	Unsubscribe Request: the CEF cancels the subscription to performance and analytics information.

6)	Unsubscribe Respond: the MnS producer responses the unsubscription successful.

The CEF could update the subscription during the duration of the subscription

Next changes

#### 6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in clause 5.1.2.2.1 of TS 32.290 [50].

Table 6.1.1.1-1 describes the use of these messages for network slice performance and analytics charging.

Table 6.1.1.1-1: network slice performance and analytics charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | CEF | CHF |
| Charging Data Response | CHF | CEF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for performance and analytics based charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Subscriber Identifier | OC | This field contains the identification of the subscriber of the network slice. |
| NF Consumer Identification | M | Described in TS 32.290 [57]. |
| NF Functionality | M | Described in TS 32.290 [57]. |
| NF Name | OC | Described in TS 32.290 [57]. |
| NF Address | OC | Described in TS 32.290 [57]. |
| NF PLMN ID | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | - | This field is not applicable. |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57]. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Notify URI | - | This field is not applicable. |
| Triggers | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | - | This field is not applicable. |
| Network Slice Performance and Analytics Charging Information | OC | This field holds the network slice information, which is reported to the CHF. |

Editor's Note: whether reuse the Multiple Unit Usage or new structure for the multiple performance charging data reports is ffs.

#### 6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CEF as used for performance and analytics based charging.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Result | OC | Described in TS 32.290 [57]. |
| Invocation Sequence Number | - | This field is not applicable. |
| Session Failover | OC | Described in TS 32.290 [57]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | - | This field is not applicable. |

Editor's Note: the table of charging data response message content is ffs.

#### 6.1.3.2	Network slice performance and analytics CHF CDR data

If enabled, network slice performance and analytics CDRs shall be produced for each network slice (i.e. S-NSSAI).

The fields of network slice performance and analytics CHF CDR are specified in table 6.1.3.2-1.

Table 6.1.3.2-1: Network slice performance and analytics CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field is the identifier of subscriber. |
| NF Consumer Information | M | This field holds the information of the CEF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. CEF) |
| NF Name | OC | This field holds the name of the CEF used. |
| NF Address | OC | This field holds the IP Address of the CEF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the CEF. |
| Record Opening Time | M | Described in TS 32.298 [57]. |
| Duration | M | Described in TS 32.298 [57]. |
| Record Sequence Number | C | Described in TS 32.298 [57]. |
| Cause for Record Closing | M | Described in TS 32.298 [57]. |
| Local Record Sequence Number | OM | Described in TS 32.298 [57]. |
| Record Extensions | OC | Described in TS 32.298 [57]. |
| NSPA Charging Information | OM | This field holds the network slice performance and analytics charging information defined in clause 6.2.1.2. |

### 6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by CEF for network slice performance and analytics converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3-1 defines the basic structure of the supported fields in the Charging Data Request message for CEF converged charging.

- Table 6.2.3-1: Supported fields in Charging Data Request message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | - |
| Subscriber Identifier |  | E |
| NF Consumer Identification |  | E |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | - |
| Retransmission Indicator |  | - |
| One-time Event |  | E |
| One-time Event Type |  | E |
| Notify URI |  | - |
| Triggers |  | E |
| Multiple Unit Usage |  | - |
| NSPA Charging Information |  |  |
| Trigger |  | E |
| Network Slice Instance Assistance Identifier |  | E |
| Performance and analytics information |  | E |

Table 6.2.3-2 defines the basic structure of the supported fields in the Charging Data Response message for CEF converged charging.

- Table 6.2.3-2: Supported fields in Charging Data Response message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Sequence Number |  | - |
| Session Failover |  | E |
| Triggers |  | - |
| Multiple Unit information |  | - |

End of changes