3GPP TSG-SA5 Meeting #132e 	S5-204309

- 17 – 28 August 2020, E-meeting

- Source:	Huawei

- Title:	Add use case and potential solutions of mobility performance analysis

- Document for:	Approval

Agenda Item:	6.6.3

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	3GPP TR 28.809: "Management and orchestration; Study on enhancement of Management Data Analytics (MDA)"

[2]	3GPP TR 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[3]	3GPP TR 28.628: " Self-Organizing Networks (SON) Policy Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

# 3	Rationale

Existing use cases regarding mobility analysis include handover optimization and Inter-gNB Beam Selection Optimization. The former mainly focuses on the selection of optimal target gNB according to handover resource analysis. The latter mainly focuses on beam level HO optimization analytics.

There are multiple handover mechanisms for different purposes, e.g. Dual Active Protocol Stack (DAPS) to reduce service interruption time during handover, Conditional Handover (CHO) to improve handover robustness, RACH-less handover to reduce handover latency etc. The SON MRO solutions can handle multiple handover robust issues such as too early handover, too late handover and handover to wrong cell etc. Furthermore, handover mechanisms are also related to NSA and SA deployment architecture. In different scenarios, handover solutions will have differents impacts on the mobility performance. The capability to provide analytics report to identify the most optimal handover mechanism should be studied.

This contribution proposes to add the use case of mobility performance analysis.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.809 [1].

1st of Change

### 6.5.x	Mobility performance analysis

#### 6.5.x.1	Use case

The mobility performance related problems may be resulted from different causes, e.g., too long mobility interruption time for latency sensitive services, low handover successful rate due to poor coverage of the cell-edge, handover failure due to lack of handover resources, too-early/too-late/pingpong handovers due to inappropriate handover parameters.

In addition, there are different handover mechanisms, e.g. Dual Active Protocol Stack (DAPS) to reduce service interruption time during handover, Conditional Handover (CHO) to improve handover robustness, RACH-less handover to reduce handover latency etc. SON MRO solutions can handle multiple handover robustness issues such as too early handover, too late handover, handover to a wrong cell and etc. Furthermore, handover mechanisms are also related with NSA and SA deployment architecture. In different scenarios, handover solutions will have differents impacts on the mobility performance. The analytics report to identify the most optimal handover mechanism may be provided by MDAS producer. For example, to satisfy the requreiments of 0ms mobility interruption time for some URLLC services, MDAS may propose to prioritize the usage of DAPS. If handover successful rate is low due to coverage issue in the cell edge, MDAS may recommend CHO instead of gNB triggered handover mechanisms. To provide optimal handover mechanisms and the corresponding handover related parameters, MDAS may consider multiple factors, e.g. radio conditions, cell load, service requirements, handover successful rate, history performance data around handover, UE RF finger prints etc. MDAS may compare performance of different handover mechansims and propose optimal handover mechanisms, e.g. the prioritization of different handover mechanisms in different conditions.

MDAS can be used to analyse service experience and network performance during handover period in different mobility scenarios. It may also be able to provide the recommendations of optimal handover parameters, resource configurations and mobility relalted policies. Mobility performance analysis should cover the following aspects:

-	Mobility scenarios in NSA and SA deployment architectures;

-	Optimal handover mechanisms, e.g. DAPS, CHO, RACH-less handover etc;

-	Optimal handover parameter and resource configurations;

-	Coordination with SON MRO mechanisms to improve handover robustness;

-	Mechanisms for fast handover failure recovery;

The MDAS producer is able to, from the perspective of the management aspects, provide the mobility performance analytics report. This analytics report can be considered as an input to support SLS assurance to perform further evaluation.

#### 6.5.x.2	Potential requirements

REQ-MOB_PMF_CON-1		The MDAS producer should have a capability to provide the analytics report of mobility performance.

REQ-MOB_PMF_CON-2		The analytics report describing the mobility performance should contain the following information describing the mobility related performance aspects:

-	Optimal handover mechanism and the corresponding parameters for DAPS, CHO, RACH-less handover and NR SON MRO scenarios etc;

#### 6.5.x.3	Possible solutions

##### 6.5.x.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide optimal handover mechanisms and the corresponding configurations regarding parameters and resources. As the table in 6.5.x.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the optimal handover mechanisms. This procedure may be triggered by the request or periodically.

##### 6.5.x.3.2	Data required for mobility performance analysis

Following table shows the potential data required to analyse the mobility performance.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | sNSSAIList; Inter-gNB handovers: see clause 5.1.1.6.1 of TS 28.552[8]; Intra-gNB handovers: see clause 5.1.1.6.2 of TS 28.552[8]; Handovers between 5GS and EPS: see clause 5.1.1.6.3 of TS 28.552[8]; RRC Connection Re-establishment: see clause 5.1.1.17 of TS 28.552[8]; Inter-AMF handovers: see clause 5.2.5.1 of TS 28.552[8]; Handovers from 5GS to EPS: see clause 5.2.5.3 of TS 28.552[8]; Handovers from EPS to 5GS: see clause 5.2.5.4 of TS 28.552[8]; Number of handover events, Number of HO failures, Number of too early HO failures, Number of too late HO failures, Number of HO failures to wrong cell, Number of unnecessary HOs to another RAT: see clause 4.3.5 of TS 28.628[y]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552[8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; NWDAF analytical data: UE mobility analytics, UE Communication Analytics; see 6.7.2 and 6.7.3 of TS 23.288 [y]; |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs related with RAN user plane congestion. |

Note: The above parameters may not be the complete list.

##### 6.5.x.3.3	Analytics report for mobility performance analysis

Following table shows the potential information carried in the analytics report of mobility performance analysis.

| Analytics Report of mobility performance | Attribute Name | Description |
| --- | --- | --- |
|  | Mobility performance issue identifier | The identifier of the mobility performance issueanalysis; |
|  | Root cause of mobility performance issue | The root cause of mobility performance issues, e.g, too long mobility interruption time for latency sensitive services, low handover successful rate due to poor coverage of the cell-edge, too-early/too-late/pingpong handovers due to inappropriate handover parameters |
|  | Recommended handover mechanisms | Recommended handover mechansims according to network conditions, e.g.,DAPS, CHO, RACH-less handover; Note: The DAPS and CHO mechanism are mutually exclusive. |
|  | Recommended handover related parameters | Corresponding configurations of handover related parameters, e.g., the range of handover offset. |
|  | Time duration | The time duration the identified handover mechanism or handover related parameters are recommended to apply. |
|  | Location | The geographical area or the cells where the the identified handover mechanism and handover related parameters are applied. |

End of changes