3GPP TSG-SA5 Meeting #132e 	S5-204337rev2

e-meeting 17th 28th August 2020	(revision of xx-yyxxxx)

- Source:	China Mobile

- Title:	New WID on N40 Interface Enhancements to Support GERAN and UTRAN

- Document for:	Approval

Agenda Item:	7.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	N40 Interface Enhancements to Support GERAN and UTRAN

## Acronym: TEI17_NIESGU

## Unique identifier: TBD

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI17_NIESGU | SA2 | 870009 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

For operators deploying a combined node serving as both SMF+PGW-C and PGW-C to provide network services for 2G/3G/4G/5G users, it is beneficial to use N40 interface for charging. SA2 is conducting a new WID on N7/N40 Interfaces Enhancements to Support GERAN and UTRAN, which indicates that PGW supports N40 interface and CHF supports processing 2G/3G accesses relevant charging information. Therefore, charging aspects need to be addressed.

## 4	Objective

The objective of this work item is to specify charging enhancements to support 2G/3G based on N40 interface, such as:

-        Specify charging functionality enhancements for PGW to support N40 interface;

-        Specify corresponding parameters and open API impacts;

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 32.240 | Update charging architecture for 5G systems-service based interface | TSG SA#92 (June 2021) |  |
| 32.251 | Update possible charging information for PGW to support N40 interface | TSG SA#92 (June 2021) |  |
| 32.255 | Update possible charging information to support GERAN and UTRAN | TSG SA#92 (June 2021) |  |
| 32.290 | Update NF Consumers for Nchf_ConvergedCharging service | TSG SA#92 (June 2021) |  |
| 32.291 | Update Nchf_ConvergedCharging service API to support GERAN and UTRAN | TSG SA#93 (Sept. 2021) |  |
| 32.298 | Update CHF-CDR ASN.1 to support GERAN and UTRAN | TSG SA#93 (Sept. 2021) |  |

## 6	Work item Rapporteur(s)

Dong, Jia, China Mobile, dongjia@chinamobile.com

## 7	Work item leadership

SA5

## 8	Aspects that involve other WGs

SA2, CT3, CT4

## 9	Supporting Individual Members

Supporting IM name

China Mobile

Matrixx

Huawei

Deutsche Telekom

Ericsson

China Telecom

Nokia

Nokia Shanghai Bell

CATT