3GPP TSG-SA5 Meeting #132-e	S5-204367rev2

- Online, , 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0145 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Add missing object creation variant with HTTP POST (REST SS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the REST SS there are two possibilities to create an object. The first one is with HTTP PUT. In this case the id of the new resource is generated by the MnS consumer. The second one is HTTP POST. In this case the id of the new resource is generated by the MnS producer. The second possibilty is missing, but required in numerous use cases, for example when creating a network slice object. The client does not want to be bothered with ids in this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The possibility to create objects with HTTP POST is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objects cannot be created with id creation by the MnS producer, which is a problem in many use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.1.1, 12.1.1.1.1, 12.1.1.1.2, 12.1.1.1.2.1 (New), 12.1.1.1.2.2 (New), 12.1.1.3.1, 12.1.1.3.2.1.2, 12.1.1.3.2.4 (New), 12.1.1.4.1, 12.1.1.4.2.4, 12.1.1.4.2.4a (New), 12.1.1.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding OpenAPI changes are in S5-204368 / CR 0146 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

#### 11.1.1.1	createMOI operation

##### 11.1.1.1.1	Description

This operation is invoked by createMOI operation service consumer to request the createMOI operation service provider to create a Managed Object instance in the MIB maintained by the createMOI operation service provider. This operation will create only one Managed Object instance.

The createMOI service consumer supplies the values of all attributes that are supported, i.e. a) attributes whose Support Qualifier is M and b) attributes whose Support Qualifier is O. The special cases are:

1)	If the attribute has a default value specified. In such case, if the createMOI service consumer supplies a value, the supplied value is used; otherwise, the default value is used.

2)	If the attribute is specified as capable of carrying a null value or carrying no information. In such case, if the createMOI service consumer supplies a (non-null) value, the supplied value is used; otherwise, the null value is used.

3)	If the attribute does not have a default value specified and is specified as incapable of carrying null value and incapable of carrying no information, if there is a createMOI service provider defined default value, then that value will be used.

##### 11.1.1.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | class | This parameter specifies the class of the new managed object instance. |
| managedObjectInstance | CM | DN | This parameter specifies the instance of the managed object that is to be created. This is a full DN according to 3GPP TS 32.300 [5]. If this parameter is absent or carries no information, the service producer shall assign the instance name. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the new managed object. These values override the values for the corresponding attributes derived from the default value set specified in the definition of the managed object's class. |

##### 11.1.1.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| managedObjectInstance | CM | DN | This parameter shall be present, if the service producer assigns the instance value of the new managed object instance. The parameter can be absent or carry no information, if the instance name is assigned by the service consumer |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the new managed object and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) |  |

The option with instance name creation by the service producer shall be used exclusively for interactions with the Slice NRM specified in TS 28.541 [3].

##### 11.1.1.1.4	Results

In case of success, the ManagedEntity instance has been created. In case of failure, indication of the failure is provided in the Output parameters.

Next modification

# 12	Management services – Stage 3

## 12.1	Generic provisioning management service

### 12.1.1	RESTful HTTP-based solution set

#### 12.1.1.1	Mapping of operations

##### 12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| createMOI | PUT | {MnSRoot}/ProvMnS/{MnSVersion}/ {URI-LDN-first-part}/{className}={id} | M |
|  | POST | {MnSRoot}/ProvMnS/{MnSVersion}/ {URI-LDN-first-part}/{className} | M |
| getMOIAttributes | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |

##### 12.1.1.1.2	Operation "createMOI"

###### 12.1.1.1.2.1	Mapping to HTTP PUT

HTTP PUT is used to create a single resource representing a managed object instance, if the identifier of the new resource is generated by the Service Consumer.

Table 12.1.1.1.2.1-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| managedObjectClass managedObjectInstance | path | …/{className}={id} | className: string id: string | M |
| attributeListIn | request body | n/a | resourcePut-RequestType | M |

Note 1: Void.

Table 12.1.1.1.2.1-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | resourceCreation-ResponseType | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for creating a resource is as follows:

1.	The Service Consumer sends a HTTP PUT request to the Service Provider.

- The target URI identifies the location of the new resource to be created.

- The message body shall carry the complete representation of the resource to be created.

2.	The Service Provider sends a HTTP PUT response to the Service Consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource and the message body the complete representation of the new resource.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

###### 12.1.1.1.2.2	Mapping to HTTP POST

HTTP POST is used to create a single resource representing a managed object instance, if the identifier of the new resource is generated by the Service Provider.

Table 12.1.1.1.2.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| managedObjectClass | path | …/{className} | className: string | M |
| attributeListIn | request body | n/a | resourcePost-RequestType | M |

Table 12.1.1.1.2.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | resourceCreation-ResponseType | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for creating a resource is as follows:

1.	The Service Consumer sends a HTTP POST request to the Service Provider.

- The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the class name of the resource to be created.

- The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The Service Provider sends a HTTP POST response to the Service Consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource and the message body the complete representation of the new resource.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

Next modification

#### 12.1.1.3	Resources

##### 12.1.1.3.1	Resource structure

Figure 12.1.1.3.1-1 shows the resource structure of the Provisioning MnS.

Figure 12.1.1.3.1-1: Resource URI structure of the Provisioning MnS

Table 12.1.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| MOI | …/{className}={id} | PUT | Creates a resource representing a managed object instance with identifier creation by the Service Consumer |
| className | …/{className} | POST | Creates a resource representing a managed object instance with identifier creation by the Service Producer |
| MOI | …/{className}={id} | GET | Retrieves one or multiple resources representing managed object instances |
| MOI | …/{className}={id} | PATCH | Modifies one or multiple resources representing managed object instances |
| MOI | …/{className}={id} | DELETE | Deletes one or multiple resources representing managed object instances |
| notificationSink | /notificationSink | POST | Sends notifications |

##### 12.1.1.3.2	Resource definitions

###### 12.1.1.3.2.1	Resource "…/{className}={id}"

12.1.1.3.2.1.1	Description

This resource represents a managed object instance.

12.1.1.3.2.1.2	URI

Resource URI: {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id}

The resource URI variables are defined in the following table.

Table 12.1.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| URI-LDN-first-part | See clause 4.4.2 of TS 32.158 [15] |
| className | Class name of the targeted resource |
| id | T of the targeted resource |

Next modification

###### 12.1.1.3.2.4	Resource "…/{className}"

12.1.1.3.2.4.1	Description

This resource represents a collection resource.

12.1.1.3.2.4.2	URI

Resource URI: {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}

The resource URI variables are defined in the following table.

Table 12.1.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| URI-LDN-first-part | See clause 4.4.2 of TS 32.158 [15] |
| className | Class name of the resource to be created |

12.1.1.3.2.4.3	HTTP methods

12.1.1.3.2.4.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.1.1.3.2.4.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| resourcePostRequestType | The resource representation of the resource to be created | M |

Table 12.1.1.3.2.4.3.1-3: Data structures supported by the POST response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| resourceCreation-ResponseType | 201 Created | This status code shall be returned when the resource is created. The representation of the created resource is returned in the response message body. | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

Next modification

#### 12.1.1.4	Data type definitions

##### 12.1.1.4.1	General

Table 12.1.1.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| dateTime-Type | 12.1.1.4.4.2 | Date and time |
| long-Type | 12.1.1.4.4.2 | Long type |
| uri-Type | 12.1.1.4.4.2 | Type of an URI |
| Types used in paths |  |  |
| className-PathType | 12.1.1.4.4.2 | Used in the path to identify a resource object |
| id-PathType | 12.1.1.4.4.2 | Used in the path to identify a resource object |
| Types in query parts |  |  |
| attributes-QueryType | 12.1.1.4.2.18 | Used in the query part of HTTP GET to select the attributes of the scoped resource objects to be returned. |
| fields-QueryType | 12.1.1.4.2.1 | Used in the query part of HTTP GET to select the resource object properties (attributes) to be returned |
| filter-QueryType | 12.1.1.4.2.2 | Used in the query part of HTTP GET, HTTP PATCH and HTTP DELETE to filter scoped resource objects |
| scope-QueryType | 12.1.1.4.2.3 | Used in the query part of HTTP GET, HTTP PATCH and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the path component of the URI |
| Types used in request bodies |  |  |
| resourcePut-RequestType | 12.1.1.4.3.4 | Used in the request body of HTTP PUT describing the resource to be created or updated |
| resourcePost-RequestType | 12.1.1.4.3.4a | Used in the request body of HTTP POST describing the resource to be created |
| jsonMergePatch-RequestType | 12.1.1.4.2.19 | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/merge-patch+json" (RFC 7396 [37]) |
| jsonEnhancedMergePatch-RequestType | 12.1.1.4.2.20 | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/3gpp-merge-patch+json" (TS 32.158 [15]) |
| jsonPatch-RequestType | 12.1.1.4.2.5 | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/json-patch+json" (RFC 6902 [36]). |
| 3gppJsonPatch-RequestType | 12.1.1.4.2.21 | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/3gpp-json-patch+json" (TS 32.158 [15]) |
| Types used in response bodies |  |  |
| error-ResponseType | 12.1.1.4.2.6 | Used in the response body of all HTTP methods in case an error occurs when the Service Producer processes the request. |
| resourceCreation-ResponseType | 12.1.1.4.2.7 | Used in the response body of HTTP PUT and HTTP POST describing the resource created |
| resourceDeletion-ResponseType | 12.1.1.4.2.8 | Used in the response body of HTTP DELETE identifying the URIs of the deleted resources |
| resourceModification-ResponseType | 12.1.1.4.2.9 | Used in the response body of HTTP PATCH describing the set of modified resources |
| resourceRetrieval-ResponseType | 12.1.1.4.2.10 | Used in the response body of HTTP GET to return the resources identified in the request for retrieval, or the selected attributes in case the fields query parameter is used |
| Types used for resources |  |  |
| resourceRepresentationType | 12.1.1.4.2.11 | Used for resource representations |
| Types used in notifications |  |  |
| notifyMOICreation-NotifType | 12.1.1.4.2.15 | Used in the request body of HTTP POST for the notification type notifyMOICreation |
| notifyMOIDeletion-NotifType | 12.1.1.4.2.16 | Used in the request body of HTTP POST for the notification type notifyMOIDeletion |
| notifyMOIAttributeValueChange-NotifType | 12.1.1.4.2.17 | Used in the request body of HTTP POST for the notification type notifyMOIAttributeValueChange |
| notifyMOIChanges-NotifType | 12.1.1.4.2.22 | Used in the request body of HTTP POST for the notification type notifyMOIChanges |
| Types referenced by the definitions above |  |  |
| additionalText-Type | 12.1.1.4.4.2 | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| mOIChange-Type | 12.1.1.4.3.4 | Single MOI change reported by notifyMOIChanges |
| correlatedNotification-Type | 12.1.1.4.3.2 | Describes the correlated notifications of a single source |
| notificationId-Type | 12.1.1.4.4.2 | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| notificationType-Type | 12.1.1.4.4.3 | Notification type (notifyMOICreation, etc.) |
| sourceIndicator-Type | 12.1.1.4.4.4 | Indicates the source of the operation that led to the generation of the notification. |
| scopeType-Type | 12.1.1.4.4.5 | Scope type of a scope |
| scopeLevel-Type | 12.1.1.4.4.2 | Scope level of a scope |
| systemDN-Type | 12.1.1.4.4.2 | Type of the System DN |
| operation-Type | 12.1.1.4.4.6 | Enum wth "create", "delete" and "replace" |

Next modification

###### 12.1.1.4.2.4	Type resourcePut-RequestType

Table 12.1.1.4.2.4-1: Definition of type resourcePut-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | resourceRepresentation-Type | Used in the request body of HTTP PUT describing the resource to be created. | M |

###### 12.1.1.4.2.4a	Type resourcePost-RequestType

Table 12.1.1.4.2.4a-1: Definition of type resourcePost-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | resourceRepresentation-Type | Used in the request body of HTTP POST describing the resource to be created. | M |

Next modification

12.1.1.4.4.3	Enumeration notificationType-Type

Table 12.1.1.4.4.3-1: Enumeration notificationType-Type

| Enumeration value | Description |
| --- | --- |
| notifyMOICreation | Notification type is notifyMOICreation |
| notifyMOIDeletion | Notification type is notifyMOIDeletion |
| notifyMOIAttributeValueChange | Notification type is notifyMOIAttributeValueChange |
| notifyMOIChanges | Notification type is notifyMOIChanges |

End of modifications