3GPP TSG-SA5 Meeting #132e 	S5-204401

- e-meeting 17th 28th August 2020

- Source:	Nokia

Title:	Update of Handover use case and solution considering user-trajectory info in subclause 6.5

- Document for:	Approval

Agenda Item:	6.6.3

# 1	Decision/action requested

Propose to update use case and solution in subclause 6.5.1.

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP draft TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

# 3	Rationale

This contribution enhances the current Handover Optimization use case and potential requirements.

# 4	Detailed proposal

First Change

## 6.5 Mobility management related issues

### 6.5.3	Handover optimization based on UE trajectory

#### 6.5.3.1	Use case

Handover optimization can benefit from knowledge about the trajectory on which the user may be moving. A trajectory here is a sequence of location coordinates, i.e. a vector that captures the sequences of coordinates within a certain time interval, derived from historical UE location data, that identifies the different directions which users may take starting at a given point. At a city junction, for example, the probability of a handover success for a user moving straight through the junction may be different from that for a user who is turning left or turning right. So, at that junction, the straight, left and right direction indicate three possible trajectories. The MDAS producer should be able to analyse historical handover performance data in combination with historical UE location and the radio characteristics like RSRP and SINR considering the possible user trajectories to identify the optimal handover configurations and target cell prioritization.

6.5.3.2	Potential requirements

REQ-HO_OPT_TR_CON-1 		The MDAS producer should have a capability to provide an analytics report indicating the possible candidate user trajectories across the cell boundaries and provide radio configuration and target gNB selection specific to each trajectory.

#### 6.5.3.3	Possible solutions

6.5.3.3.1 		Solution Description

The solution considers UE trajectories and radio conditions for selecting the target gNB. The current resource consumption is analysed with the future/predicative resource consumption to decide if the target gNB is optimal for handover or not.

6.5.3.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Consumed Radio Resource | Radio resource utilization: The physical radio resource utilization of the target gNB, see clause 5.1.1.2 of TS 28.552[8]; |
| MDT data | UE measurements related to RSRP, RSRQ, SINR time-stamped and annotated with UE location information. |

6.5.3.3.4 		Analytics report on user trajectory-based handover optimization

The gNB trajectory-based handover optimization analytics report contains the following information.

| Analytics Report of use tranjectory-based handover | Attribute Name | Description |
| --- | --- | --- |
|  | User trajectory | The predicted UE location that identifies a trajectory across a cell boundary. |
|  | Radio characteristics on user trajectory | Radio parateres e.g. RSRP, SINR, on the predicted user trajectory across a cell boundary. |
|  | Recommended actions | Recommendation for optimal gNB configuration and/or target gNB selection/prioritization based on the user trajectory information. |

End of Change