3GPP TSG-SA5 Meeting #132e 	S5-204426rev1

- e-meeting 17th 28th August 2020

- Source:	NEC

- Title:	pCR TR 28.809 Further clarifications relating to network resource allocation

- Document for:	Approval

Agenda Item:	6.6.3

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.809 v 0.4.0; Study on enhancement of Management Data Analytics (MDA)

# 3	Rationale

Network resource utilisation level is an important factor affecting slice provisioning and end user service experience. Som clarifications are therefore proposed for some relevant existing use cases.

# 4	Detailed proposal

Start of 1st modification

### 6.2.2	Resource utilization analytics

#### 6.2.2.1	Use case

The network is a resource limited system, it is therefore quite imperative to ensure an optimum resource utilization for the network so that the required resources can be efficiently allocated while ensuring no wastage or under allocation of resources to cause additional CapEx and OpEX.

The resources usage for a network, a portion of network or a network slice could be higher or lower during different time periods depending on the traffic patterns. The traffic patterns could also vary in different geographical locations (e.g., business area, entertainment area and residential area) of the network, and could vary for different network slices. It could happen that at some point in some areas or network slices the resources are in shortage status while in some other areas or network slices there are abundant resources. This may result in that the users cannot be satisfactorily served in some areas or network slices due to lack of resources even though the overall maximum capacity is sufficient. Resource shortage may affect the QoS and potentially impact user quality of service experience, e.g., lowering down the user data throughput, prolong the user data delay, raise the rejections and failures for establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increase the drops of the existing connections, sessions and resources and deteriorates user quality of service experience. Especially in shared scenarios, it is usually that network slices may share the same RAN resources, TN resources and/or some CN network resources. When resources become limited, the suddenly high increase in resource usage by one network slice may potentially affect the performance of other network slices. If resources are not limited but one network slice is in relatively higher resource usage and the others are in lower resource usage, the proportion of resource distribution may need to be reallocated without provisioning  additional spare resources in order to achieve efficient and optimum network resource usage, which can effectively reduce the CapEx and OpEx.

Therefore, it would be desirable that the spare resource of the under-utilized usage areas or network slices can be re-allocated to areas or network slices that requires more resources within the same period of time to prevent the resource shortage from happening.

The MDA can analyze the current and historical performance data related to resource usage, network traffic and user quality of service experience for the network or network slices, and identify the ongoing issues on resource utilization and predict potential issues.

The MDAS producer provides  analytics report describing the ongoing and/or potential resource utilization issues to the authorized consumers. The issues need to be described precisely, including (but not limited to) the information about which part of the network or network slice has encountered or is going to encounter the resource utilization issue, it is resource shortage or resource excess, over which time period, etc. The MDAS producer may also provide the recommendations to solve the resource utilization issues identified by the analytics report.  The recommended actions may be for example to schedule the “scale-in” and “scale out” of VNFs to dynamically (re-)allocate the virtualized resources to where they are needed, or to create/update the resource allocation policy for different network slices to allow the network slices getting different percentage of resources in different time periods according the traffic patterns and user quality of service experience. In case of sharing scenario, the MDAS producer should also weighs the resource usages of the shared network resources which belong to different network slices. If reallocating the already allocated network resource can solve the resource utilization issues, no other newly allocated network resource is needed.

The MDAS producer gets the execution reports of the actions taken by the MDAS consumer to solve the resource utilization issue, and takes the execution reports into account in the following analysis.

The MDAS producer continue analyzing the reported issue and provides update(s) if there is any status change (e.g., solved, mitigated or deteriorated) till it is solved.

#### 6.2.2.2	Potential requirements

REQ-RES-ANA-1	The MDAS producer should have a capability to provide the analytics report describing the resource utilization issue.

REQ-RES-ANA-2	The analytics report describing the resource utilization issue should contain the following information:

-	The identifier of the resource utilization issue;

-	Indication that it is an ongoing issue or potential issue;

-	The time period(s) during which the resource utilization issue has happened or is potentially going to happen;

- 	Indication that resource issue is shortage or excess over each time period;

-	Percentage of resource shortage or excess in each time period;

-	The network entities involved in the resource utilization issue;

-	The network slices involved in the resource utilization issue;

-	The recommended actions to solve the resource utilization issue.

REQ-RES-ANA-3	The MDAS producer should have a capability to weigh the allocation of shared network resource when providing the analytics report describing the resource utilization issue.

#### 6.2.2.3	Possible solutions

##### 6.2.2.3.1	Solution description

The MDAS producer correlates and analyzes the ongoing and/or potential resource utilization issues based on the the current and historical performance data related to resource usage and network traffic for the network or network slices. The required data can be from RAN domain or CN domain or both. Based on the analysis above, the MDAS producer is able to provide the domain specific or cross domain analytics report as defined in 6.2.2.3.4 related with resource utilization analytics triggered by event or periodically.

##### 6.2.2.3.2	Required data for resource utilization analysis for RAN domain

The management data required to analyze the RAN related resource utilization are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for resource utilization analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [3]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [4]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [4]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [4]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552[3]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS28.530 [5] and clause 7.15 of TS28.531[6]. |
| Network topology | Network topology data | The topology of the network for resource utilization analytics. |
| User service experience | Analysis data | User service experience relevant attributes and/or analytics analysis obtained from NWDAF or AF, see clause 6.4 of TS 23.288 [18]. |

Note: The above parameters may not be the complete list.

##### 6.2.2.3.3	Required data for resource utilization analysis for CN domain

The management data required to analyze the CN related resource utilization are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for resource utilization analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | User subscription data by performance measurement for AMF as defined in clause 5.2 of TS 28.552 [3]; Performance measurement data for SMF as defined in clause 5.3 of TS 28.552 [3]; Performance measurement data for UPF as defined in clause 5.4 of TS 28.552 [3]; Performance measurement data for NF as defined in clause 5.7 of TS 28.552 [3]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [4]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [4]; PDU sessions for network slice instance: number of PDU sessions and PDU session establishment time of network slice instance, see clause 6.4.1 and clause 6.4.3 of TS 28.554 [4]; Virtualised resource utilization: Virtualised resource utilization of Network Slice Instance, see clause 6.4.2 of TS 28.554 [4]; |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS28.530 [5] and clause 7.15 of TS28.531[6]. |
| Network topology | Network topology data | The topology of the network for resource utilization analytics. |

Note: The above parameters may not be the complete list.

##### 6.2.2.3.4	Analytics report for resource utilization analysis

Following table provides the potential contents of the domain specific or cross domain analytics report of resource utilization analysis based on the required data received as described in 6.2.2.3.2 and 6.2.2.3.3.

| Analytics Report of resource utilization analysis | Attribute Name | Description |
| --- | --- | --- |
|  | Resource utilization issue Identifier | The identifier indicates the resource utilization issue |
|  | Indication of resource utilization issue type | Indicates the type of the resource utilization issue, e.g., ongoing or potential resource utilization issue |
|  | Time period | Describes the time period(s) during which the resource utilization issue has happened or is going to happen |
|  | Indication of resource usage demand | Indicates that resource issue is shortage or excess in each time period |
|  | Percentage of resource usage demand | Describes percentage of resource shortage or excess in each time period |
|  | List of network entities | Lists the network entities involved in the resource utilization issue |
|  | A List of network slices | List of the network slices involved in the resource utilization issue |
|  | Recommended actions | Describes the recommended actions to solve the resource utilization issue |

Start of 2nd modification

### 6.3.3	Service experience related analysis

#### 6.3.3.1	Use case

In 5G network, the variety of the services provided to the verticals is an important feature.  The network requirements which support different services may vary a lot. For example, the users who require URLLC service have high requirements on the network latency and reliability, while the users who require NB-IoT service considers the maximum access UE numbers as the first priority for network requirement. The analysis of service experience of different vertical consumers is important to support the SLA assurance from the network aspect.

The UE level’s experience analysis based on the information from AF provided by NWDAF, e.g., the service MOS may be utilized by MDAS producer as one of the service performance inputs. The network performance measurements from RAN, e.g., the UE IP throughput, coverage, latency, successful handover ratio, etc. and network performance measurements from core network could also be used by MDAS producer for the analysis of service fulfilment by the network. Resource utilization efficiency may impact  the user quality of service experience as described in clause 6.2.2. The analytics information from combining the user quality of service experience and, network performance and network resource utilisation analysis could be made available to operators or verticals for further action and analysis. For example, root cause for service deteriorating and recommendation actions may be provided by MDAS producer if some of the SLA requirements cannot be achieved, operators could perform the network optimization actions when needed. The MDAS producer may also coordinate with other management services producer to perform close loop SLA assurance of the network under the operator configured policies.

End of modifications