3GPP TSG-SA5 Meeting #133-e	S5-205189

- Online, , 12th Oct 2020 - 21st Oct 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0155 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Correct notifyHeartbeat (stage 2, REST SS, OpenAPI definition) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The notification header of notifyHeartbeat has a few errors. The OpenAPI definition of notifyHeartbeat is not using definitions from comDefs.yaml. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified how to populate the notification header. Replaced local definitions with definitions from comDefs.yaml. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The notification header cannot be populated with correct values. Also, not using the definitions from comDefs.yaml may result in inconsistencies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.1.2, 12.4.1.2.1, 12.4.1.2.2, 12.4.1.3 (new), 12.4.1.4 (new), 12.4.1.5 (new), A.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

#### 11.4.1.1	Notification notifyHeartbeat

##### 11.4.1.1.1	Definition

This notification notifies the subscribed consumer(s) that the MnS producer heartbeat period has expired or that a MnS consumer requested the emission of an immediate heartbeat notification.

The emission of heartbeat notifications is controlled by the HeartbeatControl IOC (3GPP TS 28.622 [11]).

##### 11.4.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | HeartbeatControl.objectClass |  |
| objectInstance | M | HeartbeatControl.objectInstance | Instance controlling the emission of this notifyHeartbeat notification. |
| notificationId | M | -- |  |
| notificationType | M | "notifyHeartbeat" |  |
| eventTime | M | -- | Time at which the notification is emitted. The semantics of Generalised Time specified by ITU-T [17] shall be used here. |
| systemDN | M | -- |  |
| heartbeatNtfPeriod | M | HeartbeatControl.heartbeatNtfPeriod |  |

##### 11.4.1.1.3	Triggering event

###### 11.4.1.1.3.1	From-state

stateBeforeHeartbeatNotification1 OR stateBeforeHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification1 | The internal countdown timer of the MOI emitting the notifyHeartbeat notification has reached the value ‘0’ (zero). |
| stateBeforeHeartbeatNotification2 | The value of the attribute triggerHeartbeatNtf of the MOI emitting the notifyHeartbeat notification is TRUE. |

###### 11.4.1.1.3.2	To-state

stateAfterOHeartbeatNotification1 OR stateAfterOHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification1 | If From-state is stateBeforeHeartbeatNotification1 then: the internal countdown timer of the MOI is reset to the value of its heartbeatNtfPeriod attribute. |
| stateAfterHeartbeatNotification2 | If From-state is stateBeforeHeartbeatNotification2 then: the value of the internal countdown timer of the MOI is not affected. |

Next modification

## 12.4	Heartbeat

### 12.4.1	RESTful HTTP-based solution set

#### 12.4.1.1	Mapping of operations

N/A

#### 12.4.1.2	Mapping of notifications

##### 12.4.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.4.1.2.1-1.

Table 12.4.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notifications | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| notifyHeartbeat | POST | /notificationSink | M |

##### 12.4.1.2.2	Notification "notifyHeartbeat"

The IS notification parameters are mapped to SS equivalents according to table 12.4.1.2.2-1.

Table 12.4.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | request body | href | Uri | M |
|  | request body |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType (notifyHeartbeat) | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| heartbeatNtfPeriod | request body | heartbeatNtfPeriod | integer | M |

#### 12.4.1.3	Usage of HTTP

N/A.

#### 12.4.1.4	Resources

N/A.

#### 12.4.1.5	Data type definitions

##### 12.4.1.5.1	General

Table 12.4.1.5.1-1: Data types defined in the present document

| Data type | Reference | Description |
| --- | --- | --- |
| HeartbeatNotificationTypes | 12.4.1.4.2.2 | Haertbeat notification types |

Table 12.4.1.5.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| Uri | TS 28.623 [44] | URI type |
| NotificationId | TS 28.623 [44] | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| NotificationType | TS 28.623 [44] | Notification type |
| DateTime | TS 28.623 [44] | Date and time |
| SystemDN | TS 28.623 [44] | systemDN type |
| NotificationHeader | TS 28.623 [44] | Notification header |

##### 12.4.1.5.2	Structured data types

None.

##### 12.4.1.5.3	Simple data types and enumerations

###### 12.4.1.5.3.1	General

This subclause defines simple data types and enumerations that are used by the data structures defined in the previous subclauses.

###### 12.4.1.5.3.2	Simple data types

Table 12.4.1.4.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

###### 12.4.1.5.3.3	Enumeration HeartbeatNotificationTypes

Table 12.4.1.4.3.3-1: Enumeration HeartbeatNotificationTypes

| Enumeration value | Description |
| --- | --- |
| notifyHeartbeat | Notification type is notifyHeartbeat |

Next modification

## A.5.1	OpenAPI document "heartbeatNtf.yaml"

openapi: 3.0.1

info:

title: Heartbeat notification

version: 16.6.0

description: >-

OAS 3.0.1 definition of the heartbeat notification

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532 V16.6.0; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

paths: {}

components:

schemas:

HeartbeatNotificationTypes:

type: string

enum:

- notifyHeartbeat

NotifyHeartbeat:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

heartbeatNtfPeriod:

type: integer

End of modifications