- 3GPP TSG SA WG5 Meeting #136-e	S5-212040

- March 01 – 09, 2021, e-Meeting	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.814 5GC NF performance assurance use case

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

# 3	Rationale

The performance of EAS in EDN will be impacted by the performance of 5GC NFs, such as UPF, PCF, therefore, it is necessary for ECSP to collect the 5GC NF performance measurements.

This contribution proposes the 5GC NFs performance assurance use case for TR 28.814 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558 “Architecture for enabling Edge Applications”.

[3]	3GPP TR 23.758 “Study on application architecture for enabling Edge”.

[4]		3GPP TS 23.501 “System architecture for the 5G System (5GS)”.

[5]					3GPP TS 28.531 “Management and orchestration; Provisioning”.

[6]	ETSI GS NFV-IFA 011 V3.3.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification”.

[7]	ETSI GS NFV-IFA 013 V3.4.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification”.

[8]	3GPP TS 28.550 “Management and orchestration; Performance assurance”.

[9]	3GPP TS 28.532 “Management and orchestration; Generic management services”.

[10]	3GPP TS 28.622 “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[x]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”.

1st modified section

## 6.2	Specification level use cases and requirements

### 6.2.x	EAS performance assurance assisted by 5GC NF performance measurements

#### 6.2.x.1	Goal

The goal is to enable ECSP management system to collect the measurements of 5GC NFs (e.g. UPF, PCF, …) that are related to the EAS performance, where the measurements can be used to evaluate the EAS performance, and determine the actions to mitigate the issues if necessary. For example, PLMN management system has created PM job to receive UPF data volume measurements (see subclause 5.4 in TS 28.552 [x]). ECSP management system subscribes to receive the UPF data volume measurements from the PLMN management system. Then ECSP management system can correlate the UPF data volume measurements with the EAS measurements to determine if poor EAS performance is caused by UPF overload. If so, potential actions should be taken to mitigate the issue.

#### 6.2.x.2	Description

1. ECSP management system requests PLMN management system to collect the measurements of the related 5GC NFs (e.g. UPF, PCF, …) that may affect the EAS performance.

2. The PLMN management system collects and reports the 5GC measurements to the ECSP management system.

#### 6.2.x.3	Requirements

REQ-5GC-PA-FUN-1 3GPP management service producer should have the capability allowing authorized consumers (e.g. ECSP management system) to request the collection of measurements of the related 5GC NFs (e.g. UPF, PCF, …) that may affect the EAS performance.

REQ-5GC-PA-FUN-2 3GPP management service producer should have the capability to report the 5GC NF measurements to the authorized consumers (e.g. ECSP management system).

End of modified section