- 3GPP TSG SA WG5 Meeting #136-e	S5-212042

- March 01 – 09, 2021, e-Meeting	revision of S5-21abcd

- Source:	Intel

- Title:	pCR 28.814 5GC NF fault supervision use case

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2]	TS 23.558 “Architecture for enabling Edge Applications”

# 3	Rationale

The performance of EAS in EDN will be impacted by the faults of 5GC NFs, such as UPF, PCF; therefore, it is necessary for ECSP to be notified when the 5GC NFs faults are detected.

This contribution proposes the 5GC NFs fault supervision use case for TR 28.814 [1].

# 4	Detailed proposal

1st modified section

## 6.2	Specification level use cases and requirements

### 6.2.x	EAS fault supervision assisted by 5GC NF alarms

#### 6.2.x.1	Goal

The goal is to enable ECSP management system to receive alarms associated with 5GC NFs (e.g. UPF, PCF, …) that can impact the EAS performance. For example, ECSP management system subscribes to receive from PLMN management system an alarm of the UPF that transports the edge application data. ECSP management system determines if such UPF alarm may impact the EAS performance. If so, action should be taken to mitigate the issue.

#### 6.2.x.2	Description

1. ECSP management system subscribes to PLMN management system to receive alarm notifications for 5GC NF(s) that can impact the EAS performance.

2. PLMNmanagement system detects alarm(s) from a given NF.

3. PLMNmanagement system sends the NF alarm notification to ECSP management system.

#### 6.2.x.3	Requirements

REQ-5GC-FS-FUN-1 3GPP management service producer should have the capability allowing authorized consumer (e.g. ECSP management system) to subscribe the alarm notifications for associated 5GC NF that may impact the EAS performance

REQ-5GC-FS-FUN-2 3GPP management service producer should have the capability to send NF alarm notifications to the authorized consumer (e.g. ECSP management system).

End of modified section