| 3GPP TR 28.817 V0.2.0 (2020-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on access control for management service (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concepts and overview	6

4.1	Overview	6

4.2	Concept	7

4.2.1 access control	7

4.2.2 authentication	7

4.2.3 authorization	7

5	Use cases for access control for management service	7

5.1	use case – MnS is accessed by a digital portal of the operator	7

5.1.1	Description	8

5.1.2	Issue and gaps	8

Potential security issues:	8

Gap:	8

5.2	use case – MnS is accessed by an external consumer of the 3GPP management system	9

5.2.1	Description	9

5.2.2	Issue and gaps	9

Potential security issues:	9

Gap:	9

5.3	use case – MnS is accessed by a consumer in the same domain of the 3GPP management system	10

5.3.1	Description	10

5.3.2	Issue and gaps	10

Potential security issues:	10

Gap:	10

5.4	use case – MnS is accessed by a consumer in the different domain of the 3GPP management system	10

5.4.1	Description	10

5.4.2	Issue and gaps	11

Potential security issues:	11

Gap:	11

5.5	use case – trust relationship between MnS producer and consumer	11

5.5.1	Description	11

5.5.2	Issue and gaps	12

6	Potential requirements for access control for management service	12

7	Possible solutions for access control for management service	14

8	Conclusions and recommendations	14

Annex <X> (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

can		indicates that something is possible

cannot		indicates that something is impossible

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

# Introduction

# 1	Scope

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and overview

## 4.1	Overview

Service based architecture of 3GPP management system enables a management service (MnS) consumer to access and utilize capabilities of a MnS producer to provision or monitor logical networks, services or resources allocated to the consumer. 3GPP management system is built on multiple management domains (see[x]). There’re interactions between external entities and the 3GPP management system, between management functions (MnFs) of different management domains, or between MnFs in the same management domain.

Without access control for management services/interface, an attacker may gain access to capabilities of the 3GPP management system to exhaust the network resources and potentially bring down the networks and services. Attackers may also modify the configuration of other customers’ networks and resources to fail their SLA. In addition, attackers could intercept management data of other customers and infer the competitor’s business secrete and damage reputation of the management service provider. Therefore, access control for protecting MnSs provided by 3GPP management system is essential for service and network management and orchestration, especially in commercial environment where the network resource and management capabilities are shared by multiple consumers/tenants.

This study will investigate use cases related access control for the 3GPP management system, raise potential capability requirement for 3GPP management system and propose solutions to solve the issues. Access from external machine entities, between machine entities of same or different management domains, as well as from machine entities on behalf of human users will be considered in this study.

## 4.2	Concept

### 4.2.1 access control

Access control in 3GPP management system represents the set of features and procedures that allow controlling how management service consumer and management service producer communicate and interact with each other, hence protect the 3GPP management system and managed services as well as resources from unauthorized access.

Access control component(s) could determine the level of authorization of a consumer after an authentication procedure has successfully completed. In addition, access control component(s) could track the access activities of a consumer to enforce accountability for the consumer’s actions.

### 4.2.2 authentication

Authentication in 3GPP management system represents the set of securityfeatures and procedures that allow proving a management service consumer or producer is what it claims to be, based on its identity and one or more of the three factors: something a consumer/producer knows (e.g. password), something a consumer/producer has (e.g. ID badge, cryptographic key/token), something a consumer/producer is (e.g. fingerprint or other biometric data),

### 4.2.3 authorization

Authorization in 3GPP management system represents the set of features and procedures that allow ensuring an authenticated management service consumer has the corresponding access rights to services/resources related to location, time and reasons.

# 5	Use cases for access control for management service

## 5.1	use case – MnS is accessed by a digital portal of the operator

### 5.1.1	Description

On behalf of an internal or external user of a 3GPP management system, a digital portal/store front of an operator could access MnSs produced by the 3GPP management system of the operator to provision and monitor the network of the operator. There’re several example scenarios.

Scenario 1: An administrator of the operator creates/delete/update a network from the digital portal, that triggered the portal (acting as MnS consumer) calling MnSs to create/delete/update subnetwork.

Scenario 2: An administrator of the operator’s customer collects faults of a network slice (allocated to the customer) from a digital portal/store front, that triggered the digital portal/store front (acting as MnS consumer) calling MnS to subscribe and get alarms the network slice.

Scenario 3: The operator or its customer could have multiple administrators with different privileges to support separation of duties.

Note:

Internal user of 3GPP management system is a human working for the operator, e.g. an administrator of the operator, who operates on the 3GPP management system through a portal for provisioning and monitoring the 3GPP network.

External user of 3GPP management system is a human working for a customer of the operator, e.g. an administrator of the customer of the operator, who operates on the 3GPP management system through a portal/store front for monitoring and optionally provisioning the 3GPP network allocated to the customer.

### 5.1.2	Issue and gaps

#### Potential security issues:

Without proper access control, the administrator of the operator or its customer could mis-operate on the network/network slice deliberately or accidently. e.g. an external customer of an operator could delete a network of the operator, which is serving multiple customers, that could cause outage of services. Another example is an external customer of an operator could receive detail performance data, e.g. all measurements of a network slice, which was not part of SLA between the operator and customer.  In addition, a malicious user could fake the legal user to perform attacks, e.g. spoofing, man-in-the-middle, etc., that could cause availability and confidentiality compromising. Further, the malicious entity cannot be identified without tracking and auditing the access activities of the consumer.

Therefore, access control, including identification, authentication, authorization and auditing, on a digital portal/store front, as internal MnS consumer and on behalf of human user of 3GPP management system, is required to protect the MnSs and corresponding managed resources from unauthorized reading and writing. In addition, mutual authentication between digital portal/store front and MnS producer, both being digital tools, is required to avoid credential or other information leaking and damage because of fake digital portal/store front or MnS producer.

Note: internal MnS consumer is a digital tool owned by the operator who provides 3GPP management system and network

#### Gap:

Identification of MnS producer, digital portal/store front as internal MnS consumer, and internal /external user of the 3GPP management system (who would indirectly access MnS of the 3GPP management system through digital portal/store front), is not supported in existing 3GPP management system.

Authentication between MnS producer and digital portal/store front (acting as internal MnS consumer) is not supported in existing 3GPP management system.

Authorization of a digital portal/store front (acting as internal MnS consumer) on behalf of human user of the 3GPP management system is not supported in existing 3GPP management system.

Note: Generally digital portal/store front could support authentication of internal/external user of the 3GPP management system but always need to interact with 3GPP management system for granular authorization of the user, as authorization is highly depends on business logic of the 3GPP management system.

Tracking and audit the activities/behaviours of digital portal/store front as an internal MnS consumer or internal/external user of the 3GPP management system is not supported in existing 3GPP management system.

## 5.2	use case – MnS is accessed by an external consumer of the 3GPP management system

Editor’s notes: suppose in the same time only single  administrator of a vertical is bound to an external tool in this use case, supporting tool/application with binding multiple administrators/users of the vertical in the same time is FFS

### 5.2.1	Description

A vertical customer may utilize service provided by an operator to build their own E2E service, therefore a management tool of the vertical customer, which is in the data center of the vertical, could access 3GPP management system of the operator to provision and monitor services offered by the operator. E.g. An application/management tool of a vertical could call MnSs provided by an operator to create/delete/update a network slice and monitor performance and fault of the network slice.

### 5.2.2	Issue and gaps

#### Potential security issues:

Without proper access control on the external tools/applications which access the MnS of 3GPP system, attacker could utilize fraudulent tool to perform attacks, e.g. man-in-the-middle, DDoS, etc., that could cause availability and confidentiality compromising. Further, the malicious entity cannot be identified without tracking and auditing the access activities of the consumer.

Therefore, access control, including identification, authentication, authorization and auditing, on an external MnS consumer is required to protect the MnSs and corresponding managed resources from unauthorized reading and writing. In addition, mutual authentication between external MnS consumer and MnS producer is required to avoid credential or other information leaking and damage because of fake MnS consumer or MnS producer.

Note: external MnS consumer is a digital tool owned by the vertical customer and deployed outside of data centre of the operator.

In addition, as the tool/application acting as external MnS consumer is not in the data centre of operator, which can be in vertical data centre or any other location, and the functionality of the tool could be out the scope of 3GPP, the security position of the tool can be dynamic and unexpectable. Therefore, stronger, more flexible and adaptive access control are required to adapt the context change of the tools as the external MnS consumer.

#### Gap:

Identification of MnS producer and external MnS consumer is not supported in existing 3GPP management system.

Authentication between MnS producer and external MnS consumer is not supported in existing 3GPP management system.

Authorization of a external MnS consumer on behalf of administrator of a vertical, is not supported in existing 3GPP management system.

Tracking and audit the activities/behaviours of an external MnS consumer is not supported in existing 3GPP management system.

## 5.3	use case – MnS is accessed by a consumer in the same domain of the 3GPP management system

Editor’s notes: suppose the MnS consumer discussed in this case is management function/entity complied with 3GPP specification

### 5.3.1	Description

A digital tool, acting as a MnS consumer, could access MnSs produced by a MnS producer in the same management domain of an operator to perform different tasks.  E.g. Decision entity in network slice management domain may access analytics entity in the same domain to get analytics report for better decision.

### 5.3.2	Issue and gaps

#### Potential security issues:

Without proper access control between tools in the same management domain, the management function with low sensitive level could access MnS labelled as sensitive that compromised confidentiality of the system, e.g. discovery management function accesses sensitive data exposed by data MnS would possibly leak sensitive trace/MDT data to less protected entity. In addition, man-in-the-middle attack could be performed between two management functions/entities without mutual authentication between them.

Therefore, access control, including identification, authentication, authorization and auditing, on a MnS consumer of same management domain is required to protect the MnSs and corresponding managed resources from unauthorized reading and writing. In addition, mutual authentication between MnS consumer and MnS producer in same domain is required to avoid credential or other information leaking and damage because of fake MnS consumer or MnS producer.

#### Gap:

Identification of MnS producer and MnS consumer in same management domain is not supported in existing 3GPP management system.

Authentication between MnS producer and MnS consumer in same management domain is not supported in existing 3GPP management system.

Authorization of MnS consumer in same management domain is not supported in existing 3GPP management system.

Tracking and audit the activities/behaviours of a MnS consumer in same management domain is not supported in existing 3GPP management system.

## 5.4	use case – MnS is accessed by a consumer in the different domain of the 3GPP management system

Editor’s notes: suppose the MnS consumer discussed in this case is management function/entity complied with 3GPP specification

### 5.4.1	Description

A digital tool in data center of an operator, acting as a MnS consumer, could access MnSs produced by a MnS producer in another management domain of a same or different operator to provision and monitor network or resources.  E.g. NSMF could create/delete/update RAN and/or network slice subnet through MnSs provided in RAN and/or CN domain(s) and monitor performance and fault of the network slice through network slice subnet MnSs of the domain(s),.

The management functions/entities in one management domain have different functionalities, then different privileges to access the MnSs of other domains. E.g. Execution entity in network slice management domain may have permission to create/delete/update network slice subnet in RAN/CN domain, while analytics entity in network slice management domain may have permission to collect performance, trace and alarm data from RAN/CN domain, and discovery management entity may have no rights to access any MnS of RAN/CN domain.

### 5.4.2	Issue and gaps

#### Potential security issues:

Without proper access control between tools of 3GPP system, attacker could utilize fraudulent tool to perform attacks, e.g. man-in-the-middle, DDoS, etc., that could cause availability and confidentiality compromising. Further, the malicious entity cannot be identified without tracking and auditing the access activities of the consumer. In addition, without proper authorization, tools with least privilege could mis-operate on critical resource and cause outage of system.

Therefore, access control, including identification, authentication, authorization and auditing, on a MnS consumer of another management domain is required to protect the MnSs and corresponding managed resources from unauthorized reading and writing. In addition, mutual authentication between MnS consumer and MnS producer in different domain is required to avoid credential or other information leaking and damage because of fake MnS consumer or MnS producer.

In addition, as the MnS consumer may be in the data centre of another operator, stronger access control, especially stronger and feasible cross network domains authentication is required.

#### Gap:

Identification of MnS producer and MnS consumer in another management domain is not supported in existing 3GPP management system.

Authentication between MnS producer and MnS consumer in another management domain is not supported in existing 3GPP management system.

Authorization of MnS consumer in another management domain is not supported in existing 3GPP management system.

Tracking and audit the activities/behaviours of a MnS consumer in another management domain is not supported in existing 3GPP management system.

## 5.5	use case – trust relationship between MnS producer and consumer

### 5.5.1	Description

Mutual Trust should be established between MnS consumer and producer inside a management domain or inter different management domains before they interact with each other, to ensure confidentially, integrity, availability and regulation compliance of the management systems.

To build trust relationship between different entities (e.g. MnS consumer and producer), traditionally, there’re several trust models defined to establish trust relationship between different entities and allow one entity to obtain the levels of trust needed to form partnerships, collaborate with other organizations, share information, or receive information/services. The typical trust models were defined in NIST800-39 including validated trust, direct historical trust, mediated trust, mandated trust, and hybrid trust. Similarly, ETSI NFV introduced several trust models, e.g. direct trust, transitive trust, delegating trust (see ETSI GS NFV-SEC 003)

As shown in below diagram extracted from TS 28.533 ( see Figure 5.3-1: An example of Management Service deployment framework), a MnS consumer and a MnS producer could be in the same management domain, or different domains of same operation, or management domains of different stakeholders, e.g. MnS consumer is in a vertical OT system and MnS producer is in an operator’s management system.

The trust relationship between MnS consumer and producer is different for different scenarios aforementioned, therefore trust model for these scenarios could be different, and corresponding access control between MnS consumer and producer would be different as well. E.g. Validated trust model could be applied to MnFs (as MnS producer and consumer) in the same 3GPP management domain, then single factor authentication and predefined authorization policies could be used for access control between the two MnFs.  Mediated, mandated, or hybrid trust model could be more suitable for MnF (as MnS producer) in 3GPP management system and MnS consumer in vertical OT system, accordingly multi-factor authentication and SLA based authorization policies could be used for access control between the producer and consumer.

In addition, the trust relationship between same MnS consumer and producer could be changed dynamically as security context change of either MnS consumer or producer. E.g. change of operational status, package upgrade to support new features, scale to other region, security compromising, etc. Therefore, the trust model and related access control need to be updated accordingly to adapt dynamically changed situation.

Note: zero-trust model is most stringent trust model but also implies high cost. how to pragmatically apply zero-trust model in to 3GPP management system is FFS.

### 5.5.2	Issue and gaps

Trust relationship and trust model for 3GPP management system have not been specified or studies in 3GPP or other SDOs.  Applying same trust model and security control for different trust relationship could incur either undue control (e.g. apply strict access control everywhere that caused complexity and high cost) or lack of protection on some susceptible interfaces (e.g. underestimate the threats and risks on external interfaces that resulted successful attack ).

Therefore, building correct and adaptive trust model for MnS producer and consumer of 3GPP management system, and applying appropriate access control according to the trust model need to be studied and supported by 3GPP management system.

# 6	Potential requirements for access control for management service

Editor’s notes: the potential requirements derived from different use cases could be overlapped with each other, the duplicated requirements will be removed when merge the agreed pCRs to the draft of the TR.

- REQ-MNSAC-01 The 3GPP management system may have the capability to identify a MnS producer

- REQ-MNSAC-02 The 3GPP management system may have the capability to identify a digital portal acting as internal MnS consumer

Note: internal MnS consumer is a digital tool owned by the operator who provides the 3GPP management system and network

- REQ-MNSAC-03 The 3GPP management system may have the capability to identify an internal user who indirectly access MnS through a digital  portal acting as an internal MnS consumer

Note: For example, internal user may be administrator of the operator

- REQ-MNSAC-04 The 3GPP management system may have the capability to identify an external user who indirectly access MnS through a digital portal acting as an internal MnS consumer

Note: For example, external user may be administrator of the operator’s customer

- REQ-MNSAC-05 The 3GPP management system may have the capability to support mutual authentication between MnS producer and internal MnS consumer

- REQ-MNSAC-06 The 3GPP management system may have the capability to support authorization of an internal user of the 3GPP management system of the operator based on operator’s policies

- REQ-MNSAC-07 The 3GPP management system may have the capability to support authorization of an external user of the 3GPP management system of the operator based on operator’s policies and SLA between the operator and the customer the external user represented

- REQ-MNSAC-08 The 3GPP management system may have the capability to support tracking activities of an internal MnS consumer

- REQ-MNSAC-09 The 3GPP management system may have the capability to support tracking activities of an internal/external user of the 3GPP management system

- REQ-MNSAC-10 The 3GPP management system may have the capability to support provisioning more users/accounts with different privileges in 3GPP management system by authorized internal/external users

- REQ-MNSAC-11 The 3GPP management system may have the capability to identify a MnS producer

- REQ-MNSAC-12 The 3GPP management system may have the capability to identify an external MnS consumer

- REQ-MNSAC-13 The 3GPP management system may have the capability to support mutual authentication between MnS producer and external MnS consumer

- REQ-MNSAC-14 The 3GPP management system may have the capability to support multi-factor authentication of an external MnS consumer

- REQ-MNSAC-15 The 3GPP management system may have the capability to support adaptive authorization of an external MnS consumer based on operator’s policies, SLA between the operator and the customer the MnS consumer represented, and security context of the external MnS consumer

- REQ-MNSAC-16 The 3GPP management system may have the capability to support tracking activities of an external MnS consumer

- REQ-MNSAC-17 The 3GPP management system may have the capability to support tracking activities of an internal/external user of the 3GPP management system

- REQ-MNSAC-18 The 3GPP management system may have the capability to identify a MnS producer

- REQ-MNSAC-19 The 3GPP management system may have the capability to identify a MnS consumer in same management domain

- REQ-MNSAC-20 The 3GPP management system may have the capability to support mutual authentication between MnS producer and MnS consumer in same management domain

- REQ-MNSAC-21 The 3GPP management system may have the capability to support authorization of a MnS consumer in same management domain based on standards and operator’s policies

Note: standards may be 3GPP specification. E.g. some default access control policies for authorization may be defined in 3GPP specification, e.g. creating subnetwork can only be allowed by internal MnS consumer

- REQ-MNSAC-22 The 3GPP management system may have the capability to support tracking activities of a MnS consumer in same management domain

- REQ-MNSAC-23 The 3GPP management system may have the capability to identify a MnS producer

- REQ-MNSAC-24 The 3GPP management system may have the capability to identify a MnS consumer in another management domain

- REQ-MNSAC-25 The 3GPP management system may have the capability to support mutual authentication between MnS producer and MnS consumer in another management domain

- REQ-MNSAC-26 The 3GPP management system may have the capability to support mutual authentication between MnS producer and MnS consumer in another network domain

- REQ-MNSAC-27 The 3GPP management system may have the capability to support authorization of a MnS consumer in another management domain based on standards and operator’s policies if both consumer and producer belong to same operator

- REQ-MNSAC-28 The 3GPP management system may have the capability to support authorization of a MnS consumer in another management domain based on standards, operator’s policies and agreements between operators if  the consumer and producer belong to different operators

- REQ-MNSAC-29 The 3GPP management system may have the capability to support management domain of management consumer or management producer to assign different privileges to different management entities/functions in the management domain of management consumer

- REQ-MNSAC-30 The 3GPP management system may have the capability to support tracking activities of a MnS consumer in another management domain

- REQ-MNSAC-31 The 3GPP management system may have the capability to support evaluating trust relationship (or trust level) between MnS producer and consumer.

- REQ-MNSAC-32 The 3GPP management system may have the capability to support creating trust model for MnS producer and consumer based on trust relationship between MnS producer and consumer.

- REQ-MNSAC-33 The 3GPP management system may have the capability to support defining access control polices for interaction between MnS producer and consumer based on trust model for the MnS producer and consumer.

Note: e.g. access control policies could be multi-factor authentication, access allowed in specific time or for specific duration, access allowed in specific region, etc.

- REQ-MNSAC-34 The 3GPP management system may have the capability to support reevaluating trust relationship (or trust level) between MnS producer and consumer based on changes of either MnS consumer or producer, and update trust mode and corresponding access control polices accordingly.

# 7	Possible solutions for access control for management service

# 8	Conclusions and recommendations

# Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-11 | SA5#134e | xxx | - | - | - | Initial skeleton | 0.0.0 |
| 2021-02 | SA5#135e | S5-211450 S5-211451 S5-211452 S5-211453 S5-211454 S5-211455 |  |  |  | access control overview access control concept use case - MnS is accessed by a digital portal of the operator use case - MnS is accessed by an external consumer of the 3GPP system use case - MnS is accessed by a consumer in the same domain use case - MnS is accessed by a consumer in the different domain of 3GPP system | 0.1.0 |