3GPP TSG-SA5 Meeting #137-e	S5-213418

# Online, , 10th May 2021 - 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0510 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct inconsistencies in definitions around network slice management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Inconsistencies between stage 2 and stage 3 on naming of parameter defined for configuration of user deterministic communication; Mistake in spelling of “uLThptPerSlice” attribute name in ch.6.3.3 and N.2.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Data type and parameters names used for deterministic communication is changed to deterministicComm and DeterministicComm; Mistake in spelling of uLThptPerSlice attribute name is corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3, 6.3.7, 6.4, N.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/commits/S5-213418_R-17_CR28.541_Correct_inconsistencies_in_definitions_around_network_slice_management |  |  |  |  |  |  |  |  |

1st modified section

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This data type represents the properties of network slice related requirement that should be supported by the  NetworkSlice instance in 5G network. The network slice can be tailored based on the specific requirements adhered to SLA agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP), see clause 2 of [50]. An NSP may add additional requirements not directly derived from SLA’s, associated to the NSP internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageArea | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| deterministicComm | O | T | T | F | T |
| dLThptPerSlice | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSlice | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| userMgmtOpen | O | T | T | F | T |
| v2XCommModels | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| jitter | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| reliability | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| nBIoT | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| positioning | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |

NOTE:	The attributes in ServiceProfile represent mapped requirements from an NSC (e.g. an enterprise) to an NSP

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Next modified section

### 6.3.7	DeterministicComm <<dataType>>

#### 6.3.7.1	Definition

This data type represents the properties of the deterministic communication for periodic user traffic. Periodic traffic refers to the type of traffic with periodic transmissions.

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| availability | M | T | F | F | T |
| periodicityList | M | T | T | F | T |

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Next modified section

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| availability | This parameter specifies the communication service availability requirement, expressed as a percentage. The communication service availability is defined in clause 3.1 of TS 22.261 [28]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| operationalState | It indicates the operational state of the network slice or the network slice subnet. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice or the network slice subnet. It describes the permission to use or prohibition against using the managed object instance, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: NsInfo multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nSInstanceId | This attribute specifies the identifier of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nsName | This attribute specifies the name of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| description | This attribute specifies the description of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| category | This attribute specifies the category of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: character, scalability | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| tagging | This attribute specifies the tagging of a service requirement/attribute of GST in character category (see GSMA NG.116 [50]). allowedValues: performance, function, operation | type: ENUM multiplicity: 1…3 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| exposure | This attribute specifies exposure mode of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: API, KPI | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the network slice new to be created or the existing network slice to be re-used. sNSSAList is defined in subclause 4.4.1 |  |
| perfReq | This parameter specifies the requirements to the network slice subnet in terms of the scenarios defined in the TS 22.261 [28] and TS 22.104 [51], i.e. the "performance requirements for high data rate and traffic density scenarios" in TS 22.261 [28], "periodic deterministic communication, aperiodic deterministic communication, non-deterministic communication, and mixed traffic" in TS 22.104 [51]. It is a structure containing the following elements: - list of perfReq Depending on the sST value, the list of perfReq will be - list of eMBBPerfReq or - list of uRLLCPerfReq or - list of mIoTPerfReq NOTE 1: the list of mIoTPerfReq is not addressed in the present document. allowedValues: - list of eMBBPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. An entry has the following attributes: expDataRateDL (Integer), expDataRateUL (Integer), areaTrafficCapDL (Integer), areaTrafficCapUL (Integer), overallUserDensity (Integer), activityFactor (Integer), (see table 7.1-1 of TS 22.261 [28]). - list of uRLLCPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in clauses 5.2 through 5.5 of TS 22.104 [51]. An entry has the following attributes: cSAvailabilityTarget (Float), cSReliabilityMeanTime (String), , expDataRate (Integer), msgSizeByte (String), transferIntervalTarget (String), survivalTime (String), , , (see table 5.2-1, table 5.3-1, table 5.4-1 and table 5.5-1 of TS 22.104 [51]). NOTE 2: Limitation on attribute values in SliceProfile is not addressed in the present document. NOTE 3: The attributes inside perfReq here need further breaking down to define requirements for each subnetwork under different SST values. | type: PerfReq multiplicity: *1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice or network slice subnet instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of Tracking Areas for the network slice . allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| topSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through all domains of the network slice and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| CNSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through CN domain of the network slice and is used to evaluate the delay in CN domain, e.g. time between received UL/DL packet on N3/N6 interface of UPF and successfully sent out the packet on N6/N3 interface. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| RANSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through RAN domain of the network slice and is used to evaluate the delay in RAN domain, e.g. time between received UL/DL packet on air interface/NgU of gNB and successfully sent out the packet on NgU/air interface of the gNB. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice may be shared with another network slice(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet may be shared with another network slice subnet(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice | type: ServiceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet | type: SliceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type in a ServiceProfile to be supported by a network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| delayTolerance | An attribute specifies the properties of service delivery flexibility, especially for the vertical services that are not chasing a high system performance. See clause 4.3 of TS 22.104 [51]. | type: DelayTolerance multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DelayTolerance.support | An attribute specifies whether or not the network slice supports service delivery flexibility, especially for the vertical services that are not chasing a high system performance. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| deterministicComm | An attribute specifies the properties of the deterministic communication for periodic user traffic, see clause 4.3 of TS 22.104 [51]. | type: DetermininsticComm multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterministicComm.availability | An attribute specifies whether or not the network slice supports deterministic communication for period user traffic. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterministicComm.periodicityList | An attribute specifies a list of periodicities supported by the network slice for deterministic communication. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| dLThptPerSlice | This attribute defines achievable data rate of the network slice in downlink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in downlink that is available ubiquitously across the coverage area of the slice. | type: DLThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerUEPerSubnet | This attribute defines data rate supported by the network slice subnet per UE. | type: DLThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| guaThpt | This attribute describes the guaranteed data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| maxThpt | This attribute describes the maximum data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLThptPerSlice | This attribute defines achievable data rate of the network slice in uplink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in uplink that is available ubiquitously across the coverage area of the slice. | type: ULThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerUEPerSubnet | This attribute defines data rate supported by the network slice subnet per UE. | type: ULThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxPktSize | This parameter specifies the maximum packet size supported by the network slice or the network slice subnet, refer NG.116 [50]. | type: MaxPktSize multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxPktSize.maxsize | This parameter specifies the maximum packet size supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice, refer NG.116 [50]. | type: MaxNumberofPDUSessions multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxNumberofPDUSessions.nOofPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| kPIMonitoring | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: KPIMonitoring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| KPIMonitoring. kPIList | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| nBIoT | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. | type: NBIoT multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NBIoT.support | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| synchronicity | An attribute specifies whether synchronicity of communication devices is supported, Two cases are most important in this context, see clause 3.4.29 of NG.116 [50]: - Synchronicity between a base station and a mobile device and - Synchronicity between mobile devices. | type: Synchronicity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Synchronicity.availability | An attribute specifies whether synchronicity of communication devices is supported, see NG.116 [50]. allowedValues: "NOT SUPPORTED", "BETWEEN BS AND UE", "BETWEEN BS AND UE & UE AND UE". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Synchronicity.accuracy | An attribute specifies the accuracy of the synchronicity, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| userMgmtOpen | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. | type: UserMgmtOpen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| UserMgmtOpen.support | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| v2XCommModels | An attribute specifies whether or not the V2X communication mode is supported by the network slice. | type: V2XCommMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| V2XCommMode.v2XMode | An attribute specifies whether or not the V2X communication mode is supported by the network slice. allowedValues: "NOT SUPPORTED", "SUPPORTED BY NR". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| coverageArea | An attribute specifies the coverage area of the network slice, i.e. the geographic region where a 3GPP communication service is accessible, see Table 7.1-1 of TS 22.261 [28]) and NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| termDensity | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: TermDensity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| TermDensity.density | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| positioning | An attribute specifies whether the network slice provides geo-localization methods or supporting methods, see clause 3.4.20 of NG.116 [50]. | type: Positioning multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.availability | An attribute specifies if this attribute is provided by the network slice and contains a list of positioning methods provided by the slice. If the list is empty this attribute is not available in the network slice and the other parameters might be ignored, see NG.116 [50]. Comma separated multiple values are allowed: CIDE-CID (LTE and NR), OTDOA (LTE and NR), RF fingerprinting, AECID, Hybrid positioning, NET-RTK. | type: ENUM multiplicity: 1..6 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.predictionfrequency | An attribute specifies how often location information is provided. This parameter simply defines how often the customer is allowed to request location information. This is not related to the time it takes to determine the location, which is a characteristic of the positioning method, see NG.116 [50]. allowedValues: "PERSEC", "PERMIN", "PERHOUR". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.accuracy | An attribute specifies the accuracy of the location information. Accuracy depends on the respective positioning solution applied in the network slice, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| activityFactor | An attribute specifies the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. See Table 7.1-1 of TS 22.261 [28]). | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uESpeed | An attribute specifies the maximum speed (in km/hour) supported by the network slice or network slice subnet at which a defined QoS can be achieved. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| jitter | An attribute specifies the deviation from the desired value to the actual value when assessing time parameters, see clause C.4.1 of TS 22.104 [51]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| survivalTime | An attribute specifies the time that an application consuming a communication service may continue without an anticipated message. See clause 5 of TS 22.104 [51]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| reliability | An attribute specifies in the context of network layer packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [28] and TS 22.104 [51]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| NetworkSlice.networkSliceSubnetRef | This holds a DN of NetworkSliceSubnet relating to the NetworkSlice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnet.networkSliceSubnetRef | This holds a list of DN of constituent NetworkSliceSubnet supporting NetworkSliceSubnet instance | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedFunctionRef | This holds a list of DN of ManagedFunction instances supporting the NetworkSliceSubnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| ipAddress | This parameter specifies the IP address assigned to a logical transport interface/endpoint. It can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). See note 1 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceId | This parameter specifies the identify of a logical transport interface. It could be VLAN ID (See IEEE 802.1Q [39]), MPLS Tag or Segment ID. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nextHopInfoList | This parameter is used to identify ingress transport node. Each node can be identified by any of combination of IP address of next-hop router of transport network, system name, port name, IP management address of transport nodes. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| qosProfileRefList | This parameter specifies reference to QoS Profile for a logical transport interface. A QoS profile includes a set of parameters which are locally provisioned on both sides of a logical transport interface. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| maxDLDataVolume | An attribute specifies the maximum DL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552[69]). The unit is MByte/day. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxULDataVolume | An attribute specifies the maximum UL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552[69]). The unit is MByte/day. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| serviceType | An attribute specifies the standardized network slice type. allowedValues: eMBB, URLLC, MIoT, V2X. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| epApplicationRef | This parameter specifies a list of application level EPs associated with the logical transport interface. See note 2. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| epTransportRef | This parameter specifies a list of transport level EPs associated with the application level EP (i.e. EP_N3 or EP_NgU) or network slice subnet. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| sliceSimultaneousUse | This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and if so, with which other classes of network slices. allowedValues: “0”, “1”, “2”, “3”, “4”. “0”: Can be used with any network slice “1”: Can be used with network slices with same SST value “2”: Can be used with any network slice with same SD value “3”: Cannot be used with another network slice “4”: Cannot be used by a UE in a specific location | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NOTE 1: There is no direct relationship between localAddress/remoteAddress in EP_RP and ipAddress in EP_transport. While the localAddress/remoteAddress in EP_RP could be exchanged as part of signalling between GTP-u tunnel end points, ipAddress in EP_transport is used for transport routing. NOTE 2: Application level EP represents EP_RP defined in TS 28.622 (see [30]). e.g. including EP_NgC, EP_N3, etc... |  |  |

Next modified section

## N.2.4	module _3gpp-ns-nrm-serviceprofile.yang

<CODE BEGINS>

submodule _3gpp-ns-nrm-serviceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslice { prefix ns3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "A network slice instance in a 5G network.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-06-23 {

description "Initial revision";

reference "3GPP TS 28.541 V15.X.XX";

}

typedef availability-percentage {

description "

Percentage value of the amount of time the end-to-end communication

service is delivered according to an agreed QoS, divided by the amount

of time the system is expected to deliver the end-to-end service

according to the specification in a specific area.";

reference "3GPP TS 22.261 3.1";

type decimal64 {

fraction-digits 4; // E.g. 99.9999

range 0..100;

}

}

typedef Category-enum {

type enumeration {

enum character;

enum scalability;

}

}

typedef Tagging-enum {

type enumeration {

enum performance;

enum function;

enum operation;

}

}

typedef Exposure-enum {

type enumeration {

enum API;

enum KPI;

}

}

typedef Support-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED;

}

}

grouping ServAttrComGrp {

leaf category {

description "This attribute specifies the category of a service

requirement/attribute of GST";

type Category-enum;

}

leaf-list tagging {

description "This attribute specifies the tagging of a service

requirement/attribute of GST in character category";

when "../category = 'character'";

type Tagging-enum;

}

leaf exposure {

description "This attribute specifies exposure mode of a service

requirement/attribute of GST";

type Exposure-enum;

}

}

typedef DeterministicCommAvailability {

type Support-enum;

}

grouping DLThptGrp {

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf guaThpt {

description "This attribute describes the guaranteed data rate.";

type uint64;

units kbits/s;

}

leaf maxThpt {

description "This attribute describes the maximum data rate.";

type uint64;

units kbits/s;

}

}

typedef V2XMode-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED_BY_NR;

}

}

grouping ServiceProfileGrp {

leaf serviceProfileId {

description "Service profile identifier.";

type types3gpp:DistinguishedName;

}

list sNSSAIList {

description "The S-NSSAI list to be supported by the new NSI to be

created or the existing NSI to be re-used.";

min-elements 1;

key idx;

unique "sst sd";

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses types5g3gpp:SNssai;

}

list pLMNIdList {

description "List of PLMN IDs.";

min-elements 1;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

leaf maxNumberofUEs {

description "The maximum number of UEs that may simultaneously

access the network slice instance.";

mandatory true;

type uint64;

}

leaf-list coverageArea {

min-elements 1;

description "A list of TrackingAreas where the NSI can be selected.";

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate utilization

performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

reference "3GPP TS 22.261 clause 6.2.1";

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice instance may be shared with another network slice

instance(s).";

type types3gpp:ResourceSharingLevel;

}

//Stage2 issue: The sNSSAIList above specifies one or potentially

//              several sST objects for the service profile.

//              How do they relate?

leaf sST {

description "Specifies the slice/service type. See 3GPP TS 23.501

for defined values.";

mandatory true;

type uint32;

reference "3GPP TS 23.501 5.15.2.2";

}

leaf availability {

description "The availability requirement for a network slice

instance, expressed as a percentage.";

type availability-percentage;

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type Support-enum;

}

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess deterministicComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterministicCommAvailability

config false;

type DeterministicCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

list dLThptPerSlice {

description "This attribute defines achievable data rate of the

network slice in downlink that is available ubiquitously across

the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses DLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the network

slice per UE";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses DLThptGrp;

}

list uLThptPerSlice {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines achievable data rate of the

network slice in uplink that is available ubiquitously across

the coverage area of the slice";

uses DLThptGrp;

}

list uLThptPerUE {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines data rate supported by the

network slice per UE";

uses DLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf nOofPDUSessions {

//Stage2 issue: Not defined in 28.541, guessing integer

type uint32;

}

}

list kPIMonitoring {

description "Represents performance monitoring";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf kPIList {

//Stage2 issue: Data format not specified, low interoperability

description "An attribute specifies the name list of KQIs and KPIs

available for performance monitoring";

type string;

}

}

list userMgmtOpen {

description "An attribute specifies whether or not the network slice

supports the capability for the NSC to manage their users or groups

of users’ network services and corresponding requirements.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

type Support-enum;

}

}

list v2XCommModels {

description "An attribute specifies whether or not the V2X

communication mode is supported by the network slice.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf v2XMode {

type V2XMode-enum;

}

}

list termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

type decimal64 {

fraction-digits 1;

}

}

leaf uESpeed {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

type uint32;

units km/h;

}

leaf jitter {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the deviation from the desired

value to the actual value when assessing time parameters";

reference "TS 22.104 clause C.4.1";

type uint32;

units microseconds;

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string;

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string;

}

leaf maxDLDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

leaf maxULDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

list nBIoT {

description "An attribute specifies whether NB-IoT is supported in

the RAN in the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether NB-IoT is supported

in the RAN in the network slice";

type Support-enum;

}

}

}

}

<CODE ENDS>

End of modified section