3GPP TSG-SA5 Meeting #137-e	S5-213420

# electronic meeting, Online , 10th - 19th May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | - | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PMs on inter-gNB successful and failed handover execution per beam pair |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-ADPM5G |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inter-gNB Beam Selection Optimization use case was introduced in the MDAS study TR 28.809 and it is a part of the mobility management category of use cases selected for the normative phase. Currently there is no PMs to support analytics regarding inter-gNB handover execution per beam pair. This contribution aims to introduce these missing PMs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing PMs on inter-gNB successful and failed handover execution per beam pair. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of PM for inter-gNB handover execution per beam pair. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

###### 5.1.1.6.1.12	Number of successful handover executions per beam pair

a)	This inter gNB handover measurement provides the number of successful handover executions received by the source gNB per beam pair, i.e. beam in the source and beam in the target cell.

b)	CC

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB following a successful handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF following a successful inter gNB handover, the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeInterSSBSucc

f)	NRCellCU,
NRCellRelation, 
Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.1.13	Number of failed handover executions per beam pair

a) This inter gNB handover measurement provides the number of failed handover executions received by the source gNB per beam pair. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB indicating an unsuccessful inter gNB handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF indicating an unsuccessful inter gNB handover.

The failure causes are listed for the UE CONTEXT RELEASE in [13] and for UE CONTEXT RELEASE COMMAND in [11]. Each received message increments the relevant subcounter per failure cause by 1.

FFS how the beam pair is identified

d)	Each subcounter is an integer value.

e)	MM.HoExeInterSSBFail.cause.

Where cause identifies the failure cause of the UE CONTEXT RELEASE or UE CONTEXT RELEASE COMMAND message.

f)	NRCellCU,
NRCellRelation, 
Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

2nd Modified Section

# A.17	Monitoring of handovers

Mobility is one of the most significant feature of the mobile networks, and handover is one typical action of the mobility. The handover failure would cause service discontinuation, thus the performance of the handover has direct impact to the user experience.The handover procedure includes handover preparation, handover resource allocation and handover execution, and the performance related to handover needs to be monitored for each phase. The resources (e.g., PDU Session Resource) need to be prepared and allocated for a handover according to the QoS requirements for each S-NSSAI.

The handover could occur intra-gNB and inter-gNB for 5G networks, and for inter-gNB case the handover could happen via NG or Xn interface. The handover could occur Intra-frequency and Inter-frequency for 5G networks. The handover could also occur between 5GS and EPS.

It is also important to have information about the used beams in the source and target cells in order to optimize the handover performance taking beam ids into account.

For the handover failures, the measurements with specific causes are required for trouble shooting.

The handover parameters setting could be specific for each NCR, and the handover performance could vary significantly for different NCRs, therefore the performance needs to be measured per NCR to support handover parameters optimization when necessary.