3GPP TSG SA WG5 Meeting 137-e		S5-213063r1

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei

- Title:	Add MDA reporting related service component

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple MDA capabilities are described in [1]. To require and subscribe desired MDA reports by the MDAS consumer, the MDA service has the capability to support the consumer to set input parameters. For different MDA types, some common attributes including input and output can be defined. Besides, three reporting mechnisms are discussed in [1], the consumer is allowed to request the desired reporting for MDA report.

This contribution is proposed to define the  MDA Reporting Control for MDA.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 28.532: "Management and orchestration; Management services"

[y]	3GPP TS 28.622: "Management and orchestration; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

Second of changes

# 8.2	MDA report requesting and reporting

### 8.2.1	Description

To request desired MDA reports by the MDAS consumer, the MDA service has the capability to support the consumer to set input parameters.

This clause defines the MDA related service components to request desired MDA reports.

### 8.2.2	Requirements

#### 8.2.2.1	Requirements on requesting MDA report

Req-MdaRequestingReq-01: The MDA producer is capable to allow the consumer to request desired analytics reports.

8.2.3	Service components

#### 8.2.3.1	Service component A

Following table provides the common service component A for MDA report requesting to support the MDA capabilities.

Table 8.2.3.1-1 Service Component A for MDA report requesting

| 8.2.3.2 Service component B Provisioning for MDA report requesting | Service Component A | Descriptions |
| --- | --- | --- |
| Creation of MDA report requesting | CreateMOI operation in clause 11.1.1.1, TS 28.532[x] | This operation is utilized together with the MOI defined in clause 8.2.3.2.3.1 to create the MDA report requesting. |
| Modification of MDA report requesting | ModifyMOI operation in clause 11.1.1.3, TS 28.532[x] | This operation is utilized together with the MOI defined in clause 8.2.3.2.3.1 to modify the MDA report requesting. |
| Deletion of MDA report requesting | DeleteMOI operation in clause 11.1.1.4, TS 28.532[x] | This operation is utilized together with the MOI defined in clause 8.2.3.2.3.1 to delete the MDA report requesting. |

This clause is to provide the common service component B for MDA report requesting and reporting for supporting the MDA capabilities.

##### 8.2.3.2.1	Imported and associated information entities and local labels

##### 8.2.3.2.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC,Top | Top |

##### 8.2.3.2.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, ManagedEntity | ManagedEntity |

##### 8.2.3.2.2	Class diagram

###### 8.2.3.2.2.1	Relationships

Figure 8.2.3.2.2.1-1: MDA report control NRM fragment

###### 8.2.3.2.2.2	Inheritance

Figure 8.2.3.2.2.2-1: MDA Report Control NRM fragment

##### 8.2.3.2.3	Class definitions

###### 8.2.3.2.3.1	MdaReportingControl

8.2.3.2.3.1.1	Descriptions

This <<IOC>> MdaReportingControl represents the MDA reporting Control parameters of particular MDA service capabilities.

To request a desired MDA report, a MDAS consumer has to create MdaReportingControl object instance on the MDAS producer.

When a MDAS consumer wishes to stop or modify MDA reporting, the MnS consumer shall delete or modify the corresponding MdaReportingControl instance.

8.2.3.2.3.1.2	Attributes

The MdaReportingControl IOC includes attributes inherited from Top IOC (defined in clause 4.3.29, TS 28.622[y]) and the following attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mdaCapabilityName | M | T | T | F | T |

Editor’s Notes: More attributes need to be added according to the discussion.

##### .2.2	Attributes

8.2.3.2.4	Attribute definitions

###### 8.2.3.2.4.1.	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mdaCapabilityName | It indicates the capability name defined in clause 7. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes