3GPP TSG SA WG5 Meeting 137-e		TDoc <S5-213131>

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei Technologies Co.,Ltd

- Title:	pCR 28.814 clarify EAS deployment solution using EASLcm IOC

- Document for:	Approval

Agenda Item:	6.5.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

[3]		3GPP TS 28.531 “Management and orchestration; Provisioning”.

# 3	Rationale

Based on the normal NF instance creation procedure by using provisioning MnS as described in TS 28.531 [3], it is required to have at least the function IOC for EAS lifecycle management.

In addition, to align with the solutions for EES and ECS lifecycle management using provisioning MnS, it is suggested to make EASLcm IOC as optional.

# 4	Detailed proposal

1st modified section

- 7.2 		EAS lifecycle management using using provisioning MnS

- 7.2.1	General

Figure 7.2.1-1 shows that the ASP requests the EAS instantiation via the provisioning MnS.

Figure 7.2.1-1: EAS lifecycle management via provisioning MnS

To support EAS deployment via provisioning MnS, the following IOCs could be defined:

-  EASLcm IOC that contains the requirement for EAS deployment:

-  EASLcm IOC ID: the identifier of the EASLcm IOC.

-  Service area: the geographical service area that the EAS serves (see clause 8.2.4 in [2]), for example geolocation (longitude, latitude); access network defined (e.g. Cell IDs, Tracking Areas); network defined (e.g. a specific EDN(s)), civic address.

-  EAS VNF information: The information needed to instantiate the EAS VNF.

-  Software image information including location, minimum RAM and disk requirements for the software image. (see clause 7.1.6.5 in in ETSI NFV IFA-011 [6])

-  Virtual compute resources including virtual CPU, virtual memory and virtual disk

-  QoS requirements: e.g. bandwidth.

- Affinity/Anti-affinity: The affinity and ant-affinity requirements for the EAS with other existing EAS on the target EDN.

-  EASFunction IOC that contains the following, but not limited to, attributes:

-  EASFunction IOC ID: the identifier of EAS VNF instance

-  reference of associated EASLcm IOC ID: the identifier of EASLcm IOC.

-  EAS profile information elements (see clause 8.2.4 in [2]).

- 7.2.2	EAS instantiation

The following procedures creates/instantiate EAS including selecting appropriate EDN to be used. The procedure is used by ASP (Application Service Provider, see TS 23.558) to host their application on an available EDN.

Figure 7.2.2-1: EAS instantiation via provisioning MnS

Note: The Provisioning MnS is exposed by the management system of entity responsible for EAS lifecycle management e.g ECSP management system.

- ASP consumes the provisioning MnS and invoke operation createMOI for EASLcm IOC to request the provisioning MnS Producer to instantiate the EAS.

- Provisioning MnS Producer selects the appropriate EDN (or set of EDNs) based on the EAS requirements provided in the request. The EDN can be selected either by considering the individual requirement or by grouping the multiple requirements as single selection criteria. When the grouping is used all the requirement specified as part of a group should be satisfied. Following are examples of various grouped selection criteria:

  - Latency and Cost can be combined as one selection criteria influencing the selection as in what latency can be assured in what cost.

  - Geo-location and Latency can be combined as one selection criteria influencing the selection as in at what location the requested latency can be provided depending on the target geographical area.

  - Affinity/Anti-affinity and Cost can be combined as one selection criteria influencing the selection as in what cost will be incurred to satisfy Affinity/Anti-affinity with a particular existing Edge Application.

- Provisioning MnS Producer selects the appropriate EES if available in the selected EDN based on the information provided in the request.

- ECSP Management System derives the requirements for VNF instance based on the EAS requirements.

- ECSP Management System requests NFVO via the Os-Ma-nfvo interface to instantiate EAS VNF instances.

- Provisioning MnS Producer creates the MOI for EASFunction IOC.

- Provisioning MnS Producer sends notifyMOICreation to notify ASP about the newly created MOIs for EASLcm IOC and EASFunction IOC.

- Provisioning MnS Producer sends notifyMOICreation to notify the consumer (ECSP) that MOIs for EASLcm IOC and EASFunction IOC have been created.

NOTE: It is assumed that ASP and ECSP consumer have subscribed to receive notification from ECSP management system.

- 7.2.3	EAS termination

ASP consumes the provisioning MnS with operation deleteMOI to request the Provisioning MnS Producer to terminate the EAS instance(s), including (but not limited to) the following attributes:

-  EAS VNF instance ID: the identifier of MOIs for EASFunction IOC indicating the EAS VNF instance(s) to be terminated.

ECSP management system will perform the following:

-  requests NFVO to terminate the VNF instances based on the EAS VNF instance ID.

-  deletes MOIs for EASFunction IOC.

- sends notifyMOIDeletion to notify the ASP that MOI for EASFunction IOC has been deleted.

- sends notifyMOIDeletion to notify the consumer (ECSP) that MOI for EASFunction IOC has been deleted.

NOTE: It is assumed that ASP and ECSP consumer have subscribed to receive notification from ECSP management system.