3GPP TSG SA WG5 Meeting 137-e		S5-213167

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei,China Telecom, China Mobile

- Title:	Add concrete IntentExpectation for radio network deployment and radio service deployment

- Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.4.0”.

# 3	Rationale

This contribution proposes to add concrete IntentExpectation for radio network deployment and radio service deployment.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services"

[X]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd Change

#### 6.2.2.2	Network and service deployment

##### 6.2.2.2.1	Class diagram

###### 6.2.2.2.1.1	Relationship

###### 6.2.2.2.1.2	Inheritence

##### 6.2.2.2.2	Class definition

###### 6.2.2.2.2.1	RadioNetworkDeployExpectation <<IOC>>

6.2.2.3.2.1.1	Definition

This IOC represents the properties of an IntentExpectation for radio network deployment. RadioNetworkDeployExpectation <<IOC>> is one realization of abstract class IntentExpectation <<IOC>>.  The RadioNetworkDeployExpectation represent the MnS consumer’s expectation on deployment of a radio network.

For RadioNetworkDeployExpectation <<IOC>>, the allowed value for attribute “expectationType” can be “RadioNetworkDeployment”.

The attribute “deploymentAreas” is used to represent a list of coverage areas to deploy the radio network. Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer. The intent attribute “deploymentAreas” can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

6.2.2.1.2.1.2	Attribute

The RadioNetworkDeployExpectation includes attributes inherited from IntentExpectation_ <<IOC>> and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| radioSetting | M | T | T | F | T |
| frequencySettings | M | T | T | F | T |
| transportSetting | M | T | T | F | T |
| networkPerformance | M | T | T | F | T |
| networkCapacity | M | T | T | F | T |
| deploymentAreas | M | T | T | F | T |

###### 6.2.2.2.2.1	RadioServiceDeployExpectation <<IOC>>

6.2.2.3.2.1.1	Definition

This IOC represents the properties of an IntentExpectation for radio service deployment. RadioServiceDeployExpectation <<IOC>> is one realization of abstract class IntentExpectation <<IOC>>.  The RadioServiceDeployExpectation represent the MnS consumer’s expectation on deployment of a radio service.

For RadioServiceDeployExpectation <<IOC>>, the allowed value for attribute “expectationType” is “RadioServiceDeployment”.

6.2.2.2.2.1.2	Attribute

The RadioServiceDeployExpectation includes attributes inherited from IntentExpectation_ <<IOC>> and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| RANSliceSubnetProfile | M | T | T | F | T |

6.2.2.2.2.1.3	Attribute constraints

##### 6.2.2.2.3	DataType definition

###### 6.2.2.2.3.1	RadioSetting <<dataType>>

6.2.2.2.3.1.1	Definition

This <<dataType>> represents the properties of a set of radio configuration parameters for the radio network to be deployed.

6.2.2.2.3.1.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellIdList | M | T | T | F | T |
| gNBIdList | M | T | T | F | T |
| tACList | M | T | T | F | T |
| pLMNList | M | T | T | F | T |
| nRPCIList | M | T | T | F | T |

6.2.2..2.3.1.3	Attribute constraints

None

###### 6.2.2.2.3.2	FrequencySetting <<dataType>>

6.2.2.2.3.2.1	Definition

This <<dataType>> represents the properties of a set of frequency configuration parameters for the radio network to be deployed.

6.2.2.2.3.2.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| arfcnDL | M | T | T | F | T |
| arfcnUL | M | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| bSChannelBwUL | M | T | T | F | T |

6.2.2.2.3.2.3	Attribute constraints

None

###### 6.2.2.2.3.3	TransportSetting <<dataType>>

6.2.2.2.3.3.1	Definition

This <<dataType>> represents the properties of a set of transport configuration parameters for the radio network to be deployed.

6.2.2.2.3.3.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ngcLocalIpAddressList | M | T | T | F | T |
| nguRemoteIpAddressList | M | T | T | F | T |
| logicInterfaceId | M | T | T | F | T |
| nextHopInfo | M | T | T | F | T |

6.2.2.2.3.3.3	Attribute constraints

None

###### 6.2.2.2.3.4	NetworkCapacity <<dataType>>

6.2.2.2.3.4.1	Definition

This <<dataType>> represents the properties of network capacity for the radio network to be deployed.

6.2.2.2.3.4.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetMaximumUENumber | M | T | T | F | T |
| targetActivityFactor | M | T | T | F | T |

6.2.2.2.3.4.3	Attribute constraints

None

###### 6.2.2.2.3.5	NetworkPerformance <<dataType>>

6.2.2.2.3.5.1	Definition

This <<dataType>> represents the properties of network performance for the radio network to be deployed.

6.2.2.2.3.5.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetAveULRANUEThpt | M | T | T | F | T |
| targetAveDLRANUEthpt | M | T | T | F | T |

6.2.2.2.3.5.3	Attribute constraints

None

##### 6.2.2.2.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| radioSetting | It describes a set of radio configuration parameters for the radio network to be deployed. | type: RadioSetting multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| frequencySettings | It describes a list of set of frequency configuration parameters for the radio network to be deployed. | type: FrequencySetting multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| transportSetting | It describes a set of transport configuration parameters for the radio network to be deployed. | type: TransportSetting multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| networkCapacity | It describes the properties of network capacity for the radio network to be deployed | type: NetworkCapacity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| networkPerformance | It describes the properties of network performance for the radio network to be deployed | type: NetworkPerformance multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANSliceSubnetProfile | It describes the SLS for the radio service to be deployed. For the detailed definition of RANSliceSubnetProfile see clause 6.3.23 in TS 28.541[X]. | type: RANSliceSubnetProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellIdList | It describes a list of cell identity which can be assigned to the radio network to be deployed. The detailed definition of cell identity see TS 28.541[X]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdList | It describes a list of gNB Id which can be assigned to the radio network to be deployed. The detailed definition of gNB Id see TS 28.541[X]. | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tACList | It describes a list of nRTAC which can be assigned to the radio network to be deployed. The detailed definition of nRTAC see TS 28.541[X]. | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNList | It describes a list of PLMN Id which can be assigned to the radio network to be deployed. The detailed definition of PLMN Id see TS 28.541[X]. | type: PLMNId multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPCIList | It describes a list of nRPCI which can be assigned to the radio network to be deployed. The detailed definition of nRPCI Id see TS 28.541[X]. | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | It describes the arfcnDL for the radio network to be deployed. The detailed definition of arfcnDL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | It describes the arfcnUL for the radio network to be deployed. The detailed definition of arfcnUL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | It describes the bSChannelBwDL for the radio network to be deployed. The detailed definition of bSChannelBwDL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | It describes the bSChannelBwUL of the radio network to be deployed. The detailed definition for bSChannelBwUL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngcLocalIpAddressList | It describes a list of ngcLocalIpAddress of the NG control plane interface (NG-C) for the radio network to be deployed. | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nguRemoteIpAddressList | It describes a list of nguLocalIpAddress of the NG user plane interface (NG-U) for the radio network to be deployed. | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceId | It describes a logical transport interface for the radio network to be deployed The detailed definition of logicInterfaceId see TS 28.541. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nextHopInfoList | It describes the ingress transport node for the radio network to be deployed The detailed definition of nextHopInfolist see TS 28.541[X]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetMaximumUENumber | It describes the maximum number of should be supported by the radio network to be deployed. | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetActivityFactor | It describes the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the radio network. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveULRANUEThpt | It describes the average UL RAN UE throughput for the radio network to be deployed. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveDLRANUEThpt | It describes the average UL RAN UE throughput for the radio network to be deployed. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| deploymentAreas | It describes a list of coverage areas for the network to be deployed. Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer | type: CoverageArea multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes