- 3GPP TSG-SA5 Meeting #137e	S5-213238rev1

- 10 May to 19 May 2021, E-meeting

- Source:	China Telecom

- Title:	Add the MDA assisted energy saving

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.310: "Energy efficiency of 5G".

[4]	3GPP TS 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[9]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[10]	3GPP TS 28.532: "Management and orchestration; Generic management services".

# 3	Rationale

This pCR is to add the MDA assisted energy saving analysis for TS 28.104 based on the initial version (v0.0.0) [1].

# 4	Detailed proposal

Start of modifications (all new text)

### 7.6.1	MDA assisted energy saving analysis

#### 7.6.1.1	Description

Energy saving is a critical issue for the 5G operators. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NF (e.g. UPF etc.), and the energy saving activation decision making may be based on the various information such as performance measurements of the related cells/UPFs as described in TS 28.552 [2], the energy saving policies set by operators as specified in TS 28.310 [3] and the energy saving instructions provided by MDAS producer as described in TR 28.813 [4].

The MDAS producer may provide analytics report describing the energy saving analytics and decisions. The MDAS consumer may take the report to make energy saving decisions according to their current energy saving state.

The analytic report should contain the following information:

-	The identifier of the energy saving instruction described in the analytics report;

-	Location of geographical area, UPFs or the cells where the unreasonable energy consumption exists;

-	Recommended 5GC NF deployment options in high-value traffic region (e.g. location of VNF in context of energy saving).

-	Root cause of the energy consumption issue;

-	Predictions related to the trends of traffic load which could be used as references for making decision on energy saving;

-	Current energy saving state of the cells.

-	Recommended optimal prediction data models to assist the energy saving related decision making;

-	Recommended energy saving decisions on cell or UPFs;

The MDAS producer should be informed of service experience changes and traffic load changes after the recommended decisions have been taken to make energy saving decisions, to update the recommended decisions and fine-tune the accuracy of the analytics.

The MDAS producer may provide the network wide analytics and cooperate with Core and RAN domains to recommend the energy saving decision on cell/UPF, which could guarantee the overall network and service performance are not affected as much as possible.

#### 7.6.1.2	MDA type

The MDA type for MDA assisted energy saving alaysis is: EnergySavingAnalysis.EnergySavingIssue.

#### 7.6.1.3	Analytics inputs

The management system has the overall view of network data information and it could also take the inputs from the control plane analysis (e.g., the analytics provided by NWDAF). The analytics inputs for energy saving analysis are provided in table 7.6.1.3-1.

Table 7.6.1.3-1: Analytics inputs for energy saving analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period; | 5.1.1.19.2 of TS 28.552 [2] |
|  | PNF Energy consumption: The energy consumed; | 5.1.1.19.3 of TS 28.552 [2] |
|  | PNF Temperature: The temperature over the measurement period; | 5.1.1.19.4 of TS 28.552 [2] |
|  | PNF Voltage: The voltage; | 5.1.1.19.5 of TS 28.552 [2] |
|  | PNF Current: The current; | 5.1.1.19.6 of TS 28.552 [2] |
|  | PNF humidity: The percentage of humidity during the measurement period; | 5.1.1.19.7 of TS 28.552 [2] |
|  | PDCP Data Volume of NR cells: The transmitted PDCP data volume; | 5.1.2.1 and 5.1.3.6 of TS 28.552 [2] |
|  | Virtual resource usage of NF: The resource usage of virtual network functions; | 5.7.1 of TS 28.552 [2] |
|  | Data volume of UPF; | 5.4 of TS 28.552 [2] |
|  | Delay related measurements of UPF; | 5.4 of TS 28.552 [2] |
|  | Number of PDU sessions with SSC mode 1 on UPF. | TS 28.552 [2] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc | 5.1.1.2 and 5.1.1.4 of TS 28.552 [2] |
|  | Network data: UE throughput, the mobility management data (e.g., rate of successful handover executions), packet delay data (e.g., air-interface delay), etc. | 5.1.1.1, 5.1.1.3 and 5.1.1.6 of TS 28.552 [2] |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [6], TS 26.247 [7], TS 26.114 [8] |
| NRM | MOIs of the cells, UPFs and SMFs. | TS 28.541 [9] |
| Alarm information | The alarm information of the cells, UPFs and SMFs. | TS 28.532 [10] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [5] |
| Prediction Data Models | The prediction data models, which may be based on ML algorithms and trained to be able to produce the expected training output for consumers, e.g., base station energy-saving scenarios prediction data models, traffic load prediction data models. | ML for MDA in “10.1, 10.2, 10.3” |
| Note1: The above parameters may not be the complete list. NOTE2: The prediction data models, including the model related information (e.g., including Model Id). The Model ID can be defined in ML model training for MDA.The model related information may include but not only model Id. NOTE3: MDAS may also obtain NF location or other inventory information such as energy efficiency and the energy cost of the data centers. The inventory information can be defined in NRMs. |  |  |

#### 7.6.1.4	Analytics output

To achieve an optimized balance between the energy consumed and the performance provided by the network, MDAS can be used to provide an analytics output by analysing the above information comprehensively to assist the energy saving. The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the MDA reports (see clause 7.2), are provided in table 7.6.1.4-1.

Table 7.6.1.4-1: Analytics output for energy saving analysis

| Information element | Definition | Data type | Support qualifier | Remarks |
| --- | --- | --- | --- | --- |
| Location | The geographical area, UPFs or the cells where the unreasonable energy consumption exists or recommended 5GC NF deployment options or NR cells in high-value traffic geographical area. | Location data | M |  |
| Root cause | The root cause of the part of the energy consumption that may be conserved, e.g., ultra-low traffic load area with energy consumption, excessive energy consumption | Root cause data | M |  |
| Trends of the traffic load | Predictions related to the trends of traffic load (e.g., PRB utilization rate, RRC connection number, etc.) which could be used as references for making decision on energy saving | Trends of the traffic load data | M |  |
| Current Energy Saving State of the cells | The information of the current energy saving state of the cells, e.g., the cells which are currently in the energy saving state. | Energy Saving State data | M |  |
| Recommended prediction data models | Describes the optimal prediction data models which can be recommended from the prediction data models (e.g., including Model Id) to provide more accurate information prediction results to assist the energy saving related decision making. | Recommendation data | M |  |
| Energy saving recommendations | For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of NR cells. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of UPFs. | Recommendation data | M |  |

End of modifications