- 3GPP TSG SA WG5 (Telecom Management) Meeting #137e	S5-213262rev1

- electronic meeting, online, 10 - 19 May 2021 	revision of S5-20xabc

- Source:	Huawei Technologies

- Title:	pCR TR 28.822 Add the UDM based solution for 5G VN Group

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.822 introducing the addition of UDM based solution for 5G VN Group.

# 2	References

[1]	3GPP TR 28.822 "Telecommunication management; Charging management; Study on charging aspect of 5G LAN-type Services".

# 3	Rationale

This pCR proposes to introduce the solutions in TR 28.822 " Telecommunication management; Charging management; Study on charging aspect of 5G LAN-type Services".

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.822[1].

1st change

### 5.1.4 	Solutions

#### 5.1.4.X		Solution #1.X: Add the UDM based solution for 5G VN Group

##### 5.1.4.X.1	General description

This solution#1.X which relying on CHF/5G Converged Charging System for UDM Converged Charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As per the TS 23.501[X], in order to support the dynamic 5G VN Group management, the following proceduces are provided by the UDM:

-	5G VN Group identification and membership:

-	the UDM allocates the Internal Group ID for a newly created 5G VN Group and maps the External Group ID to Internal Group ID.

-	the UDM updates the Internal Group ID-list of the corresponding UE's subscription data in UDR, if needed.

-	The UDM updates the Group Identifier translation in the Group Subscription data with the Internal Group ID, External Group ID and list of group members, if needed.

-	5G VN Group data: The UDM stores/updates the 5G VN group data (PDU session type, DNN and S-NSSAI, Application descriptor, Information related with secondary authentication / authorization) in UDR.

According the difference interactions between CHF and UDM, the solution#1.Y will be present as the following:

-	Charging Function (CHF) exposing Nchf_ConvergedCharging service

-	UDM as the NF service producer to provide the Nudm_SDM_Get service.

-	CEF as the NF Consumer to consume the Nchf_ConvergedCharging Service and Nudm_SDM_Get service.

The Charging Data Request and Charging Data Response are exchanged between the CEF and the CHF, based on event based charging scenarios specified in TS 32.290 [X].

The Charging Data Request is issued by the the CEF towards the CHF when certain following conditions (chargeable events) are met.

-	Addition of 5G VN group;

-	Modification of 5G VN group;

-	Deletion of 5G VN group;

The following charging Information is characterized for 5G VN group.

-	External Group ID;

-	Internal Group ID;

-	DNN;

-	S-NSSAI;

-	list of group members if needed(Optional);

-	PDU session type;

-	Application descriptor

Editor’s Note: the charging information for UDM based solution is ffs.

##### 5.1.4.X.2		Solution #1.X: UDM Charging Enablement Function(CEF) based solution

###### 5.1.4.X.2.1	Architecture description

The converged charging architecture for UDM Charging Enablement Function(CEF) based solution as depicted in figure 5.1.4.X.2.1-1 UDM based solution-CEF and are proposed for the event based charging of 5G VN Group.

Figure 5.1.4.X.2.1-1: UDM based solution-CEF

###### 5.1.4.X.2.2	Procedures description

The figure 5.1.4.X.2.2-1 describes the high level charging procedure for UDM based solution (CEF) Converged charging for 5G VN group. The intention of solution #1.X is to reuse the current services exposed by UDM.

Figure 5.1.4.X.2.2-1: Message flow for UDM Converged charging (CEF) for 5G VN group

1. The CEF decides to subscribe the 5G VN Group information based on the charging requirement.

2. The CEF sends the 5G VN Group information subscription Request to the UDM.

3. The UDM sends the 5G VN Group information subscription successful Response to the CEF, including current VG group information, e.g. the Internal Group ID, Internal Group ID-list of the corresponding UE's subscription data.

3ch-a. The CEF sends Charging Data Request [Event] to CHF for 5G VN Group subscription information.

3ch-b. The CHF creates a CDR.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CEF.

4. The UDM sends the notifications when the subscribed the 5G VN Group information is modified.

4ch-a. The CEF sends Charging Data Request [Event] to CHF for the received notifications.

4ch-b. The CHF creates a CDR.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CEF.

End of changes