3GPP TSG-SA5 Meeting #137-e	S5-213287

- Online, , 10th May 2021 – 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0040 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | × |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.535 Targeted ACCL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It would be beneficial if the ACL can target a particular location while assuring the SLS. Strict SLS assurance for a service can be provided using ACL for a high-valued vertical customer present in a particular location. The example of this may include SP assuring assurance goal of guaranteed throughput for an eMBB service inside a customer premises. Further, it would be even beneficial to target particular UE(s) in a particular location for SLS assurance. Strict SLS assurance can be provided for a VIP customer(s) present in a particular location. The example of this may include SP assuring assurance goal of guaranteed throughput for an eMBB service for a particular UE(s) inside a customer premises |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR propsoses an ability for an ACCL to target a particular location and UE while performing SLS assurance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In efficient COSLA implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | × | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.1.x	Targeted Assurance Closed Control Loop.

The goal of this use case is to provide the consumer of an assurance closed loop the ability to set targets (example location, UEs) per assurance goal(s) of an ACCL. This implies that an ACCL will be assuring goal(s) pertaining to the configured targets only. The SLS requirement may come for a particular location. It would be beneficial if the ACL can target a particular location while assuring the SLS. Strict SLS assurance for a service can be provided using ACL for a high-valued vertical customer present in a particular location. This will require configuring the ACL accordingly and would need ACL to collect the performance measurements, while monitoring the network, for the same target location. The example of this may include SP assuring assurance goal of guaranteed throughput for an eMBB service inside a customer premises.

End of 1st Modified Sections

2nd Modified Sections

## 6.2	Requirements

REQ-CSA-CON-XX The 3GPP Management System shall have the ability to provide SLS assurance within a particular location.

End of 2nd Modified Sections