3GPP TSG SA WG5 Meeting 137-e		S5-213327rev1

- electronic meeting, online, 10 - 19 May 2021

- Source: 	NEC, Intel, China Telecom

- Title:	pCR draft TS28.104 add text to MDA functionality and service framework clause

- Document for:	Approval

Agenda Item:	6.4.18 Enhancements of Management Data Analytics Service

# 1	Decision/action requested

The groups is asked to discuss and approve the propoal

# 2	References

[1]	S5-213038 Initial version (v0.0.0) of TS 28.104

[2]	S5-213039 add structure of TS 28.104 Management Data Analytics (MDA)

# 3	Rationale

Draft specification structure for TS28.104 has been proposed for Management Data Analytics (MDA) [2] and this pCR proposes a text for the MDA functionality and service framework clause.

# 4	Detailed proposal

1st Modified Section

4				MDA functionality and service framework

4.1			General framework

Figure 4.1.x illustrates the MDA general functionality and service framework. Depending on the scenario, MDAS producer may collect data as input for MDA by acting as an MnS Consumer, and/or as an NWDAF subscriber, and/or as a consumer of another MDAS producer, and/or a non-3GPP management data consumer. The MDAS producer exposes the MDA reports to MDAS Consumer(s).

Figure 4.1.x: Functional overview and service framework of MDA

4.2 		Interaction with Core and RAN domains

The MDAS producer provides analytics data for management purposes based on input data related to different types of NFs or entities in the network, e.g., data reported from gNB and/or specific core network function(s). Depending on the scenario and when needed, the MDAS producer may use the analytics results produced by NWDAF as input.

MDAS may act as 3GPP domain-specific (e.g., RAN or CN) or as 3GPP cross-domain. Figure 4.2-x illustrates the coordination between NWDAF, gNB and MDAS producer(s) for data analytics purpose.

RAN domain MDAS producer

MDAS

3GPP cross-domain consumer

3GPP cross-domain MDAS producer (domain MDAS consumer)

CN domain MDAS producer

gNB

MDAS

MDAS

NWDAF

Other Core NF

Nnf

Nnwdaf

MDAS

MDAS

MnS

MnS

Nwdaf

RAN domain

Core domain

Figure 4.2.x Example of coordination between NWDAF, gNB and MDAS producer

-The NWDAF may consume MDAS and provide analytics service for 5GC NF for control purpose.

-The CN Domain MDAS producer may consume the service provided by NWDAF and other 5GC NFs for MDA purpose.

-The gNB may consume the MDAS for RAN control purpose.

-The RAN Domain MDAS producer may consume the MnS service provided by/for gNB for MDA purpose.

-The 3GPP cross domain MDAS Producer may consume (acting as Domain MDAS consumer) MDAS provided by domain-specific (RAN and/or CN) MDAS producer(s), and produce MDAS that may be consumed by 3GPP cross-domain MDAS consumer(s).

End of Modified Section