3GPP TSG SA WG5 Meeting 137-e		S5-213566d1

- e-meeting, 10th - 19th May 2021					                     						revision for S5-213392

- Source:	Alibaba group

- Title:	Add Description of Concept and Roles to TR 28.824

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR specifies the concepts and roles related to network management capability exposure.

# 4	Detailed proposal

Start of 1st Change

# 4	Overview

## 4.X	General

### 4.X.1	Concepts related to network management capability exposure

### 4.X.1.1 Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can conditionally indirectly consume the MnS within 3GPP management system via BSS. Exposed MnS (eMnS) represents the MnS that can be exposed by MnS producer to the external MnS consumer via BSS. eMnS may rely on a dedicated MnF (e.g. EGMF) or other standards like TMF that manages the exposure aspects.

### 4.X.2	Roles related to network management capability exposure

### 4.X.2.1	Exposed MnS consumer

The logical entity, external to the 3GPP management system, consuming an eMnS is called an eMnS consumer. An eMnS consumer is equivalent to MnS consumer which is outside the 3GPP management system. An eMnS consumer (e.g. verticals such as Industry, Internet Company, etc) may take the role of a CSC or NSC. The eMnS consumers usually have specific service requirements on 5G network. . In order to ensure access to  eMnS, the stakeholder (e.g. MNO, MVNO) usually makes contract with the vertical. The contract may contain the agreement on what eMnS, optionally under what condition, can be exposed. The condition can be certain constraint of eMnS access based on the contract, e.g. the access quota of certain eMnS, the access frequency of certain MnS, etc. Once the contract has been signed, the BSS may interact with OSS in order to complete the related configuration (e.g. what MnS, optionally under what condition, can be exposed) regarding the exposure of MnS within OSS. After that, the eMnS consumer can get access to the eMnS via BSS (e.g. Service Catalog).

### 4.X.2.2	Exposed MnS producer

The logical entity producing an eMnS that can be consumed by a eMnS consumer (e.g. verticals and service provider) outside the 3GPP management system is called exposed MnS producer. An eMnS producer can take the role of a CSP or NSP. The vertical makes the contract with the stakeholder (e.g. MNO, MVNO) regarding the expose of MnSs.  The contract can include what kinds of eMnS, optionally under certain condition, can be exposed to the eMnS consumer. The eMnS producer can expose certain eMnSs that are allowed to be accessed by the eMnS consumer through BSS (e.g. Service Catalog) upon the completion of configuration with respect to the exposure of eMnS. The contract can include what kinds of eMnS, optionally under certain condition, can be exposed to the eMnS consumer.

End of Change