| 3GPP TR 32.847 V0.3.0 (2021-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects Charging management Study on Charging Aspects for Network Slicing Phase 2 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General background	8

4.1	Background from GSMA	8

4.2	Background from 3GPP network architecture	10

4.3	Network Slice Management Charging	10

5	Potential charging requirements	10

6	Key Issues	11

6.1	Key Issue #1: Converged Charging for max. number of UEs per network slice	11

6.1.1	General Description	11

6.1.2	Solution#1 New NetworkSliceConvergedCharging service	11

6.1.3	Solution#2	14

6.1.x	Evaluation	14

6.2	Key Issue #2: Converged Charging for max. number of PDU sessions per network slice	14

6.2.1	General Description	14

6.3	Key Issue #3: Volume based Converged Charging per network slice	14

6.3.1	General Description	14

6.3.2	Solution#3.1 Volume aggregation by NS Tenant CCS	15

6.3.2.1	General description	15

6.3.2.2	Architecture description	16

6.3.2.3	Flow description	16

6.3.3	Solution#3.2 Volume control by CCS based on performance collection	19

6.3.3.1	General description	19

6.3.3.2	Architecture description	19

6.3.3.3	Flow description	19

6.4	Key Issue #4: Converged Charging for max duration per network slice	22

6.4.1	General Description	22

6.5	Key Issue #5: Converged charging for number of simultaneous UEs per network slice	22

6.5.1	General Description	22

6.5.2	Solution#5.1 Event based charging	22

6.5.2.1	General	22

6.5.2.2	Charging information	23

6.5.2.3	Trigger mechanism	23

6.5.3	Solution#5.2 Session based charging	23

6.5.3.1	General	23

6.5.3.2	Charging information	23

6.5.3.3	Trigger mechanism	23

6.5.x	Evaluation	23

6.6	Key Issue #6: Converged Charging for number of unique UEs per network slice	24

6.6.1	General Description	24

6.6.2	Solution#1	24

6.6.3	Solution#2	24

6.6.x	Evaluation	24

6.7	Key Issue #7: Converged charging for number of concurrent PDU sessions per network slice	24

6.7.1	General Description	24

6.7.2	Solution#1	25

6.7.3	Solution#2	25

6.7.x	Evaluation	25

6.8	Key Issue #8: UE Converged Charging based on network slice charging	25

6.8.1	General Description	25

6.8.2	Solution#8.1 NS Tenant CCS separated from UE CCS	25

6.8.2.1	General description	25

6.8.2.2	Architecture description	26

6.8.2.3	Flow description	26

6.9	Key Issue #9: Converged Charging for actual duration per network slice	29

6.9.1	General Description	29

Annex A (informative): Change history	30

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document continues the study on charging aspects of network slicing covered by 3GPP TR 32.845 [2] but no longer limited to charging data collection functionalities without quota management as specified for Network Slice management charging in 3GPP TS 28.202 [3] and for Network slice performance and analytics charging in 3GPP TS 28.201 [4].

The following is studied:

-	Network slice converged charging for Network Slices enabled over 3GPP 5GS;

-	Correlation aspects between Network Slice charging and the end user charging accessing the Network Slice;

-	Inter-Provider (B2B) charging aspects by considering the different Organizations (e.g. MNO, Verticals).

The Converged Charging service aspects associated to the 3GPP TR 23.700-40 [5] conclusion on network slice information is also considered.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 32.845: "Charging management; Study on charging aspects of network slicing”.

[3]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[5]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2”.

[6]	GSMA 5GJA NG.116: "Generic Network Slice Template".

[7]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[8]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[9]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[10]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GJA		5G Joint Activity (Initiative of NG)

CSC	Communication Service Customer

CSP	Communication Service Provider

EPS	Evolved Packet System

GSM	Global System for Mobile Communications

GSMA	GSM Association

GST	Generic Network Slice Template

NEST	Network Slice Types

NG	Networks Group (GSMA WG)

NOP	Network Operator

NSC	Network Slice Customer

NSI	Network Slice Instance

NSP	Network Slice Provider

# 4	General background

## 4.1	Background from GSMA

GSMA 5GJA has specified in NG.116 [6] the concept of Generic Network Slice Template (GST) from which several Network Slice Types (NESTs) can be derived by assigning values to applicable attributes defined in the GST.

As per 3GPP TS 23.501 [7] is the network slice a logical network that provides specific network capabilities and network characteristics, and GSMA tailored the network slice based on the specific requirements adhered to a Service Level Agreement (SLA) agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP).

Multiple roles related to network slicing are defined by GSMA and displayed in figure 4.1-1, based on some roles specified in 3GPP TS 28.530 [8] and described in 3GPP TR 32.845 [2].

The Charging model will fullfill the following used roles:

-	Communication Service Customer (CSC): Uses communication services, e.g. end user, tenant, vertical.

-	Communication Service Provider (CSP): Provides communication services. Designs, builds and operates its communication services. The CSP provided communication service can be built with or without network slice.

-	Network Operator (NOP): Designs, builds and operates networks and provides related services, including network services and network slices.

-	Network Slice Customer (NSC): The CSP or CSC who uses Network Slice as a Service.

-	Network Slice Provider (NSP): The CSP or NOP who provides Network Slice as a Service.

Depending on the scenarios an organisation can play one or several roles simultaneously, and apply charging models based on corresponding business relationship, e.g.:

-	Charging models for CSP to charge CSC for using communication services

-	Charging models for NOP to charge CSP for using their Networks and related Network services

-	Charging models for NOP playing NSP Role to charge CSP playing NSC Role for using their Networks and related network slices

Figure 4.1-1: Model roles in network slicing

The GST is a set of generic attributes that can characterise a type of network slice/service and is not tied to any specific network deployment.

The NEST is a GST filled with values. The attributes and their values are assigned to fulfil a given set of requirements derived from a network slice customer use case. The NEST is an input to the network slice preparation performed by NSP. NEST-related attributes can be differentiated by a Service Category.  For instance, when an attribute of a NEST includes a Service Category parameter and the UE subscription information includes a parameter value associated with that Service Category, then the UE will be subject to the differentiated service associated with the Service Category in the network slice.

Figure 4.1-2 places the GST and NEST in the context of the network slice lifecycle. As first step, the NSC provides the service requirements for the use case that it would like to fulfil, to the NSP. The NSP maps the service requirements into the attributes of the GST with the appropriate values generating a NEST.

Figure 4.1-2: GST and NEST in context of the network slice lifecycle

Editor’s Note: The addition of the concept of Network Slice Instance (NSI) is ffs.

## 4.2	Background from 3GPP network architecture

The architectural requirements per 3GPP TR 23.700-40 [5] are copied below to derive potential charging requirements:

"-	Solutions shall build on the 5G System architectural principles as in 3GPP TS 23.501 [7], including flexibility and modularity for newly introduced functionalities.

-	Subject to operator policy and national/regional regulations, solutions shall be able to exempt Emergency services, Multimedia Priority Service, and Mission Critical Service from count limits.

NOTE:	Handling of UE/PDU Session count exemption and any pre-emption to other non-mission critical services is expected to be controlled by operator policy.

-	While the support of network slicing applies only to 5GS, solutions supports subscriptions that allows EPS/5GS interworking. Therefore EPS/5GS interworking needs to be considered for the enforcement of counts for maximum number of UEs per network slice and for maximum number of PDU Sessions per network slice."

## 4.3	Network Slice Management Charging

Network Slice Management Charging specified by 3GPP TS 28.202 [3], enables the Converged Charging System (CCS) to capture for each Network Slice instance per 3GPP TS 28.530 [8] definition, the supported list of service profiles representing the Service Level Specification (SLS) between Network Slice Customer (NSC) and Network Slice Provider (NSP).

For each S-NSSAI, associated attributes and their values representing the Service Level Specification (SLS) are available to the Converged Charging system, via service profiles captured with the NetworkSlice instance.

These information are captured in NSM charging CHF record generated by CHF upon NetworkSlice instance creation, modification and termination.

# 5	Potential charging requirements

REQ-NSCH-01	The 5G charging framework should support converged charging per S-NSSAI.

REQ-NSCH-02	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs".

REQ-NSCH-03	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions".

REQ-NSCH-04	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", exempting Emergency Registered UEs

REQ-NSCH-05	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", exempting Emergency services, Multimedia Priority Service, and Mission Critical Service.

REQ-NSCH-06	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", including UEs connected in the EPS.

REQ-NSCH-07	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", including PDN connections in the EPS.

REQ-NSCH-08	The 5G charging framework should support converged charging per tenant based on assigned group of S-NSSAI(s).

REQ-NSCH-09	The 5G charging framework should support converged charging per S-NSSAI, based on "aggregated PDU sessions volume".

REQ-NSCH-10	The 5G charging framework should support converged charging per S-NSSAI, based on "PDU session maximum duration".

REQ-NSCH-11 The 5G charging framework should support individual UE converged charging based on Network Slice usage charging criteria.

REQ-NSCH-12	The 5G charging framework should support converged charging per S-NSSAI for the time an S-NSSAI is used.

# 6	Key Issues

## 6.1	Key Issue #1: Converged Charging for max. number of UEs per network slice

### 6.1.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on max. number of UEs.

Maximum number of UEs is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice and defined as the maximum number of UEs that can use the network slice simultaneously. The max. number of UEs that can use the network slice simultaneously is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging.

This investigation covers the following:

-	determination of max. number of UEs per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to max. number of UEs;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on max. number of UEs;

- 	determination of all needed charging information;

### 6.1.2	Solution#1 New NetworkSliceConvergedCharging service

### 6.1.2.1	Procedure Description

This solution addresses the Key Issue #1 by considering the new 5GC NF which has been defined as Network Slice Admission Control Function (NSACF).  While NSACF counts all UE registrations in S-NSSAI as received from the AMF(s), the CHF/5G Converged Charging System functionalities provide the charging service and quota management capabilities to this NSACF.

During the network slice provisioning, the CHF, under the ConvergedCharging service, receives and stores the "max. Nb of Reg UEs" which is considered by CCS as initial value for the network slice quota (NS_quota). This NS_quota is used by the CCS for the Network Slice quota management.

A new NetworkSliceConvergedCharging service is introduced for this Network Slice quota management between the CHF and the NSACF.

Editor’s note: The details of NetworkSliceConvergedCharging Service is FFS.

During the UEs registration, the NSACF provides the network slice admission control for all AMF(s) by counting the number of registered UEs against quota granted by the CHF under NetworkSliceConvergedCharging service consumption by the NSACF.

The following figure 6.1.2.1-1 represents the high-evel entities involved in the solution.

Figure 6.1.2.1-1 High level functional solution overview

Slice SLA are obtained by CHF at NSI creation via the ‘service profile’ as specified in TS 28.202 [3] (clause 5.1.3 "Network Slice Management charging information"), which includes the max Nb of Reg UEs per S-NSSAI and the CHF is setting the max Nb of Reg UEs as NS_quota.

Editor’s note: Futher adjustments will be added once the corresponding implementation of NSACF has been consolidated in SA2.

Editor’s note: The Update of NS_quota once the count of Reg UEs is reached the max. Nb of Reg UEs at NSACF and possible NSI update is ffs.

### 6.1.2.2	Flow description

The figure 6.1.2.2-1 describes the high-level charging procedure for Network slice converged charging based on NS_quota with setting of initial value by CHF which may provided to the NSACF during slice provisioning procedures.

Figure 6.1.2.2-1: NetworkSlice quota management - registration

The steps are with the following additions:

0ch. CHF gets per S-NSSAI the "max Nb of Reg UEs" obtained during NSI creation  and sets this as initial value for NS_quota of Nb of Reg UEs.

0. The NSACF counts the Nb of Reg UEs and check against the initial value for NS_quota (max Nb of Reg UEs).

1. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Xch-a: At least one S-NSSAI has reached the initial value for NS_quota of  Nb of Reg UEs.

Xch-b: the NSACF invokes the Nchf_NSConvergedCharging service for requesting more quota (i.e. NS_quota (max Nb of Reg UEs) of the exhausted S-NSSAI.

xch-c. The S-NSSAI for which the initial NS_quota is reached will be either granted with more NS_quota or the request for more quota is indicated as rejected, based on CCS capabilities and CHF update per S-NSSAI the value for "max Nb of Reg UEs" obtained during NSI update.

xch-d. CHF provides response to NSACF with appropriate setting of for S-NSSAI(s) based on the status of step xch-c.

2. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Editor’s note: Futher adjustments will be added once the corresponding implementation of NSACF has been approved in SA2.

### 6.1.3	Solution#2

### 6.1.x	Evaluation

## 6.2	Key Issue #2: Converged Charging for max. number of PDU sessions per network slice

### 6.2.1	General Description

This key issue is for investigating how to support network slice access based converged charging service. on max. number of PDU sessions.

Maximum number of PDU sessions is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice and defined as maximum number of concurrent PDU supported by the network slice. The number of max. number of PDU sessions is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging.

This investigation covers the following:

-	determination of max. number of PDU sessions per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to max. number of PDU sessions;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on max number of PDU sessions;

- 	determination of all needed charging information.

## 6.3	Key Issue #3: Volume based Converged Charging per network slice

### 6.3.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on total PDU sessions volume per S-NSSAI.

The volume considered is the aggregated PDU sessions volume.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on volume;

-	determination of the main interactions required to obtain network slice usage information based on volume.

-	identification of the main charging information to be collected;

### 6.3.2	Solution#3.1 Volume aggregation by NS Tenant CCS

#### 6.3.2.1	General description

This solution addresses the Key Issue#3 for REQ-NSCH-09 potential requirement and is based on functional separation between NS Tenant Converged Charging System (CCS) and UE CCS with exchanges between them.

Two types of subscribers are considered in the context of Network Slice: the Tenant the Network Slice is assigned to, which is referred to as "NS-Tenant", and the individual end-user "UE" served under the Network Slice.

Each type of subscriber is handled under a dedicated Converged Charging System (CCS): "Tenant CCS" and "UE CCS" respectively.

"UE CCS" performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF hosting individual UE account balance, provisioned with all necessary information related to subscribed-to services the UE has paid for. The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

"Tenant CCS" performs charging functionalities for NS-Tenant, and includes in particular:

-	CHF exposing Charging service(s) associated to NS-Tenant.

-	ABMF hosting NS-Tenant account balance, provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for, the subscription description includes in particular. the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

How the account balance is provisioned with the subscriber's profile (for both NS-tenant and individual UE) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The UE 5G data connectivity converged charging from SMF is specified in 3GPP TS 32.255 [9] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE PDU session charging.

A new charging service exposed by the CHF (Tenant CCS) is defined for enabling association between individual UE charging and NS tenant charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the Tenant CCS.

In this solution, the volume is the charging information used for this association: when individual UE PDU session volume charging starts, the "UE allowed volume" as determined by UE-CCS is sent to the Tenant CCS, and the Tenant CCS is then able to aggregate UEs PDU sessions volume per S-NSSAI. When the individual UE PDU session volume charging terminates, the "UE consumed volume" is sent to the Tenant CCS which recalculates the aggregation.

There is a possible option for intermediate reportings of "UE allowed volume" and "UE consumed volume" to the Tenant CCS for a better volume control by the Tenant CCS.

#### 6.3.2.2	Architecture description

Figure 6.3.2.2-1: Distributed charging architecture in reference point representation.

ChX for interaction between CHF (UE CCS) and CHF (Tenant CCS)

#### 6.3.2.3	Flow description

The figures below describe the high level charging procedure for Volume based Network slice converged charging using the distributed architecture.

Figure 6.3.2.3-1 describes the procedure when new UE PDU session starts:

Figure 6.3.2.3-1: Volume based Network slice converged charging – UE PDU session start

1.	NS tenant volume based charging per S-NSSAI covered by Tenant CCS

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.2 for individual UE charging covered by UE CCS.

9ch-b1.	UE PDU session volume charging has a dependency with NS volume charging for this S-NSSAI: determination to trigger ChS

9ch-b2a.	Charging service request to Tenant CCS with S-NSSAI, UE PDU session charging information, and UE requested Network Slice Charging information including the "UE allowed volume" which is determined by UE-CCS.

9ch-b2b.	Account, Rating and reservation control for the S-NSSAI based on new S-NSSAI total volume (increased by "UE allowed volume" received in UE PDU session charging information).

9ch-b2c.	CHF (Tenant CCS) provides appropriate answer in Charging service response.

9ch-b3.	Account, Rating and reservation control for individual UE.

9ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.  Figure 6.3.2.3.2 describes the procedure when UE PDU session is released:

Figure 6.3.2.3-2: Volume based Network slice converged charging – UE PDU session release

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.4 for individual UE charging covered by UE CCS.

2ch-b1.	UE PDU session volume charging has a dependency with NS volume charging for this S-NSSAI

2ch-b2a.	Charging service request to Tenant CCS with S-NSSAI, UE PDU session charging information, and UE requested Network Slice Charging information including the " UE consumed volume."

2ch-b2b.	Account, Rating and reservation control for the S-NSSAI based on new S-NSSAI total volume recalculated from received UE consumed volume.

2ch-b2c.	CHF (Tenant CCS) provides appropriate answer in Charging service response.

### 6.3.3	Solution#3.2 Volume control by CCS based on performance collection

#### 6.3.3.1	General description

This solution addresses the Key Issue#3 for REQ-NSCH-09 potential requirement and is based on volume control by CCS based on performance collection for a S-NSSAI via CEF.

The Tenant the Network Slice is assigned to, is referred to as "NS-Tenant" and handled under a Converged Charging System (CCS) "Tenant CCS" hosting NS-Tenant account balance.

This solution is based on 3GPP TS 28.201 [4] mechanisms, considering the total volume of aggregated PDU sessions per S-NSSAI is part of the Network slice performance and analytics (NSPA) charging information.The Mns Producer is responsible to collect volume from UPFs, and to aggregate per S-NSSAI.  and to notify on volume thresholds triggers supplied by CCS.

This solution is based on successive volume blocks reservation by CCS which are gradually debited based on volume collected under NSPA Charging.

During this mechanism, interaction with BSS takes place to either call for a recharge, notify the cost for recorded extra-volume, or revoke the NS-Tenant from using the S-NSSAI.

#### 6.3.3.2	Architecture description

#### The architecture for this solution is 3GPP TS 28.201 [4] Figure 4.2.2-1: Network slice performance and analytics converged charging architecture.

#### 6.3.3.3	Flow description

The following figure 6.3.3.3-1 describes the high level charging procedure for Volume control by CCS based on performance collection:

Figure 6.3.3.3-1: Volume control by CCS based on performance collection

1.	NS tenant volume based charging per S-NSSAI covered by Tenant CCS; account with maximum volume allowed Vm. 
Account, rating and reservation control for V1, V2,...Vn (with V1+V2+..Vn < Vm ), with V1 V2, ..Vn also configured in CEF.

Steps according to 3GPP TS 28.201 [4] clauses 5.2.2.4 and 5.2.2.2:

2.	CEF configured with subscription conditions set to Volume thresholds: V1, V2, …Vn

3. 	NSPA Charging (aggregated UPFs volume) using MnS with V1, V2, …Vn

4. 	Account, Rating control for V1 to Vn-1.

5. 	NSPA Charging (aggregated UPFs volume until Vn is reached)

6.	Account, Rating for Vn and reserve additional blocks Vn+1, Vn+2 (with Vn+1, Vn+2 < Vm)

7.	Charging data response supplied with Vn+1 and Vn+2 thresholds.

8. 	NSPA Charging (aggregated UPFs volume until Vn+2 is reached)

9.	Account, Rating for Vn+2. No more volume can be reserved

10.	Charging data response indicating "volume after Vn+2 to be considered in offline charging mode".

11. NSPA Charging for extra-volume under offline charging mode

During the process BSS is notified by the CCS when the volume is approaching the allowed limit or at any time for taking appropriate action

### 6.3.4	Evaluation

Both solutions #3.1 and #3.2 solve Key issue #3.

Solutions #3.1 allows to support network slice usage converged charging based on total PDU sessions volume per S-NSSAI aggregation by NS Tenant CCS interacting UE CCS during with individual UE charging. Pros:

-	Same NF source (i.e. SMF) used for volume usage collection by both individual UE and aggregation

-	Good level of accuracy of NS volume usage

-	No need for a new NF in the signalling path for PDU session handling since it relies on existing individual UE PDU session charging with UE CCS.

-	When NS volume limit is reached, new PDU sessions can be rejected.

-	Approaching NS volume limit can be anticipated.

Cons:

-	high level signalling between NS tenant CHFs   interfacing many UE CHFs (based on each CCS can have multiple CHFs).

-	NS tenant CCS has to maintain the actual NS total volume usage.

Solutions #3.2 allows to support network slice usage converged charging based on volume control by CCS based on performance collection for a S-NSSAI via CEF

Pros:

-	NS tenant CCS interfacing limited number of entities (aggregation handled by MnS producers)

-	No need for NS tenant CCS to maintain the actual NS total volume usage

Cons:

-	Different sources used for volume usage collection (i.e SMF for individual UE and UPFs for NS tenant) resulting in different nature of volume being counted.

-	Risk of high mismatch between sum of individual UEs volume and NS total volume

-	Risk of significant delay between when NS volume was actually consumed and when account and rating control is processed for this volume.

-	No possibility  for immediate stop by NS tenant CCS when NS volume crosses the limit.

The major drawback of the solution #3.2 is to be more in "offline charging" fashion by the lack of "online charging" control capabilities on actual usage which can be better achieved by solution #3.1.

Drawbacks of the solution #3.1 can be resolved under appropriate deployment options.

## 6.4	Key Issue #4: Converged Charging for max duration per network slice

### 6.4.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on PDU sessions maximum duration per S-NSSAI.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on PDU sessions duration;

-	determination of the main interactions required to obtain network slice usage information based on PDU sessions duration.

-	identification of the main charging information to be collected;

## 6.5	Key Issue #5: Converged charging for number of simultaneous UEs per network slice

### 6.5.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of simultaneous UEs.

The number of UEs is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of UEs that can use the network slice simultaneously. Since the maximum number of simultaneous UEs can part of the offering and in the extension part of the price for the offering, then also the number of UEs that use the network slice simultaneously can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of simultaneous UEs one price and if it does go above another price.

This investigation covers the following:

-	determination of number of simultaneous UEs passing a threshold for a network slice;

-	identification of the main charging information to be collected in addition to number of UEs;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

### 6.5.2	Solution#5.1 Event based charging

#### 6.5.2.1	General

Event based charging where the thresholds values for simultaneous number of UEs are set separately for going below (downwards) or above (upwards). This means that when a UE is added to the network slice and it at that point passes a upwards threshold it will trigger charging, and when a UE is removed from a network slice and it at this point passes a downwards threshold it will trigger charging. There may be several upwards triggers but only one downwards.

#### 6.5.2.2	Charging information

The minimum information needed to be able to charge for the number of simultaneous UE connected to a network slice can be found in table 6.5.2.2-1.

Table 6.5.2.2-1: Minimum charging information of simultaneous number of UEs

| Information Element | Description |
| --- | --- |
| Identifier of Network Slice | This field identifies Network Slice and can be the Network Slice Instance or the S-NSSAI. |
| Trigger | This field identifies the passing of the threshold upwards or downwards for simultaneous number of UEs |
| Trigger Timestamp | This field holds the time when the threshold was passed |
| Number of simultaneous UEs | This field holds the number of simulations UEs after the passing of the threshold |

#### 6.5.2.3	Trigger mechanism

Thresholds may be set at provisioning of the network slice together with other network slice profiled information and updated by configuration changes.

### 6.5.3	Solution#5.2 Session based charging

#### 6.5.3.1	General

Session based charging where the session is started and kept as long as the simultaneous number of UEs is above a certain value. This means that when a UE is added to the network slice and it at that point passes the threshold it will initiate charging, and when a UE is removed from a network slice and it at this point it again passes the threshold it will terminate charging. In between these the CHF may set new triggers.

#### 6.5.3.2	Charging information

The minimum information needed to be able to charge for the number of simultaneous UE connected to a network slice can be found in table 6.5.3.2-1.

Table 6.5.3.2-1: Minimum charging information of simultaneous number of UEs

| Information Element | Description |
| --- | --- |
| Identifier of Network Slice | This field identifies Network Slice and can be the Network Slice Instance or the S-NSSAI. |
| Trigger | This field identifies the passing of the threshold for simultaneous number of UEs |
| Trigger Timestamp | This field holds the time when the threshold was passed |
| Number of simultaneous UEs | This field holds the number of simulations UEs after the passing of the threshold |

#### 6.5.3.3	Trigger mechanism

The threshold may be set at provisioning of the network slice together with other network slice profiled information and updated by configuration changes. The CHF may provide new triggers during the charging session.

### 6.5.x	Evaluation

## 6.6	Key Issue #6: Converged Charging for number of unique UEs per network slice

### 6.6.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of UEs.

The number of Ues is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of Ues that can use the network slice simultaneously. This means that the maximum number of simultaneous Ues can part of the offering and in the extension part of the price for the offering. Simultaneous Ues will currently mean unique Ues (i.e. with a unique identifier) and at the same time. This can be extended to also allow to charge based on the number of unique Ues that connect to a network slice over a period of time. For example, if during a period the number of unique Ues is below a specific value one price and if it does go above another price for the network slice.

This investigation covers the following:

-	determination of number of unique Ues accessing the network slice passing a threshold;

-	identification of the main charging information to be collected in addition to number of Ues;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

Editor’s Note: how to determine that a UE is unique is ffs.

### 6.6.2	Solution#1

### 6.6.3	Solution#2

### 6.6.x	Evaluation

## 6.7	Key Issue #7: Converged charging for number of concurrent PDU sessions per network slice

### 6.7.1	General Description

This key issue is for investigating how to support network slice access based converged charging service on number of concurrent PDU sessions.

The number of PDU sessions is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of concurrent PDU sessions supported by the network slice. Since the maximum number of concurrent PDU sessions can be part of the offering and in the extension part of the price for the offering, then also the number of PDU sessions that use the network slice concurrently can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of concurrent PDU sessions one price and if it does go above another price.

This investigation covers the following:

-	determination of number of concurrent PDU sessions passing a threshold for a network slice;

-	identification of the main charging information to be collected in addition to number of PDU sessions;

-	identification of the main interactions with the NFs to obtain the charging information;

- 	determination of need for online and offline charging methods.

### 6.7.2	Solution#1

### 6.7.3	Solution#2

### 6.7.x	Evaluation

## 6.8	Key Issue #8: UE Converged Charging based on network slice charging

### 6.8.1	General Description

This key issue is for investigating how to support UE Converged Charging influenced by network slice converged charging of the network slice serving the UE.

UE Converged Charging is based on criteria related to the Network Slice serving a UE. For example, the tenant has paid for a Network Slice allowing a large number of UEs and when the load level of network slice is low, wants some UE to be allowed with a lower tariff for using Network resources.

This investigation covers the following:

-	determination of which entity/entities is/are suitable to provide the network slice level of charging information for UE Converged Charging.

-	determination of the main interactions for UE Converged Charging to be influenced by network slice converged charging.

-	identification of the main charging information.

### 6.8.2	Solution#8.1 NS Tenant CCS separated from UE CCS

#### 6.8.2.1	General description

This solution addresses the Key Issue#X for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is based on distributed charging architecture split between NS Tenant Converged Charging System (CCS) and UE CCS and exchanges between them.

Two types of subscribers are considered in the context of Network Slice: the Tenant the Network Slice is assigned to, which is referred to as "NS-Tenant", and the individual end-user "UE" served under the Network Slice.

Each type of subscriber is handled under a dedicated Converged Charging System (CCS): "Tenant CCS" and "UE CCS" respectively.

"UE CCS" performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF hosting individual UE account balance, provisioned with all necessary information related to subscribed-to services the UE has paid for. The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

"Tenant CCS" performs charging functionalities for NS-Tenant, and includes in particular:

-	CHF exposing Charging service(s) associated to NS-Tenant.

-	ABMF contains NS-Tenant account balance, provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for, the subscription description includes in particular. the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

How the account balance is provisioned with the subscriber's profile (for both NS-Tenant and individual UE) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The UE 5G data connectivity converged charging from SMF is specified in 3GPP TS 32.255 [9] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE PDU session charging.

The Registration management converged charging from AMF is specified in 3GPP TS 32.256 [10] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE registration.

A new charging service exposed by the CHF (Tenant CCS) is defined for enabling association between individual UE charging and NS tenant charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the Tenant CCS.

In this solution, the NS Tenant charging information used for individual UE charging can include a various set of information such as "Max nb of PDU sessions", "Nb of PDU sessions", "load level", "Max nb of registered UEs", "Nb of registered UEs"…

#### 6.8.2.2	Architecture description

Figure 6.8.2.2-1: Distributed charging architecture in reference point representation.

ChX for interaction between CHF (UE CCS) and CHF (Tenant CCS)

#### 6.8.2.3	Flow description

The figure 6.8.2.3.1 describes the high level charging procedure for UE PDU session converged Charging influenced by Network slice converged charging using distributed charging architecture.

Figure 6.8.2.3.1: UE PDU session converged Charging influenced by Network slice converged charging

1ch. NS Tenant NSM charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI in service profile: "Max nb of PDU sessions", "Max nb of UEs"…

2ch. NS Tenant NSPA based charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI KPIs.

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.2 for individual UE charging covered by UE CCS.

9ch-b1.	UE PDU session charging needs input from NS charging for this S-NSSAI. Examples of requested input: "Max nb of PDU sessions", "Nb of PDU sessions", "load level"…

9ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE PDU session charging information, UE requested NS charging information).

9ch-b2b.	Based on received "UE requested NS charging information", Tenant CCS correlates with existing NS charging information for the S-NSSAI:  NSM charging information ("Max nb of PDU sessions"..), NSPA charging information ("Nb of PDU sessions (increased by one for considering the new PDU session)", "load level",..)…

9ch-b2c.	CHF (Tenant CCS) provides "UE supplied NS charging information" in Charging service response.

9ch-b3.	Account, Rating and reservation control for individual UE based on "UE supplied NS charging information".

9ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.

The figure 6.8.2.3.2 describes the high level charging procedure for UE registration converged Charging influenced by Network slice converged charging using distributed charging architecture.

Figure 6.8.2.3.1: UE Registration converged Charging influenced by Network slice converged charging

1ch. NS Tenant NSM charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI in service profile: "Max nb of PDU sessions", "Max nb of UEs…"

2ch. NS Tenant NSPA based charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI KPIs

Steps according to 3GPP TS 32.256 [10] clause 5.2.2.2.4 for individual UE registration charging covered by UE CCS.

16ch-b1.	Allowed S-NSSAI(s) in the Allowed NSSAI received for UE registration converged charging, needs input from NS charging. Examples for criteria: "Max nb of registered UEs", "Nb of registered UEs", "load level"…

16ch-b2. For each allowed S-NSSAI for which this association is needed:

16ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE registration charging information, UE requested NS charging information).

16ch-b2b.	Based on received "UE requested NS charging information", correlates with existing NS charging information for the S-NSSAI:  NSM charging information ("Max nb of UEs"..), NSPA charging information ("Nb of registered UEs (increased by one for considering the new registration)", "load level",..)…

16ch-b2c.	CHF (Tenant CCS) provides "UE supplied NS charging information" in Charging service response.

16ch-b3.	Account, Rating and reservation control for individual UE based on "UE supplied NS charging information".

16ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.

## 6.9	Key Issue #9: Converged Charging for actual duration per network slice

### 6.9.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on actual duration per S-NSSAI.

The duration a network slice is in use can part of the offering and in the extension part of the price for the offering. The duration a network slice is in use is defined as the duration an S-NSSAI (used to identify the network slice) have at least one active PDU session.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on actual duration;

-	determination of the main interactions required to obtain network slice usage information based on actual  duration.

-	identification of the main charging information to be collected;

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 01-2021 | SA5#135e | S5-211042 |  |  |  | Initial skeleton | 0.0.0 |
| 02-2021 | SA5#135e | S5-211416 S5-211418 S5-211044 S5-211045 S5-211421 S5-211385 S5-211387 |  |  |  | Update on skeleton Rel-17 pCR TR 32.847 – add scope Rel-17 pCR TR 32.847 – add new subclause on background Rel-17 pCR TR 32.847 – add new subclause on key Issues Rel-17 pCR TR 32.847 – Introduction of key Issue #1 & #2 Rel-17 pCR 32.847 Introduction of background on NS charging information Rel-17 pCR 32.847 Introduction of potential requirements | 0.1.0 |
| 02-2021 | SA5#135e |  |  |  |  | EditHelp review | 0.1.1 |
| 03-2021 | SA5#136e | S5-212475 S5-212476 S5-212477 S5-212478 |  |  |  | Rel-17 pCR TR 32.847 - update subclause on background from GSMA Rel-17 pCR 28.847 Introduction of new requirements and Key issues - volume and duration Rel-17 pCR 32.847 Adding key issue for number of UEs per network slice Rel-17 pCR 32.847 Adding key issue for number of PDU sessions per network slice | 0.2.0 |
| 05-2021 | SA5#137e | S5-213121 S5-213359 S5-213360 S5-213617 S5-213618 S5-213619 S5-213620 S5-213621 S5-213643 |  |  |  | Introduction of Key issue#X Individual UE charging based on Network Slice Adding event based solution to key issue 5 Adding session based solution to key issue 5 Introduction of a distributed charging architecture solution for KI#3 Introduction of a solution for KI#3 based on KPI Solutions evaluation for KI#3 Introduction of a solution based on distributed charging architecture for KI#X Solution for Key Issue #1 based on new charging service Intro new requirements and KI - NS actual duration | 0.3.0 |