| 3GPP TR 28.816 V0.5.0 (2021-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects of 5GS CIoT (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and background	9

4.1	General	9

4.2	 5G data connectivity domain charging	9

4.3	CP data transfer charging	9

4.3.1	General	9

4.3.2	 CP data transfer charging with SCEF	9

4.3.3	 CP data transfer charging with NEF	10

4.4	Monitoring event charging	11

4.4.1	 General	11

4.4.2	 Monitoring event charging with SCEF	11

4.4.3	 Monitoring event charging with NEF	12

5	Use cases and key issues	13

5.1	Use cases	13

5.1.1	Use case #1: Control plane optimization in 5G data connectivity domain charging	13

5.1.1.1 	Description	13

5.1.2	Use case #2: Non-IP Data Delivery (NIDD) in control plane data transfer domain charging	14

5.1.3	Use case #3: Support of Serving PLMN Rate Control in control plane data transfer domain charging	14

5.1.4	Use case #4: Support of Small Data Rate Control in control plane data transfer domain charging	15

5.1.5	Use case #5: Handover 5GS CIoT charging with EPS interworking	15

5.1.6	Use case #6: Support for monitoring events charging	16

5.1.7	Use case #7: The access service of 5GS CIoT	16

5.1.8	Use case #8: Charging support in home-routed roaming scenario	16

5.1.9	Use case #9: Support of rate control of user data	17

5.2 	Potential charging requirements	17

5.3	Mapping of use cases	17

5.4	Charging scenarios and key issues	18

5.4.1	5G data connectivity domain charging	18

5.4.1.1	Key issues for 5GS CIoT charging in 5G data connectivity domain charging	18

5.4.1.2	 Potential solutions	18

5.4.1.3	 Evaluation	18

5.4.2	Control Plane data transfer domain charging	18

5.4.2.1	Key issues of 5GS CIoT charging in control plane data transfer domain charging	18

5.4.2.2	 Potential solutions	19

5.4.2.2.1	19

5.4.2.2.2	Charging enhancement to north bound API charging for NEF based NIDD	19

5.4.2.2.2.1 	Charging architecture in control plane data transfer domain charging for 5GS CIoT	19

5.4.2.2.2.2 	Flow for north bound API charging in case of NIDD configuration	19

5.4.2.2.2.3 	Flow for north bound API charging in case of NEF based NIDD	20

5.4.2.3	 Evaluation	21

5.4.3	Monitoring event domain charging	21

5.4.1.2	 Potential solutions	21

5.4.1.3	 Evaluation	21

Annex A: Title	21

Annex X: Change history	22

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document investigates charging enhancements on 5G data connectivity domain charging, control plane data transfer domain charging and Monitoring Events charging for 5GS CIoT for the corresponding functionalities based on TS 23.501 [3], TS 32.251 [4], TS 32.255 [6], TS 32.253 [7] and TS 32.278 [8]:

- Control Plane CIoT 5GS Optimisation

- Non-IP Data Delivery (NIDD)

- Support for Monitoring Events

- Support of rate control of user data, including small data rate control

- Category M UEs differentiation

- Exception report

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging"

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2"

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2"

[5]					3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[6]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2"

[7]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging"

[8]	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring event charging"

[9]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles"

[10]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function northbound Application Program Interfaces (APIs) charging"

[11]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [3], TS 23.502 [4], TS 32.251 [2], TS 32.255 [6], TS 32.253 [7], TS 32.254 [10], TS 32.278 [8] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AF	Application Function

AMF	Access and Mobility Management Function

API	Application Program Interface

CDR	Charging Data Record

CHF	Charging Function

CIoT	Cellular Internet-of-Things

CP	Control Plane

CPCN	Control Plane data transfer Charging Nodes

CTF	Charging Trigger Function

MECN	Monitoring Event Charging Node

MO	Mobile Originated

MT	Mobile Terminated

NEF	Network Exposure Function

NF	Network Function

NIDD	Non-IP Data Delivery

PS	Packet Switched

SCEF	Service Capability Exposure Function

SMF	Session Management Function

4	Concepts and background

4.1	General

The Packet Switched (PS) domain charging supports charging of CIoT functionalities (e.g., the usage of the radio interface for CIoT, CP CIoT Optimisation and Serving PLMN Rate Control) as described in TS 32.251 [2].

CIoT functionality is provided by the visited and home networks when the networks are configured to support CIoT. It applies to both the non-roaming case and the roaming case and some functionality may be dependent upon the existence of appropriate roaming agreements between the operators.

In 5GS CIoT has been evolved and refers to 5GS optimisations and functionality for support of Cellular Internet-of-Things (Cellular IoT, or CIoT) described in TS 23.501 [3] according to service requirements described in TS 22.261 [xx]. The specific functionality is described in the affected procedures and features of 5G service aspect, in TS 23.501 [3], TS 23.502 [4] and TS 23.503 [5].

4.2		5G data connectivity domain charging

The specification of converged offline and online charging for 5G Data Connectivity domain based on stage 2 description in TS 23.501 [3] is described in TS 32.255 [6].

This charging aspect includes the converged offline and online charging architecture and scenarios specific to the 5G Data Connectivity domain, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [9] onto the 5G Data Connectivity domain.

The charging functions specified for the 5G data connectivity charging support PDU session in SMF, service data flows, within PDU session and QoS flows within PDU session. The SMF supports Nchf interface toward CHF, to collect and send charging information as described in TS 32.255 [6].

4.3	CP data transfer charging

4.3.1	General

Functions for NIDD may be used to handle Mobile Originated (MO) and Mobile Terminated (MT) communication for unstructured data (also referred to as Non-IP), as specified in TS 23.501 [3]. The UE may obtain an Unstructured PDU session to the NEF during the PDU Session Establishment procedure. Whether or not use of NIDD API should be invoked is determined during a PDU session by the presence of a "NEF Identity for NIDD". The NEF exposes the NIDD APIs described in TS 23.502 [5] on the N33/Nnef reference point.

4.3.2		CP data transfer charging with SCEF

CP data transfer domain charging is specified in TS 32.253 [7]. Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications. The highlighting excerpt that shows 3GPP Nodes involved for Non-IP Data Delivery (NIDD) between the UE and the SCS/AS is described in Figure 4.3.2.1.

Figure 4.3.2.1: 3GPP Architecture Service Capability Exposure for Non-IP Data Delivery

As described in TS 32.253 [7], the following Control Plane data transfer Charging Nodes (CPCN), functioning as the CTF, may generate accounting metrics sets for Control Plane data transfer CDRs:

-	the SCEF, to record Control Plane data transfer;

-	the IWK-SCEF, to record Control Plane data transfer;

-	the MME, to record Control Plane data transfer (optional).

As described in TS 32.254 [10], for offline charging, SCEF functioning as the CTF, may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs:

-	the SCEF, to record Application Program Interfaces transaction.

As described in TS 32.254 [10], for online charging, northbound API online charging is based on SCEF utilizing Ro interface and application towards the OCS.

4.3.3		CP data transfer charging with NEF

NEF is specified in TS 23.501 [3] to provide Exposure of capabilities and events, NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing. The Non-roaming architecture for Network Exposure Function in reference point representation is depicted in figure 4.3.3.1.

Figure 4.3.3.1: Non-roaming architecture for Network Exposure Function in reference point representation

Converged charging is be performed by the NEF interacting with CHF using Nchf service.

The charging requirements for northbound API for Exposure Function based on NEF Northbound API as described in TS 32.254 [10], is used to specify the offline and online charging description for Northbound Application Program Interfaces (API) and NEF charging CHF CDR.

4.4	Monitoring event charging

4.4.1		General

The Monitoring Events feature is intended for monitoring of specific events in the 3GPP system and reporting such Monitoring Events via the NEF, as described in TS 23.501 [3]. For CIoT, the list of supported monitoring events is specified in table 4.15.3.1-1 of TS 23.502 [4].

4.4.2		Monitoring event charging with SCEF

The charging aspect for Monitoring Events, based on the functional stage 2 description of SCEF is specified in TS 32.278 [8]. The figure 4.4.2.1 shows overall architecture for Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications, which includes Monitoring Event functionality described in TS 32.278 [8].

Figure 4.4.2.1: 3GPP Architecture for Service Capability Exposure with SCEF

For offline charging the following Monitoring Event Charging Nodes (MECN), functioning as the CTF, may generate accounting metrics sets for Monitoring Event CDRs:

-	the MME, to record Monitoring Events;

-	the SGSN, to record Monitoring Events;

-	the IWK-SCEF, to record Monitoring Events.

As described in TS 32.254 [10], for online charging SCEF functioning as the CTF may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs:

-	the SCEF, to record Application Program Interfaces transaction.

As described in TS 32.254 [10], for online charging, northbound API online charging is based on SCEF utilizing Ro interface and application towards the OCS.

4.4.3		Monitoring event charging with NEF

NEF is specified in TS 23.501 [3] to provide Exposure of capabilities and events, NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing. The Non-roaming architecture for Network Exposure Function in reference point representation is depicted in figure 4.4.3.1.

Figure 4.4.3.1: Non-roaming architecture for Network Exposure Function in reference point representation Converged charging is be performed by the NEF interacting with CHF using Nchf service.

The charging requirements for northbound API for Exposure Function based on NEF Northbound API as described in TS 32.254 [10], is used to specify the offline and online charging description for Northbound Application Program Interfaces (API) and NEF charging CHF CDR.

5	Use cases and key issues

5.1	Use cases

5.1.1	Use case #1: Control plane optimization in 5G data connectivity domain charging

5.1.1.1 	Description

The Control Plane CIoT 5GS Optimisation is used to exchange user data between the UE and the SMF as payload of a NAS message in both uplink and downlink directions, avoiding the establishment of a user plane connection for the PDU Session as described in TS 23.501 [3].

When the Control Plane CIoT 5GS Optimisation feature is used and the PDU session type is unstructured, the SMF selects either NEF or UPF based on information in the UE’s subscription as described in TS 23.501 [3]. The control plane is used with Control Plane CIoT 5GS Optimization instead of establishment data plane connection, the SRB1bis is established for NB-IoT rather using dedicated bearer. The cost of data plane connection and dedicated radio bearer are reduced by this optimization for 5GS CIoT.

During the PDU Session Establishment procedure the AMF indicates to the SMF that Control Plane CIoT 5GS Optimisation is available for data transmission. If a PDU session shall only use Control Plane CIoT 5GS Optimisation, the AMF provides a Control Plane Only Indicator to the SMF during the PDU session establishment. A UE and SMF receiving the Control Plane Only Indicator for a PDU session shall always use the Control Plane CIoT 5GS Optimisation for this PDU session.

5.1.2	Use case #2: Non-IP Data Delivery (NIDD) in control plane data transfer domain charging

Functions of NIDD is a 5GS CIoT feature to handle Mobile Originated (MO) and Mobile Terminated (MT) communication for unstructured data (also referred to as Non-IP), that delivery to the AF using the NIDD API or using the UPF via a PtP N6 tunnel.

The use case begins during the PDU session establishment, the UE may obtain an Unstructured PDU session to the NEF and a NEF Identity for NIDD may be presented. The AMF determines the use of the Control Plane CIoT 5GS Optimisation, and can indicate Control Plane CIoT 5GS Optimisation to SMF during PDU session establishment. For Unstructured PDU Session Type, the SMF checks the determination of NIDD API invoking for a PDU session depends on the presence of NEF Identity for NIDD for the DNN/S-NSSAI combination in the subscription. SMF selects that NEF and uses the NIDD API for the PDU session if determines.

The NEF is responsible to expose the NIDD API on N33/Nnef reference point.

In control plan data transfer domain charging, offline charging is supported by Control Plane data transfer charging Nodes (CPCN), with the CTF. The 5GC NFs (e.g., AMF, SMF and NEF) may collect the information on the use of NIDD API and generating the charging information, related to the use of CP 5GS CIoT optimization and NIDD API during the unstructured PDU session toward to NEF.

The 5GS CIoT charging may require 5GS NFs to support collecting the uses of NIDD API during a PDU session.

5.1.3	Use case #3: Support of Serving PLMN Rate Control in control plane data transfer domain charging

As described in TS 23.501 [3], the support of rate control of user data in 5GS CIoT is described as two 5GS CIoT features, which are Serving PLMN Rate control and Small Data Rate Control. Serving PLMN Rate Control as a 5GS CIoT feature allows the serving PLMN to protect AMF and the Signalling Radio Bearers in the NG-RAN from the load generated by NAS Data PDUs.

The use case begins during PDU session establishment and PDU session modification, SMF can indicate the support of PDU Session Serving PLMN Rate Control information to UE and UPF/NEF. That indicated information of Serving PLMN Rate Control is used for NAS Data PDUs.

During a PDU session, the UE can limit the rate to generate uplink NAS Data PDUs comply with the Serving PLMN policy, the UPF or NEF can limit the rate to generate downlink NAS Data PDU comply with the rate indicated for the PDU Session.

The indication information of Serving PLMN Rate Control, the change of Serving PLMN policy and enforcement of limit per PDU Session may be collected as charging information and CDRs regarding the use of this 5GS CIoT feature.

In control plane data transfer domain charging, the corresponding 5GC NFs those involve the Serving PLMN Rate control in a PDU connection to NEF may distinguish the indication information of Serving PLMN Rate Control and generate charging information during PDU session establishment and PDU session modification.

The potential charging requirement of control plane data transfer domain charging for this use case is: REQ-3GPPCH-5GSCIoT-04

5.1.4	Use case #4: Support of Small Data Rate Control in control plane data transfer domain charging

As described in TS 23.501 [3], the support of rate control of user data in 5GS CIoT is described as two 5GS CIoT features, which are Serving PLMN Rate control and Small Data Rate Control.

Small Data Rate Control allows HPLMN operator to offer customer services in form of ‘number of packets per time unit’. The SMF may determine the Small Data Rate Control based on operator policy, subscription, DNN, S-NSSAI, RAT type etc. The SMF can send a Small Data Uplink Rate Control command to the UE and inform the UPF or NEF of Small Data Rate Control.

During a PDU session, the UE may comply with this uplink rate control instruction. If the UE exceeds the uplink ‘number of packets per time unit’, the UE may still send uplink exception reports if allowed. When the PDU Session is released, the AMF can stores Small Data Rate Control Status so that can be used for a subsequent establishment of a new PDU session. The SMF may receive the stored Small Data Rate Control Status during PDU session establishment, then provides the parameters to the UE and UPF/NEF.

The use of Small Data Rate Control and enforcement of limit per PDU Session may be collected as charging information and CDRs regarding the use of this 5GS CIoT feature.

In control plane data transfer domain charging, the corresponding 5GC NFs those involve the Serving PLMN Rate control in a PDU connection to NEF may distinguish information of Small Data Rate Control l and generate charging information.

The potential charging requirements of control plane data transfer domain charging for this use case are: REQ-3GPPCH-5GSCIoT-05

5.1.5	Use case #5: Handover 5GS CIoT charging with EPS interworking

The use case describes the scenario when handover from 5GS to EPS and handover from EPS to 5GS regarding 5GS CIoT features. The use case begins when 5GS CIoT UE handover procedure starts.

During interworking from 5GS to EPS, if Small Data Rate Control is used the PGW-C+SMF from PGW-U+UFF or from SCEF+NEF, passes the Small Data Rate Control Status to the AMF for AMF to store.

If Small Data Rate Control applies on PDU Session, the AMF can provide Small Rate Control Status information from the H-SMF.

During interworking from EPS to 5GS, if the AMF has stored APN Rate Control Status and the PDU Session is considered a new first PDU Session to a DNN that is the same as the APN in stored APN Rate Control Status and interworking with EPC, the AMF sends the APN Rate Control Status to the SMF. If APN Rate Control Status is received from the AMF then the SMF provides the configured APN Rate Control Status to the PGW-U+UPF.

During PDU session release procedure, if the released PDU Session used APN Rate Control in EPC then the PGW-U+UPF provides the APN Rate Control Status to the SMF. The SMF provides the APN Rate Control Status to the AMF.

The charging information related to small data rate control as 5GS CIoT feature should be considered in EPC interworking scenarios. The related information exchanged during interworking 5GS to EPS support the use of APN rate control parameters when 5GS CIoT UE backs to EPS. In 5G data connectivity domain charging and control plane data transfer charging, charging information related the interworking between 5GS and EPS should be collected by 5GS.

The potential charging requirements of 5G data connectivity domain charging for this use case are: REQ-3GPPCH-5GSCIoT-06, REQ-3GPPCH-5GSCIoT-07

5.1.6	Use case #6: Support for monitoring events charging

The Monitoring Events feature is intended for monitoring of specific events in the 3GPP system and reporting such Monitoring Events via the NEF. A number of monitoring events are supported by 5GS CIoT.

The network operator provides 5GS access service with 5GS CIoT features. The application service provider (ASP) provides a service for the 5G CIoT UEs. The network operator and the application service provider, holding AF, have business relationship to toward application function regarding 5GS CIoT applications. The NOP wants to sell monitoring capabilities to the ASP, to allow the ASP to enhance their service offerings. The NOP wants to charge for the subscription event to the monitoring as well as per the actual notifications when these events occur. The charges have those information about the UE or group of UEs monitored.

The 5GS NF providing the capability to subscribe to monitoring and get notifications is the NEF and the 5G NFs, and entity that holds the ASP application is the AF.

The network operator provides the monitoring reporting to AF for particular 5GS CIoT application. The network operator can collect and record the subscribed event reporting as charging information for CDR generation.

The NOP wants to sell monitoring capabilities to the ASP, to allow the ASP to enhance their service offerings. The NOP wants to charge for the subscription event to the monitoring as well as per the actual notifications when these events occur. The charges shall have information about the UE or group of UEs monitored.

5.1.7	Use case #7: The access service of 5GS CIoT

The support to 5GS Cellular IoT service is based on optimizations regarding NB-IoT and Category M technologies. The 5GS CIoT service is provided by home and visited network when the network is configured to support CIoT.

The network operator provides the access service to 5GS CIoT UEs. The access service to 5G network for 5GS CIoT features, provides the access of the massive amount of devices to 5GS network with 5GS CIoT optimizations. Then, the communication services like the massive access of NB-IoT devices with business values, such as power saving, small data rate but large number of devices are possible for those 5GS CIoT UEs. The network operator provides the access service with 5GS CIoT features and addresses those business use cases.

As the network operator provides massive access to device and communication service with particular 5GS CIoT optimizations, the network operator can enable the access and use of 5GS, collects and records the 5GS CIoT access information (e.g., NB-IoT RAT type) and usage information (in a PDU session) in term of NB-IoT technologies.

The network operator can provide the access service to MTC type communication with broadband access to 5GS as category M devices. The category M technology is used for high data rate access for MTC device (e.g., wearable device) in 5GS. The network operator provides the access service with a set of 5GS CIoT features like control plane optimization with N3 data path toward UPF, the schedule data rate control of user data etc.

The network operators can enable the access of 5GS CIoT LTE-M radio access type for category M devices, collects and records the 5GS CIoT access information (e.g., LTE-M RAT type) and usage information (in a PDU session)  in term of Category M technologies.

5.1.8	Use case #8: Charging support in home-routed roaming scenario

As described in TS 23.501 [3], 5GS CIoT features are provided by the visited and home networks when the networks are configured to support CIoT.

The network operators provide 5GS CIoT access in home networks and visited networks. They apply to roaming scenario depend on roaming agreements between network operators. The home network operator, based on the subscription, the access information (e.g., RAT type) and 5GC CIoT UE behaviour information (control plane 5GS CIoT optimisation), provides the network access of 5GS CIoT UEs. The visited network operator provides the network access of 5GS CIoT based on access information (e.g., RAT type), rate control of user data and 5GS CIoT UE behaviour information (control plane 5GS CIoT optimisation), according to the roaming agreements.

5.1.9	Use case #9: Support of rate control of user data

As described in TS 23.501 [3], the support of rate control of user data in 5GS CIoT is described as two 5GS CIoT features, which are Serving PLMN Rate control and Small Data Rate Control.

The network operator provides 5GS CIoT features regarding the network access and 5GS use for 5GS CIoT UEs. The rate control of user data is motivated as a set of 5GS CIoT features to protect mobile network (e.g., control plane signalling) or provide a capability to schedule use per time duration regarding the amount of 5GS CIoT UEs.

The network operator provides a capability to 5GS usage for 5GS CIoT service on the number of message per day. In case that the small data rate control is used, the SMF informs the small data rate control information to NEF/UPF. The network operator is capable to configure the small data rate control parameters (e.g., number of messages per day) to the network and network holds and transfer small data control status for a PDU session.

5.2 	Potential charging requirements

REQ-3GPPCH-5GSCIoT-01	The charging aspect of 5G data connectivity domain charging shall support charging of Control Plane CIoT 5GS Optimisation.

REQ-3GPPCH-5GSCIoT-02 The 5GS CIoT charging should support charging of control plane data transfer based on 5G system architecture.

REQ-3GPPCH-5GSCIoT-03 The charging aspect of north bound API usage should support collecting charging information related to use of NIDD API.

REQ-3GPPCH-5GSCIoT-04 The charging aspect of control plane data transfer domain should support collecting charging information related to Serving PLMN rate control for 5GS CIoT based on 5G system architecture.

REQ-3GPPCH-5GSCIoT-05 The charging aspect of control plane data transfer should support collecting charging information of Small Data Rate Control for 5GS CIoT based on 5G system architecture.

REQ-3GPPCH-5GSCIoT-06 The charging aspect of 5G data connectivity domain charging should support collecting charging information related to 5GS CIoT for interworking to EPS procedures.

REQ-3GPPCH-5GSCIoT-07 The charging aspect of control plane data transfer should support collecting charging information related to 5GS CIoT for interworking to EPS procedures.

REQ-3GPPCH-5GSCIoT-08 The charging aspect of 5GS CIoT should support collection charging information related to monitoring event

REQ-3GPPCH-5GSCIoT-09 The charging aspect may support collecting charging information related to the access information of 5GS CIoT in 5G system.

REQ-3GPPCH-5GSCIoT-10 The charging aspect should support collecting charging information of access from NB-IoT and LTE-M RAT.

REQ-3GPPCH-5GSCIoT-11 The charging aspect of 5GS CIoT should support collection charging information related to 5GS CIoT for home-routed roaming scenario.

REQ-3GPPCH-5GSCIoT-12 The charging aspect of 5GS CIoT should support collecting charging information related to 5GS CIoT small data rate control.

5.3	Mapping of use cases

In agreed use cases in this document, the mapping from use cases to particular charging aspects of SA5 charging are shown as following in table 5.3.1.

| Charging domain | 5G data connectivity domain charging | CP data transfer domain charging | Monitoring event domain charging |
| --- | --- | --- | --- |
| Use case of 5GS CIoT charging | Control plane optimization Rate control of user data Exception report Handover 5GS CIoT charging in EPS interworking Charging support in home-routed roaming scenarios Access service of 5GS CIoT | Charging support of NIDD with NEF Rate control of user data Handover 5GS CIoT charging in EPS interworking Charging support in home-routed roaming scenarios Access service of 5GS CIoT | Monitoring event charging with NEF |

5.4	Charging scenarios and key issues

5.4.1	5G data connectivity domain charging

5.4.1.1	Key issues for 5GS CIoT charging in 5G data connectivity domain charging

The purpose of this clause is to describe the key issues charging enhancement for 5GS CIoT features regarding 5G data connectivity domain charging.

Considering the use cases described in clause 5.1 and potential requirements (REQ-3GPPCH-5GSCIoT-01, REQ-3GPPCH-5GSCIoT-02 and REQ-3GPPCH-5GSCIoT-06, REQ-3GPPCH-5GSCIoT-09, REQ-3GPPCH-5GSCIoT-10, REQ-3GPPCH-5GSCIoT-11, REQ-3GPPCH-5GSCIoT-12) as described in clause 5.2, the key issues in solution for 5G data connectivity domain charging for investigation can include:

- Identify the interaction(s) with SMF and potential charging information in 5G data connectivity domain charging;

- Identify the interaction(s) with SMF and potential charging information in case of EPC interworking.

- Identify the interaction(s) with SMF and potential charging information in home-routed roaming scenario.

5.4.1.2		Potential solutions

5.4.1.3		Evaluation

5.4.2	Control Plane data transfer domain charging

5.4.2.1	Key issues of 5GS CIoT charging in control plane data transfer domain charging

The purpose of this clause is to describe the key issues of charging enhancement for 5GS CIoT features regarding control plane data transfer domain charging.

Considering the use cases described in clause 5.1 and potential requirement (REQ-3GPPCH-5GSCIoT-02, REQ-3GPPCH-5GSCIoT-03, REQ-3GPPCH-5GSCIoT-04, REQ-3GPPCH-5GSCIoT-05, REQ-3GPPCH-5GSCIoT-07, REQ-3GPPCH-5GSCIoT-12) as described in clause 5.2, the key issues in solution for control plane data transfer domain charging for investigation can include:

- Identify which network functions in 5G system potentially provides charging information;

- Identify the interaction(s) with network function(s) and potential charging information in control plane data transfer domain charging.

5.4.2.2		Potential solutions

5.4.2.2.1

This clause is used to describe potential solution of charging enhancement in control plane data transfer domain charging.

5.4.2.2.2	Charging enhancement to north bound API charging for NEF based NIDD

5.4.2.2.2.1 	Charging architecture in control plane data transfer domain charging for 5GS CIoT

The NEF embedding the CTF generates charging events to the CHF for the north bound API charging. Figure 5.4.2.2.2.1.1 shows the architectural options for the converged charging.

Figure 5.4.2.2.2.1: The converged charging architecture for control plane data transfer domain charging

5.4.2.2.2.2 	Flow for north bound API charging in case of NIDD configuration

The NIDD configuration procedure is the configuration triggered by AF or NEF initiated. The NEF embedding CTF in north bound API charging may store and collect charging information of NIDD configuration.

The Figure 5.4.2.2.2.2.1 shows the message flow of charging enhancement in case of NIDD configuration.

Figure5.4.2.2.2.2.1: The message flow of charging enhancement for NIDD configuration

Step 1-2, the NEF initiated or AF triggered NIDD configuration messages exchange as described in clause 4.25.3 TS 23.502 [4]. The AF ID, and NEF ID are included and sent to AF for asking the NEF configuration.

Step 2ch-a, NEF is triggered to send Charging data request [Event] to CHF. The NEF ID, AF ID, the MTC provider information can be included as part of charging information.

Step 2ch-b, CHF opens CHF CDR.

Step 2ch-c, CHF sends Charging data response to the NEF.

Step 3-7, the rest messages of NIDD configuration procedure as described in clause 4.25.3 TS 23.502 [4].

5.4.2.2.2.3 	Flow for north bound API charging in case of NEF based NIDD

Non-IP Data Delivery (NIDD) it is a means for delivering data via a PDU Sessions of type "Unstructured". Converged charging and PEC based charging are used in this solution.

The Figure 5.4.2.2.3.1 shows the charging enhancements of SMF and CHF and procedure when UE performs the PDU Session establishment with PDU Sessions of type "Unstructured" toward NEF.

The Figure 5.4.2.2.3.1: Message flow of north bound API charging for NIDD

Step 1. The step 1 is described in clause 4.25.2 of TS 23.502 [4].

Step 2. As described in TS 23.502 [4], if the subscription information corresponding to DNN and S-NSSAI includes the "NEF Identity for NIDD" (NEF ID), the SMF should create a PDU Session towards the NEF. The SMF sends Nnef_SMContext_Create Request with 5GS CIoT optimization related indications (NIDD information, S-NSSAI, [Small Data Rate Control parameters], [Small Data Rate Control Status]) message towards the NEF.

Step 2ch-a. The NEF sends charging data request [Event] to CHF, with corresponding charging information.

Step 2ch-b. CHF opens CHF CDR for the PDU Session of NIDD.

Step 2ch-c. CHF sends Charging data response [Event] to the SMF.

Step 3. The NEF creates an NEF PDU session Context and send invokes Nnef_SMContext_Create Response to the SMF

Step 4. The step 4 is to finish the PDU Session establishment as described in clause 4.25.2 of TS 23.502 [4].

5.4.2.3		Evaluation

5.4.3	Monitoring event domain charging

The purpose of this clause is to describe the key issues of charging enhancement for 5GS CIoT features regarding monitoring event domain charging.

Considering the use cases described in clause 5.1 and potential requirement (REQ-3GPPCH-5GSCIoT-08) as described in clause 5.2, the key issues in solution for control plane data transfer domain charging for investigation can include:

- Identify which network functions in 5G system potentially provides charging information;

- Identify the interaction(s) with network function(s) and potential charging information in monitoring event domain charging.

5.4.1.2		Potential solutions

5.4.1.3		Evaluation

Annex A:
Title

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 | SA5#133e | S5-205030 |  |  |  | Initial skeleton. | 0.0.0 |
| 2020-10 | SA5#133e | S5-205465 S5-205466 S5-205229 |  |  |  | pCR 28.816 Add scope pCR 28.816 Add introduction pCR 28.816 Add proposal for skeleton for TR 28.816 | 0.1.0 |
| 2020-11 | SA5#134e | S5-206431 S5-206432 S5-206433 S5-206118 |  |  |  | pCR 28.816 add background for data connectivity charging pCR 28.816 add background for control plane data transfer charging pCR 28.816 add background for monitoring event charging pCR 28.816 add CP optimization for data connectivity charging | 0.2.0 |
| 2021-02 | SA5#135e | S5-211378 S5-211388 S5-211392 S5-211394 S5-211399 |  |  |  | pCR 28.816 add use case of NIDD for control plane data transfer charging pCR 28.816 add use case rate control of user data in control plane data transfer charging pCR 28.816 add use case for EPS interworking pCR 28.816 add abbreviations pCR 28.816 add mapping of use cases in charging aspects | 0.3.0 |
| 2021-03 | SA5#136e | S5-212466 S5-212467 S5-212505 S5-212506 |  |  |  | pCR 28.816 add use case of support for monitoring events charging pCR 28.816 Add use case the access of 5GS CIoT device pCR 28.816 Add use case support in home-routed roaming scenario pCR 28.816 add use case rate control of user data in 5G data connectivity charging | 0.4.0 |
| 2021-05 | SA5#137e | S5-213650 S5-213308 S5-213656 S5-213651 |  |  |  | Rel-17 pCR 28.816 update use case Rel-17 pCR 28.816 update content in use case mapping Rel-17 pCR 28.816 add key issues Rel-17 pCR 28.816 add potential solution for north bound API charging | 0.5.0 |